3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2201249r01

Elbonia, 14-25 February 2022

# Title:	[Draft] LS on PDU Blocks

# Release:	Release 18

# Work Item:	FS_XRM

Source:	Qualcomm Incorporated [SA2]

To:	SA4

Cc:	RAN1, RAN2, RAN3

Contact Person:

Name:	Dario S. Tonesi

Tel. Number:

E-mail Address:	dtonesi AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-2201041rXX

1. Overall Description:

SA2 understands from TR 26.926 “Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems” that XR applications impose requirements in terms of slices, rather than in terms of single packets. SA2 understands that a slice represents a sequence of packets that includes, e.g., all the necessary information to reconstruct a video frame and is useful only if all its packets have been received by the receiver (or a continuous sequence of packets up to the first missing packet can be used).

As currently defined in TS 23.501, the 5G QoS model is based on QoS Flows. The QoS Flow is the finest granularity of QoS differentiation in the PDU Session. A QoS Flow ID (QFI) is used to identify a QoS Flow in the 5G System. User Plane traffic with the same QFI within a PDU Session receives the same traffic forwarding treatment (e.g., scheduling, admission threshold), but each packet in the flow is treated independently of other packets in that flow.

SA2 has recently started working on the FS_XRM Study Item whose goal is, among others, to study “QoS and policy enhancements for XR service and media service” (see SP-211646 for more details). Within the context of this Study Item, SA2 agreed the attached definition of PDU Block, which may represent a slice, e.g., video/audio frame/tile (see attached S2-2201041rXX).

To set the requirements on the RAN correctly, it is necessary to understand what – if any – dependency there is between the IP packets that make up a picture frame, and if so, how the “start and end” of these slices can be indicated from the application to the 5GS.

2. Actions:

To SA4:

ACTION: 	SA2 kindly asks SA4 to take the above information into account and provide feedback.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #150e		April 6-12, 2022

TSG-SA2 Meeting #151e		May 16-20, 2022