3GPP TSG-SA2 Meeting #149-e	S2-2201250

# Online, , 14th Feb 2022 - 25th Feb 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0098 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minimizing data loss for handover from non-supporting to supporting node |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | From RAN3 chair´s notes of RAN WG3 #114bis-e # MBS7_MobilityNonSupporting RAN3 decided to minimize data loss and agreed on the solution to eliminate duplicates via using the same Core Network Sequence Numbers over both the unicast N3 tunnel and shared N3 tunnel for the multicast related handover from non-MBS supporting gNB to MBS supporting gNB. Related stage 2 procedures to provide and handle this sequence number are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The MB-UPF adds a sequence number to MBS data it sends to UPFs anf NG-RAN nodes. This sequence number is used during handover from non-MBS supporting gNB to MBS supporting gNB to minimize data loss |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures for Minimization of data loss are incomplete and not aligned with RAN3 conclusions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.4, 7.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 7.2.3.4	Xn/N2 based handover from non-MBS supporting NG-RAN node

When the UE has joined the multicast MBS session and the source NG-RAN node does not support MBS, the 5GC Individual MBS traffic delivery method is used for the multicast MBS session data delivery. When the Xn/N2 based handover procedure is triggered, the UE is handed over to the target NG-RAN node per existing Xn /N2 based handover procedure defined in TS 23.502 [6].

The following applies for an Xn based handover from an NG-RAN node not supporting 5G MBS:

-	The source NG-RAN node requests the unicast QoS flows mapped to the MBS QoS Flow(s) in the associated PDU session to be handed over to the target NG-RAN node.

-	In the Path Switch Request message, the target NG-RAN node, if MBS-capable, indicates it supports MBS to all SMFs corresponding to the PDU Sessions for which the UP connection was switched to the target NG-RAN node in the N2 SM information.

-	After successful handover, if the target NG-RAN node supports MBS and the MBS session was not yet activated, the target NG-RAN triggers the establishment of shared delivery as described in clause 7.2.1.4, which triggers the MB-SMF to activate the MBS session including the multicast MBS session related information in N2 MB-SM Information as described in step 11 of clause 7.2.5.2.

After the target NG-RAN node accepts the establishment of the shared delivery and receives multicast MBS session data via the shared N3mb tunnel and the NG-RAN has switched the provision of multicast MBS session date from individual to shared delivery, the NG-RAN removes the PDU Session resources for the mapped QoS flows..

The following applies for an N2 based handover from an NG-RAN node not supporting MBS:

-	During handover preparation phase, the SMF includes the multicast MBS session related information in N2 SM Information as described in step 7 of clause 7.2.1.3 and sends it to the target NG-RAN. The target NG-RAN, if MBS-capable, indicates it supports MBS to SMF in N2 SM information. If the target NG-RAN node supports MBS, the target NG-RAN node adds the UE into the MBS session context. If 5GC Shared MBS traffic delivery for the indicated multicast MBS session has not been established towards the target NG-RAN initiates the shared delivery establishment towards the MB-SMF as described in clause 7.2.1.4 which triggers the MB-SMF to activate the MBS session including the multicast MBS session related information in N2 MB-SM Information as described in step 11 of clause 7.2.5.2.

-	Further proceeding for removing mapped QoS flows from associated PDU Session as described for Xn handover above.

2nd change

#### 7.2.3.5	Minimization of data loss

To minimize the data loss of the UE during the handover procedure:

-	If source NG-RAN supports MBS and target NG-RAN does not support MBS, the multicast MBS session data is forwarded from source NG-RAN node to target NG-RAN node via the associated QoS flow within the associated PDU session.

-	If source NG-RAN does not support MBS and target NG-RAN supports MBS, for Xn/N2 handover, the multicast MBS session data is forwarded to the target NG-RAN node as the data forwarding mechanism defined in TS 23.502 [6]. Directly after the handover the target NG-RAN node thus still receives MBS session data via individual delivery, but the MB-SMF initiates shared delivery as desribed in Clause 7.2.3.4. If received from the MB-UPF, the UPF forwards multicast MBS session data received from the MB-UPF to the target NG-RAN node with the sequence number added. The UE may uitilise the alignment of sequence numbers  received before and after the switch from from individual to shared delivery to identify duplicates and request retransmission of missing data

End of changes