# SA WG2 Meeting #149-e (e-meeting)	S2-2101267

# 14-25 February 2022, Elbonia

Title:		Draft Reply LS on alternative IMSI for MUSIM

Reply to:	LS to SA2 and CT1 on alternative IMSI for MUSIM (R2-2201718, S2-2201252)

Release:	Release 17

Work Item:	MUSIM

Source:	SA2

To:		RAN2

Cc:		CT1, RAN3

Contact person:	Lars Nord

Lars.nord@sony.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

SA2 thanks RAN2 for the LSs (R2-2111329/S2-2200018 and R2-2201718/S2-2201252) on alternative IMSI for MUSIM. SA2 has discussed the LSs and would like to provide the following information.

When SA2 selected and specified the solution using Alternative IMSI Offset, the expectation from SA2 was that minimal impact would be required on the current UE, MME and eNB implementations, except the calculation of the Alternative IMSI value using the offset.

TS 23.401 clause 4.3.33.5 (see annex) specifies that the UE may receive Accepted IMSI Offset over NAS signalling. Then:

-	For the UE:

-	The UE shall then use the Accepted IMSI Offset to calculate the Alternative IMSI value which is then used in the UE to derive UE ID which is later used in the PO/PF calculations.

-	For the MME:

-	The MME provides the UE ID, derived from the Alternative IMSI value calculated based on the IMSI and the Accepted IMSI Offset, to the eNB.

-	S1AP does not require any updates to support this MUSIM feature and no impacts to eNB PO/PF calculations.

Furthermore, SA2 believe it would be good to specify how the Alternative IMSI value is calculated in a single specification to avoid misalignment. SA2 had defined the Alternative IMSI value, as an IMSI, to fit within the IMSI address space thus minimizing any implementation impact, see 23.401 §4.3.33.5:

Alternative IMSI value = [MCC] [MNC] [(MSIN value + Accepted IMSI Offset) mod (MSIN address space)]

where: the MCC, MNC and MSIN value are the fields of the UE's IMSI as defined in TS 23.003

However during the discussions SA2 have noticed that RAN2’s running CR to 36.304 includes a calculation that is not an IMSI for it spreads beyond the IMSI address space:

Alternative IMSI value = IMSI + Accepted IMSI Offset

SA2 would like to indicate to RAN2 that the RAN2 definition can lead to higher implementation impact at least in the UE given the UE_ID calculation in this case will need to account for the larger address space of the RAN2’s Alternative IMSI value vs. a normal IMSI and SA2’s definition. SA2 would therefore like to request RAN2 to follow the definition in TS 23.401.

# 2	Actions

To RAN2

ACTION: 	SA2 asks RAN2 to take the above answers into account, and to align the Alternative IMSI value definition tot that in 23.401.

# 3	Dates of next TSG SA WG2 meetings

3GPPSA2#150-e	4 - 8 April 2022	Electronic Meeting

3GPPSA2#151-e	16 - 20 May 2022	Electronic Meeting

# ANNEX

TS 23.401 text that specifies the aspects in the RAN2 LS from clause 4.3.33.5on "Paging Timing Collision Control".

#### 4.3.33.5 Paging Timing Collision Control To avoid possible paging occasion collision and to enhance the likelihood that paging is received successfully for different USIMs, a Multi-USIM UE may provide, for at least one USIM, a Requested IMSI Offset value that is used for the determination of paging occasions. Upon reception of a Requested IMSI Offset value from UE in Attach Request or Tracking Area Update Request, a supporting MME provides an Accepted IMSI Offset value to the UE in the Attach Accept or Tracking Area Update Accept message to acknowledge it supports the feature and provide the accepted value. The Accepted IMSI Offset Value may be different from the Requested IMSI Offset provided by the UE. The Alternative IMSI value, determined as below, is stored in the UE context in the MME. If the UE does not provide any Requested IMSI Offset value in Attach Request or Tracking Area Request, the MME removes any stored Alternative IMSI value in the UE context. The UE and the network use the Accepted IMSI Offset to determine the paging occasion. The UE and MME use the Accepted IMSI Offset value to calculate the Alternative IMSI value that is determined based on UE's IMSI as follows: Alternative IMSI value = [MCC] [MNC] [(MSIN value + Accepted IMSI Offset) mod (MSIN address space)] where: the MCC, MNC and MSIN value are the fields of the UE's IMSI as defined in TS 23.003. The MME uses the Alternative IMSI value to compute the UE Identity Index Value. The MME sends the UE Identity Index Value to RAN in the Paging message (see TS 36.413 [36]) for RAN to derive the paging occasions according to TS 36.304 [34]. The UE uses the Alternative IMSI value for the determination of paging occasions as specified in TS 36.304 [34]. NOTE 1: It is recommended to avoid excessive signalling load from UE due to this procedure. NOTE 2: The MME does not remove Alternative IMSI value if the Tracking Area Update Request is for periodic Tracking Area Update.