3GPP TSG-SA WG2 Meeting #149E (e-meeting)	S2-2200214

# Feburary 14-25, 2022, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0063 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Capture the reference point of PKMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA3 send LS S3-214443 to SA2 and suggest SA2 to capture the new reference points introduced for 5G ProSe security aspect, the following is copied from SA3 LS: SA3 would like to thank SA2 for the reply LS on Layer-3 UE-to-Network Relay authentication and authorization. SA3 discussed the reply LS and provides the following answers to SA2 questions: Q1: Is the user plane solution (using a PKMF) to be documented only in SA3 specifications as in LTE ProSe design so that it does not require any change to the system architecture in TS 23.304? Does SA3 expect any support for the inter-PKMF interfaces, and would that need any SA2 documentation, or would it also be documented in SA3 specifications only? While no work is expected in SA2 for the user-plane solution, SA3 suggests SA2 to capture the new reference points introduced in 5G ProSe (i.e., between UE and PKMF and between PKMFs) in the SA2 specification. Furthermore, as defined in clause 6.3.3.2.2 in TS 33.503, 5G PKMF contacts UDM to get UE subscription data in order to check if a UE is authorized to use or provide relay service, it is also needed to capture the reference point between PKMF and UDM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding reference points between the UE and 5G PKMF, between 5G PKMFs, between 5G PKMF and UDM, in clause 4.2.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing reference points related to 5G ProSe security aspect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

### 4.2.5	Reference points

PC1:	The reference point between the ProSe application in the UE and in the ProSe Application Server. It is used to define application level signalling requirements. This reference point is not specified in this release of the specification.

PC3a:	The reference point between the UE and the 5G DDNMF. PC3a relies on 5GC user plane for transport (i.e. an "over IP" reference point). It is used to authorise 5G ProSe Direct Discovery request, and perform allocation of ProSe Application Codes / ProSe Restricted Codes corresponding to ProSe Application Identities used for 5G ProSe Direct Discovery.

PC5:	The reference point between ProSe-enabled UEs used for control and user plane for 5G ProSe Direct Discovery, 5G ProSe Direct Communication and 5G ProSe UE-to-Network Relay.

PC8:	The reference point between the UE and the 5G ProSe Key Management Function (5G PKMF). The details are defined in TS 33.503 [29].

Npc2:	The reference point between the ProSe Application Server and the 5G DDNMF. It is used to define the interaction between ProSe Application Server and 5G DDNMF for 5G ProSe Direct Discovery.

Npc4:	The reference point between the UDM and 5G DDNMF. It is used to provide subscription information in order to authorise 5G ProSe Direct Discovery request.

Npc6:	The reference point between the 5G DDNMF in the HPLMN and the 5G DDNMF in a Local PLMN (5G ProSe Direct Discovery). This reference point is used for HPLMN control of ProSe service authorization.

Npc7:	The reference point between the 5G DDNMF in the HPLMN and the 5G DDNMF in the VPLMN. It is used for HPLMN control of ProSe service authorization.

Npc8:	The reference point between the PCF and the 5G DDNMF. It is used to define the interactions between the 5G DDNMF and the PCF to e.g. get a PDUID from the PCF.

Npc9:	The reference point between the 5G PKMF of the 5G ProSe Remote UE and the 5G PKMF of the 5G ProSe UE-to-Network Relay. The details are defined in TS 33.503 [29].

Npc10:	The reference point between the UDM and 5G PKMF. It is used to provide subscription information in order to authorise if a UE can act as a 5G ProSe Remote UE or a 5G ProSe UE-to-Network Relay.

NOTE:	Npc2, Npc4, Npc6, Npc7, Npc8, Npc9 and Npc10 show the interactions that exist between the NF services in the NFs. These reference points are realised by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

* * * * End of Changes * * * *