3GPP TSG-SA2 Meeting #149-e	S2-2200323

# Online, , 14th Feb 2022 - 25th Feb 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1256 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routing of IMS traffic via localized IMS media plane functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-01-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solutions for Routing of IMS traffic via localized IMS medfia plane functions were concluded by CT4 as part of the FS_eIMS5G2 study in TR 23.700-12. CT4 requests SA2 to agree corresponding normative stage 2 changes in incoming LS S2-2200204 of CT4 and concluded for key issue 1 (see TR 23.700-12), |  |  |  |  |  |  |  |  |
| Summary of change: |  | The concluded solutions in the FS_eIMS5G2 study for key issue 1, Solutions #1 (Conveying UPF FQDN to IMS nodes) and solution #4 ("Conveying UPF service area to IMS nodes via PCF N5") are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Routing of IMS traffic via localized IMS medfia plane functions is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Annex AC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex AC (normative):
Routing of IMS traffic via localized IMS media plane functions

# AC.1	General

The 5GC enables traffic to be routed via localized UPF (close to RAN and UE). The optional enhancements in the present Annex enable a selection of localized IMS media plane functions in proximity to such a localized UPF. An UDM/HSS based solution and a PCC based solution are provided as alternatives.

# AC.2	UDM/HSS based solution

During the establishment of a PDU session with a UE for an DNN used by the IMS, the SMF selects an UPF and stores the identity of that UPF (e.g. UPF FQDN) for the UE in the UDM. The UDM in turn stores the identity of that UPF (e.g. UPF FQDN) in the UDR as part of the SMF registration.

The HSS retrieves the IMS PDU session's UPF identity from the UDR and forwards it to the S-CSCF via Cx (or Nhss-ims) when an S-CSCF registers for the UE. When handling a mobile originating call setup procedure, the S-CSCF includes the UPF identity in the call setup requests. An upstream BGCF may use the UPF identity to select an MGCF in proximity to the UPF based on a locally configured information. The MGCF may use the received UPF identity to select an IM-MGW in proximity to the UPF.

If an IMS-AS handling a MO call setup requests from the UE or an IMS-AS handling a MT call setup requests of the UE desires the selection of an MRF close to the UPF for the UE, it retrieves the UPF identity via HSS from the UDR, uses this information for the MRFC selection, and includes the UPF identity in the related call setup requests. The MRFC may use the received UPF identity to select an MRFP in proximity to the UPF.

# AC.3	PCC based solution

An UPF identity or service area is conveyed from SMF to PCF and on to P-CSCF during multimedia session establishment. The PCF either stores the UPF identity or service area as part of the SM policy or fetches the UPF identity or service area from the SMF when requested by P-CSCF.

When the P-CSCF receives a session establishment request, it requests and obtains the UPF identity or service area from the PCF as part of the policy authorization for the session establishment and may select (based on a local configuration) an IMS-AGW in proximity to the UPF. The P-CSCF forwards the UPF identity or service area retrieved from PCF in the mobile originating session establishment request. Other IMS functions (e.g. BGCF, MGCF, IMS-AS, MRFC) may use the information to select an MGCF, IM-MGW, MRFC or MRFP in proximity to the UPF.