3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200590

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	New KI: 5GS assisted FL group performance monitoring

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract: It proposes a new key issue of 5GS assisted FL group performance monitoring.

1. Introduction/Discussion

This contribution is related to WT#1.3about 5GS assisted Federated Learning (FL) group performance monitoring:

WT#1.3.	Study whether and how 5GS provides assistance to AF and the UE for the AF and UE to manage the FL operation and model distribution/redistribution (i.e. FL members selection, group performance monitoring, adequate network resources allocation and guarantee.) to facilitate collaborative Application AI/ML based Federated Learning operation between the application clients running on the Ues and the Application Servers.

As defined in Table 7.10-3 of clause 7.10, TS 22.261, during the application level FL model training phase, the intermediate ML model information exchange between the UE and the AF/AS will occupy a large amount of network bandwidth. Therefore, it is necessary to study how to avoid the impacts of the application level FL model training on other UEs camping on the same network.

Table 7.10-3 KPI Table of Federated Learning between UE and Network Server/Application function

| # Max allowed DL or UL end-to-end latency | # DL experienced data rate | # UL experienced data rate | # DL packet size | # UL packet size | # Communication service availability | # Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1s | 1.0Gbit/s | 1.0Gbit/s | 132MByte | 132MByte |  | Uncompressed Federated Learning for image recognition |
| 1s | 80.88Mbit/s | 80.88Mbit/s | 10Mbyte | 10Mbyte | TBD | Compressed Federated Learning for image/video processing |
| 1s | TBD | TBD | 10MByte | 10MByte |  | Data Transfer Disturbance in Multi-agent multi-device ML Operations |

Furthermore, as shown in Figure 1, one iterator process for the FL model training phase is as follows:

-	Step 1: Each UE in the FL group derives the local ML model information based on the its own raw data and sends the local ML model information to the AF/AS.

-	Step 2: When the local ML model information from all the UEs in the FL group are received, the AF/AF aggregates all the local ML model information into a global ML model information and feedbacks the global ML model information to each UE.

-	Step 3: Each UE in the FL group derives the update local ML model information based on the global ML model information.

Figure 1: Training process of application level Federated Learning

Therefore, in order to increase the efficiency of the FL model training, it is necessary to study how to minimize the latency divergence when the FL model training server receiving the local ML model from different UEs.

Based on the above, the following aspects need to be considered for 5GS assisted FL group performance monitoring:

-	How to avoid the impacts on other UEs services because of the application level FL model training.

-	How to minimize the latency divergence when the FL model training server receiving the local ML model from different UEs, in order to increase the training efficiency.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-80.

# * * * * First change (All new text) * * * *

## 5.X	Key Issue #X: 5GS assisted FL group performance monitoring

### 5.X.1	General description

This key issue addresses how 5GS assisted FL group performance monitoring.

As defined in Table 7.10-3 of clause 7.10, TS 22.261, during the application level FL model training phase, the intermediate ML model information exchange between the UE and the AF/AS will occupy a large amount of network bandwidth. Therefore, it is necessary to study how to avoid the impacts on the current UE services because of the application level FL model training.

Furthermore, one iterator process for the FL model training phase is as follows:

-	Step 1: Each UE in the FL group derives the local ML model information based on the its own raw data and sends the local ML model information to the AF/AS.

-	Step 2: When the local ML model information from all the UEs in the FL group are received, the AF/AF aggregates all the local ML model information into a global ML model information and feedbacks the global ML model information to each UE.

-	Step 3: Each UE in the FL group derives the update local ML model information based on the global ML model information and its own raw data.

Therefore, in order to increase the efficiency of the FL model training, it is necessary to study how to minimize the latency divergence when the FL model training server receiving the local ML model from different UEs.

In summary, the following aspects need to be considered for 5GS assisted FL group performance monitoring:

-	How to avoid the impacts on other UEs services because of the application level FL model training.

-	How to minimize the latency divergence when the FL model training server receiving the local ML model from different UEs, in order to increase the training efficiency.

# * * * * End of changes * * * *