SA WG2 Meeting #S2-149E	S2-2201260

14 - 25 February, 2022, Electronic meeting

3GPP TSG-CT3 Meeting #119bis-e	C3-220400

# E-Meeting, 17th – 21st  January 2022

Title:	LS on handling of UE-Slice-MBR(s) change related to Allowed NSSAI change

Release:	Release 17

Work Item:	eNS_Ph2

Source:	CT3

To:	SA2

Cc:	-

Contact person:

Susana Fernández

Susana.fernandez@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

CT3 has specified the handling of UE-Slice-MBR changes reported to the PCF. Discussions have been held on the scenarios when the PCF gets informed about the "UE-Slice-MBR change" trigger.

According to TS 23.503, the PCF is informed about the subscribed UE-Slice-MBR for each S-NSSAI in the Allowed NSSAI at AM Policy Association creation. Since UDM stores the subscribed UE-Slice-MBR for each S-NSSAI, it is CT3 understanding that the AMF provides the PCF with the subscribed UE-Slice-MBR(s) for the S-NSSAIs that relate to the Allowed NSSAI only. The allowed NSSAI is deducted based on AMF internal configuration or information received from the NSSF.

According to the same specification, "the conditions for reporting of UE-Slice-MBR(s) changes are that the PCF provided Policy Control Request Trigger the AMF to report list of Subscribed UE-Slice-MBR. The AMF receives the authorized list of UE-Slice-MBR value for each S-NSSAI in the Allowed NSSAI from the PCF".

It is CT3 understanding that, when there is a change in the individual value of a subscribed UE-Slice-MBR related to a subscribed S-NSSAI in the UDM, the AMF will arm the UE-Slice-MBR change towards the PCF if the change is related to S-NSSAI(s) that are within the Allowed NSSAI.

However, CT3 does not come to a common understanding on the behaviour at the AMF and PCF when there is an Allowed NSSAI change, and as a consequence of that, the list of UE-Slice-MBR(s) for S-NSSAIs previously provided to the PCF do not apply any more or there are new S-NSSAIs for which the PCF have not been previously informed.

Question: Could SA2 clarify how the change of allowed NSSAI for a UE impacting the PCF provisioned UE-Slice-MBR(s) should be handled?

# 2	Actions

To SA2

ACTION: CT3 kindly asks SA2 to reply the question above and update their TSs if needed.

# 3	Dates of next TSG CT WG 3 meetings

3GPP TSG CT3#120-e	17th - 25th  February 2022	E-Meeting

3GPP TSG CT3#121-e	04th – 08th   April 2022	E-Meeting