3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202033

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Solution for KI#1: Mobile base station relay operation authorization using Forbidden Areas

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR / Rel-18

Abstract: A solution to solve the authorization for mobile base station relay operation using forbidden areas.

1. Introduction/Discussion

This contribution introduces a new solution for KI#1 to solve the authorization for mobile base station relay operation.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-05.

# * * * * First change * * * *

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |
| X | X |  |  |  |

# * * * * Second change * * * *

## 6.X	Solution #X: Authorization for mobile base station relay operation using Forbidden Areas

### 6.X.1	General

Mobile base station relays may perform the mobile base station relay operation depending on its location. When a mobile base station relay moves among different areas (e.g., Area A and Area B), it is possible that the operation of the mobile base station relay is allowed in Area A but not allowed in Area B.

For a Rel-16 IAB UE, 5GC supports the authorization of the IAB node. As described in TS 23.501 [2], during the IAB UE registration procedure, the UDM provides the IAB-Operation allowed indication to AMF. The AMF sends an indication that the IAB-node is authorized to the NG-RAN, via NGAP UE context setup or modification messages.

This solution proposes to reuse the design for the authorization of Rel-16 IAB node, and use the Forbidden Area to control the operation of mobile base station relay. The operator can configure the area(s) where the operation of mobile base station relay is not allowed as a Forbidden Area.

6.x.2	Functional descriptions

The existing functionality for a Forbidden Area is reused. The operator configures the area(s) where the operation of mobile base station relay is not allowed as a Forbidden Area.

6.x.3	Procedures

No new procedure is defined.

### 6.X.3	Impacts on services, entities, and interfaces

None.

# * * * * End of changes * * * *