SA WG2 Meeting #150E	S2-2201992r01

April 6 – 12, 2022, Elbonia

Source:	China Telecom, Ericsson, Samsung

Title:	New solution for NWDAF assisted URSP determination

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	eNA_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes a new solution for NWDAF assisted URSP.

# Discussion

This paper proposes a solution for KI#6 on NWDAF-assisted URSP. The corresponding Key Issue is described as:

NWDAF can already provide some network analytics to PCF for policy decisions as described in clause 6.1.1.3 in TS 23.503 [x]. However, further investigation is required on whether and how analytics can be used to assist in the generation of URSP Rules.

In this key issue, the following aspects need to be studied:

-	Whether any and which components of the URSP rules can benefit from analytics.

-	Whether and how existing Analytics IDs, or new Analytics ID(s) can be used to assist in the generation of URSP Rules.

-	What procedures trigger the subscription to these Analytics IDs.

-	Whether new (set of) interactions(s) are required to assist in the generation of URSP Rules as defined in Rel17, and how to define the new interactions if needed.

-	What information should be collected (or provided) as input (or output) by the NWDAF for these Analytics IDs

This solution proposes the enhancement on existing Analytics ID and how to use existing Analytics IDs to assist PCF on URSP determination.

# Proposal

Update TR 23.700-81 with the proposed changes below.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 6 |  |  |  |
| #X NWDAF assisted URSP determination | X |  |  |  |

# * * * * Next change * * * *

## 6.X	Solution #X: <NWDAF assisted URSP determination>

### 6.X.1	Description

This solution is proposed to address Key Issue #6: NWDAF-assisted URSP.

A URSP rule consists of traffic descriptor and route selection descriptor. Traffic descriptor is determined e.g., by application guidance from AF and is relatively fixed, which may not need analytics from NWDAF. This solution mainly proposes the existing analytics and enhanced analytics from NWDAF to assist to adjust the route selection descriptor and the related components of route selection descriptors can be reflected by Analytics ID(s). And the mapping between RSD components and output from Analytics IDs are showed in Table 6.X.1-1.

Table 6.X.1-1. NWDAF assisted URSP determination

| Route selection components | Analytics ID(s) |
| --- | --- |
| Network Slice Selection | "Service Experience", "Load level information", "Dispersion Analytics" |
| DNN Selection | "Service Experience", "UE Communication", |
| Non-Seamless Offload indication | "UE Communication", "WLAN performance", "Load level information", "Network Performance", "User Data Congestion" |
| SSC Mode Selection | "Service Experience"; Enhancement is needed |
| PDU Session Type | "Service Experience"; Enhancement is needed |
| Access Type preference | "Service Experience"; Enhancement is needed |
| Route selection validation criteria | Analytics ID(s) |
| Time Window | Based on the validity period and spatial validity provided in the Analytics ID used for RSD generation. |
| Location Criteria | Based on the validity period and spatial validity provided in the Analytics ID used for RSD generation. |

Editor’s note: Other existing Analytics IDs to assist in URSP generation is FFS.

Editor’s note: Other usages of NWDAF analytics for URSP is FFS.

The PCF generates a URSP Rule based on the Service Parameters provided by the AF, or alternatively locally configured in the PCF or both, then the PCF checks that the RSC of the URSP Rule complies with the UE context Policy control subscription information for a SUPI. Then, the PCF checks if Analytics are needed for further refinement of the candidate values.

In this solution, the PCF is configured, depending on operator policies, to subscribe to those Analytics ID(s) listed in the Table 6.X.1-1., then output of these Analytics are used to:

-	determine if traffic of the application is to be offloaded outside the PDU Session.

-	either prioritize (or remove some of) the list of candidate values for the Network Slice Selection component or for the DNN Selection component.

-	Allocate the SSC Mode value, and the PDU Session Type,

-	Select the preferred Access Type.

These are some examples of how the PCF assigns the values of the different Route Selection Components:

Network Slice Selection: PCF can request or subscribe to:

- Analytics ID "Service Experience" for a particular application that is part of the traffic descriptor, via the candidate network slices by setting filter information. The PCF may select or prioritize the network slice that provides the best service experience for the application.

- Analytics ID “Load Level Information” for the candidate network slices and select the one with lowest load.

- Analytics ID “Dispersion Analytics” to get, e.g. Data volume dispersion Analytics type. The PCF may calculate the average data rate in the network slice by subscribing to notifications of network analytics related to Data Volume Dispersion in the network slice for a duration of interest when it sets the Target of Analytics Reporting as "any UE" and the Analytics Filter as the S-NSSAI and an Area of Interest. The PCF may update URSP on slice selection policy for the associated UEs in the area of interest based on this notification from the NWDAF based on this notification from NWDAF.

- Both, first the list of candidate slices and the service experience it provides for the application is provided and then the load level for those slices are checked.

- Or just subscribe to Analytics ID "Service Experience" for a particular application, for slices that are not heavy loaded.

- Or just to subscribe to Analytics ID “Dispersion Analytics” as explained above for certain slice(s).

DNN Selection: Similar to the first bullet, The PCF may subscribe to Analytics ID on “Service Experience” for a particular application that is part of the traffic descriptor, via the candidate DNNs. The PCF may select or prioritize some of the DNN that provides the best service experience for the application. The PCF may subscribe to notifications of network analytics related to "UE Communication". The PCF may update the URSP on DNN selection policy for the associated SUPI, group of UEs or UEs associated with the Application Identifier based on this notification from NWDAF (e.g. considering Traffic characterization, Traffic Volume, Spatial activity or inactivity detection)

Non-Seamless Offload indication: PCF can request or subscribe "UE Communication" to get information on the location where UE will use the application and then get the information about the WLAN performance of this location by requesting or subscribing the Analytics ID "WLAN performance". If the WLAN performance is good enough, PCF may choose to configure this indication for the application in that area to reduce the load of 3GPP access. PCF may subscribe to notifications of network analytics related to "Network Performance". The PCF may update this indication within URSP of the associated UEs within the area of interest based on this notification from NWDAF (e.g. on gNB status and gnB resource usage information). PCF may subscribe to notifications of network analytics related to "User Data Congestion". The PCF may update this indication within URSP of the associated SUPI based on this notification from NWDAF (e.g. Network Status Indication).

SSC Mode Selection, PDU Session Type and Access Type preference: the PCF may subscribe to Analytics ID on “Service Experience” for a particular application that is part of the traffic descriptor, via the candidate SSC mode, PDU Session Type and Access Type preference. The PCF may select the SSC mode, PDU Session Type and Access Type preference that provides the best service experience for the application. these factors should also be captured by "Service Experience" Analytics. This solution enhanced the "Service Experience" Analytics in clause 6.X.1.1 and 6.X.1.2. SSC Mode Selection: the PCF may subscribe to Analytics ID on “Expected UE behaviour” for this SUPI. The PCF may select the SSC mode depending on the mobility pattern (e.g. If no mobility then the SSC mode is not relevant and can be omitted).

Time Window and Location Criteria: the Time Window in the RSD Validation Criteria can be set based on the time validity period and spatial validity in the Analytics ID(s) used as input in the RSD generation. The PCF may need to select the validity period and spatial validity out of those provided in the output of each Analytics ID(s).

#### 6.X.1.1	Input Data

Based on input data of Observed Service Experience related network data Analytics as described in clause 6.4.2 in TS 23.288[1], table 6.X.1.1-1 provides enhanced input data by adding PDU Session related information and table 6.X.1.1-2 provides enhanced input data access type.

Table 6.X.1.1-1 QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| UE ID | AMF | (list of) SUPI(s). If UE IDs are not provided as Target of Analytics Reporting for slice service experience, AMF returns the UE IDs matching the AMF event filters. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow. |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow. |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow. |
| UPF info | SMF | UPF ID/address/FQDN information for the UPF serving the UE. |
| DNAI | SMF | Identifies the access to DN to which the PDN session connects. |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow. |
| QFI | SMF | QoS Flow Identifier. |
| PDU Session type | SMF | Type of the PDU Session. |
| SSC Mode | SMF | SSC Mode selected for the PDU Session. |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| Packet transmission | UPF | The observed number of packet transmission. |
| Packet retransmission | UPF | The observed number of packet retransmission. |

Table 6.X.1.1-1: UE level Network Data from 5G NF related to the Service Experience

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| UE ID | AMF | (list of) SUPI(s). |
| Access Type | AMF | The Access type the UE camps on. |
| RAT Type | SMF | The RAT type the UE camps on. |

Editor’s note: whether and how to collect data from 4G access and non-3GPP access is FFS.

#### 6.X.1.2	Output Analytics

Based on output analytics of Observed Service Experience related network data Analytics for applications as described in clause 6.4.3 in TS 23.288[x], table 6.X.1.2-1 provides enhanced output analytics by adding PDU Session related information and access type.

Table 6.X.1.2-1 Service Experience Statistics

| Application service experiences (0..max) (NOTE 1) | List of observed service experience information for each Application. |
| --- | --- |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered (see NOTE 4). |
| > UPF Info | Indicating UPF serving the UE. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) with the same application service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > RAT Type | Indicating the RAT type for which the application service experience analytics applies. |
| > Frequency | Indicating the carrier frequency of UE's serving cell(s) where the application service experience analytics applies. |
| > SSC Mode | SSC Mode selected for the PDU Session used to associate with the application. |
| > PDU Session Type | Type of PDU Session used to associate with the application. |
| > Access Type | Access Type when the UE establishes a PDU Session for the application. |

### 6.X.2	Procedures

The same as procedures of related Analytics ID in TS 23.288 [x].

### 6.X.3	Impacts on Existing Nodes and Functionality

PCF:

- Subscribes or requests Analytics ID (s) on Table 6.X.1-1. consider them for URSP determination.

Editor’s note: Other Analytics IDs to assist in URSP generation is FFS.

NWDAF:

- supports enhanced Analytics ID "Service Experience".

Editor’s note: Other usages of NWDAF analytics for URSP is FFS.

SMF:

- Nsmf_EventExposure, PDU Session Establishment and/or PDU Session Release is extended to include the SSC mode per PDU Session.

# * * * * End of changes * * * *