3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202478

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	vivo

Title:	New Solution: Mobility procedures

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR / Rel-18

Abstract: this paper proposes a new solution for mobility performance.

1. Introduction

This solution addresses KI#2 “Efficient mobility for UEs connecting to/disconnecting from VMR” and KI#3 “Efficient mobility and service continuity when served by VMR”.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-05.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: “System architecture for the 5G System (5GS); Stage 2”.

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2"

[x]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2"

# * * * * Second change * * * *

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |
| Y |  | X | X |  |

# * * * * Third change (NEW TEXT) * * * *

## 6.Y	Solution #Y: Mobility procedures

### 6.Y.1	General

This solution addresses Key Issue #2 and Key Issue #3 for mobility.

This solution assumes that the TACs allowed to be broadcasted by the Mobile Base Station Relay (MBSR) are pre-configured, and the gNB-CU is able to instruct the Mobile Base Station Relay to broadcast one of them, which can be changed when the gNB-CU changed or determined by the gNB-CU. The pre-configuration of TACs for a MBSR is able to be obtained by the AMF serving the UE if the UE camps on the MBSR, and the Registration Area (RA) configured for the UE includes those TACs so that the UE will not perform mobility registration.

Following figure shows an example of moving scenario. In this example, 1) the UE first is serving by AMF1, the RA configured in UE does not include any TAC that is allowed to be broadcasted by any MBSR. 2) The UE onboards a vehicle equipped with a MBSR, the MBSR is serving by AMF2 and broadcast TAC1, which is not included in the current RA of the UE. The UE performs mobility registration via the MBSR, the AMF1 provides new RA to the UE, which includes TAC1 and TAC2 that is allowed to be broadcasted by the MBSR. 3) The UE moves along with the MBSR, the MBSR changes the serving AMF to AMF3 during the mobility, the MBSR is instructed to broadcast TAC2, but the UE will not perform mobility registration and the serving AMF still is AMF1 because the TAC2 is in current RA of the UE. During the mobility, the NG-RAN selects any of the allowed TAC for the MBSR to broadcast, which means the TAC may be reused during the mobility, a simple example is that only one TAC is allowed for a MBSR, so the broadcasted TAC for the MBSR will not change no matter where the MBSR moves to. 4) The UE gets off the vehicle, camps on macro cell, and the TAC broadcasted by the cell is not in the current RA of the UE, so the UE performs mobility registration, which changes the serving AMF to AMF4.

Figure 6.Y.1-1: Example of moving scenario

### 6.Y.2	Functional descriptions

The following figure shows the data model for the mobility management:

Figure 6.Y.2-1: Data model for mobility management

Each UE is served by an AMF (AMF serving UE, A.K.A. AMF-UE), which holds the UE Context. When the UE is relayed by a MBSR, the UE Context will include a Link ID linking to the MBSR that is dynamically assigned by the AMF. The Link ID includes AMF ID indicating the AMF assigning the Link ID, and includes a unique number assigned by the AMF indicated by the AMF ID. The UEs served by the same AMF and relayed by the same MBSR will be assigned with same Link ID. The UE Context also includes an AMF ID associating with the Link ID, the AMF ID indicates the AMF serving the MBSR. The 5G-GUTI included in the UE Context may be assigned by another AMF-UE if the UE does not perform mobility registration when moving with the MBSR (c.f. clause 6.Y.3.1, 6.Y.3.3, and 6.Y.3.4).

Each Mobile Base Station Relay (MBSR) is served by an AMF (AMF serving MBSR, A.K.A. AMF-MBSR), which holds the MBSR Context. The MBSR Context includes the dynamic MBSR ID (i.e., 5G-GUTI) that known by NG-RAN, and includes multiple Link IDs that dynamically assigned by AMFs serving the UEs that relayed by the MBSR. Different Link ID in the MBSR Context indicates different AMF.

The NG-RAN serving a MBSR holds the MBSR Context that includes one or more Link IDs, each Link ID associates with one or more UEs. In case the UE does not perform mobility registration when moving together with MBSR and AMF-UE changed, the 5G-GUTI of the UE may refer to the old AMF-UE, while the Link ID refers to the new AMF-UE (c.f. clause 6.Y.3.1, 6.Y.3.3, and 6.Y.3.4).

### 6.Y.3	Procedures

#### 6.Y.3.1	UE performs Service Request or Registration Request via Mobile Base Station Relay

Figure 6.Y.3.1-1: UE performs Service Request or Registration Request via Mobile Base Station Relay

1.	The UE camps on a MBSR.

2.	The UE initiates Service Request or Registration Request towards 5GC. The MBSR forwards the N1 message to the NG-RAN.

3.	The NG-RAN forwards the N1 message and includes the MBSR ID, e.g., 5G-GUTI of the MBSR, and may include the TAI of the MBSR in the underlayer N2 message. In the case of registration procedure, the NG-RAN may select a new AMF-UE.

4.	In case new AMF-UE is selected, the AMF-UE requests UE Context transfer from the old AMF-UE.

5.	When the UE Context includes a Link ID and associated AMF ID, if the associated AMF ID is different from the AMF ID in the received MBSR ID, the AMF-UE unsubscribes to the AMF-MBSR indicated by the associated AMF ID with the Link ID, meanwhile allocates a Link ID, which includes the AMF ID of the AMF-UE, associates the UE with the allocated Link ID, maps the allocated Link ID to the AMF ID in the received MBSR ID, and subscribes to the AMF-MBSR indicated by the received MBSR ID with the allocated Link ID and the received MBSR ID, the AMF-MBSR associates the MBSR with the Link ID.

If the associated AMF ID is same as the AMF ID in the received MBSR ID, the AMF-UE allocates a Link ID and performs the following:

-	If AMF-UE does not change, the AMF-UE update the subscription to the AMF-MBSR with the Link ID, the allocated Link ID, and the received MBSR ID (conditional update). If the MBSR indicated by the received MBSR ID has been subscribed with the Link ID, the AMF-MBSR returns an indication to the AMF-UE and keeps the subscription with the Link ID and the association between the Link ID and the MBSR, otherwise the AMF-MBSR decrease the count for the Link ID, associate the MBSR with the allocated Link ID, and increase the count for the allocated Link ID. If the indication is received, the AMF-UE releases the allocated Link ID and keeps the association between the Link ID and the UE, otherwise, the AMF-UE associates the UE with the allocated Link ID.

-	If AMF-UE changed, the AMF-UE unsubscribed to the AMF-MBSR with the Link ID, meanwhile the AMF-UE subscribes to the AMF-MBSR with the allocated Link ID and the received MBSR ID, associates the UE with the allocated Link ID, and associates the allocated Link ID with the AMF ID in the received MBSR ID. The AMF-MBSR maps the allocated Link ID to the MBSR.

When the UE Context does not include a Link ID, the AMF-UE allocates a Link ID, which includes the AMF ID, for the received MBSR ID, and subscribe to the AMF-MBSR with the allocated Link ID and the received MBSR ID.

When subscribe request with a Link ID is received, the AMF-MBSR increase the count for the Link ID, and when unsubscribe request with a Link ID, the AMF-MBSR decrease the count of the Link ID.

6.	The new AMF-UE sends a N2 message with the Link ID obtained in step 5, the N2 message may include a N1 message. In case of registration procedure, if the TAI of the MBSR does not change, for the sake of saving mobility registration for UEs behind the MBSR, the AMF-UE may send Registration Accept to the UE with TAI List that only includes the TAI received in step 3.

7.	The NG-RAN associates the Link ID with the MBSR ID and the UE ID. The NG-RAN sends an RRC message to the UE, the RRC message may include the N1 message. The usage of Link ID is refered to clause 6.Y.3.3, 6.Y.3.4, and 6.Y.3.5.

8.	If the UE camps on a MBSR, the UE may suspend the mobile registration if the UE keeps camping on the MBSR (e.g., the TAI/TAC broadcasted by the MBSR does not change and the NG-RAN sends the TAI to the AMF-UE).

9.	When the event met, the AMF-MBSR notifies the AMF-UE with the Link ID and associated information, c.f. clause 6.Y.3.3 and 6.Y.3.4.

10.	The UE camps on another MBSR or NG-RAN.

11.	When the mobile registration is suspended, the UE resumes the mobile registration (e.g., the TAI/TAC broadcasted by a MBSR is not provisioned to the UE before the UE camps on the MBSR and when UE camps on a MBSR, only the TAI/TAC of the MBSR is provisioned to the UE). If the UE camps on another MBSR, the steps 2-9 are performed with a different Link ID, different MBSR ID, and a possibly different AMF-MBSR, if the UE camps on NG-RAN directly, the registration procedure described in TS 23.502 [x] clause 4.2.2 is performed.

12.	If no MBSR ID is received and a Link ID and associated AMF ID exist in the UE Context, the AMF-UE unsubscribes towards the AMF-MBSR indicated by the associated AMF ID with the Link ID.

#### 6.Y.3.2	UE handover to connect/disconnect to/from Mobile Base Station Relay

6.Y.3.2.1	N2 handover

Figure 6.Y.3.2.1-1: UE N2 handover to connect/disconnect to/from Mobile Base Station Relay

1.	The UE is in CM-CONNECTED state and may be relayed by a MBSR.

2.	The source NG-RAN determines to perform handover for the UE and sends the Handover Required (UE info) message to the AMF-UE. The AMF-UE may select a new AMF-UE for the handover procedure. The UE info does not include MBSR ID.

3.	In case new AMF-UE is selected, the AMF-UE invokes Create UE Context Request (UE Context, UE info) towards the new AMF-UE. The UE Context does not include the Link ID and the associated AMF ID if the UE was relayed by a MBSR and no Link ID is received.

4.	The handover procedure for the UE continues.

5.	In case step 3 is performed, the new AMF-UE responds to the AMF-UE.

6.	In case step 3 is performed and Link ID is included in the UE Context, the AMF-UE unsubscribes to the AMF-MBSR indicated by the associated AMF ID with the Link ID.

7.	The AMF-UE sends Handover Command (UE info) to the target NG-RAN.

8.	The target NG-RAN sends the Handover Command to the UE.

9.	The UE tunes to the target NG-RAN, which may be via another MBSR.

10.	The target NG-RAN sends Handover Confirm to the (new) AMF-UE. If another MBSR is used, the target NG-RAN includes the MBSR ID and may include the TAI/TAC of the MBSR in the Handover Confirm.

11.	The handover procedure for the UE continues.

12.	If MBSR ID is received and the UE will not perform mobility registration, the AMF-UE allocates a Link ID for the received MBSR ID, and subscribe to the AMF-MBSR with the allocated Link ID and the received MBSR ID.

6.Y.3.2.2	Xn handover

Figure 6.Y.3.2.2-1: UE Xn handover to connect/disconnect to/from Mobile Base Station Relay

1.	The UE handover to leave a MBSR from direct NG-RAN connection or from another MBSR, or to connect to a MBSR from direct NG-RAN connection or from another MBSR. The Xn handover is performed

2.	The target NG-RAN sends the Path Switch ([MBSR ID], UE info) message to the AMF-UE. If the UE handover to directly connect to NG-RAN, the MBSR ID is not included. If the UE handover to a MBSR, the MBSR ID is included.

3.	Considering that UE will not handover to the same MBSR, the new AMF-UE may perform conditional subscription update as described in step 5 of clause 6.Y.3.1, or unsubscribe and then subscribe to the AMF-MBSR as described in step 5 of clause 6.Y.3.1.

4.	The handover procedure for the UE continues. The AMF-UE sends the Link ID obtained in step 3.

#### 6.Y.3.3	Mobile Base Station Relay mobility registration procedure

Figure 6.Y.3.3-1: Mobile Base Station Relay mobility registration procedure

1.	The MBSR sends Registration Request towards the 5GC via the NG-RAN.

2.	The NG-RAN may select a AMF-MBSR different from the old AMF-MBSR, the AMF-MBSR queries UE Context for the MBSR from the old AMF-MBSR and the old AMF-MBSR returns MBSR Context with the Link IDs.

3.	If AMF-MBSR is changed, the AMF-MBSR notifies the AMF change event with the Link ID to the AMF-UE. The AMF-UE associates the Link ID with the AMF ID of the AMF-MBSR.

4.	The AMF-MBSR sends Registration Accept to the MBSR, whose underlayer N2 message includes the Link ID.

5.	The NG-RAN decides whether to change the AMF serving the UEs associated with a Link ID according to the Link ID.

6.	If new AMF-UE is selected, the NG-RAN sends AMF Relocation Request (Link ID) to the new AMF-UE.

7.	The new AMF-UE invokes Bulk UE Context Transfer Request (Link ID) towards the AMF-UE according to the Link ID. The AMF-UE responds the context of UEs associated with the Link ID. The UE context includes the Link ID and the associated AMF ID indicating the AMF-MBSR that the AMF-UE has subscribed event to.

8.	The new AMF-UE allocate a new Link ID, associates the new Link ID with the UEs, and responds to the NG-RAN with the new Link ID. The NG-RAN replace the Link ID with the new Link ID.

9.	The new AMF-UE updates the subscription with the Link ID and the new Link ID (unconditional update) towards the AMF-MBSR associated with the Link ID. The AMF-MBSR replace the Link ID with the new Link ID.

#### 6.Y.3.4	Mobile Base Station Relay handover with UEs

Editor's note:	The MBSR handover together with UEs needs coordination with RAN WGs.

6.Y.3.4.1	N2 handover

Figure 6.Y.3.4.1-1: Mobile Base Station Relay N2 handover with UEs

1.	The source NG-RAN determines to perform handover for the MBSR and sends the Handover Required (MBSR info) message to the AMF-MBSR. The AMF-MBSR may select a new AMF-MBSR for the handover procedure.

2.	The source NG-RAN also sends Handover Required (UE info (MBSR ID), Link ID) per the UE associated with the MBSR towards the AMF-UE according to the Link ID. The target NG-RAN can combine multiple Handover Required messages in a bulk way sending to the same destination. The Link ID indicates that the UE is moving together with the MBSR associated with the Link ID.

The AMF-UE obtains the context of UEs associated with the Link ID, and performs following steps 3-8 per UE.

3.	The AMF-UE may determine to select a new AMF-UE for the specific UE, and invokes Create UE Context Request (UE Context (Link ID, AMF ID), UE info) towards the new AMF-UE. The UE info includes the MBSR ID, the UE Context includes the Link ID and the associated AMF ID indicating the AMF-MBSR.

4.	The handover procedure for the UE continues. In case new AMF-UE is not selected and Link ID is included in step 3, the AMF-UE sends the Link ID and the UE info, which includes the MBSR ID, to the target NG-RAN. In case new AMF-UE is selected and Link ID is included in step 3, if the new Link ID related to the Link ID has not been allocated, the new AMF-UE allocates a new Link ID. The new AMF-UE sends the new Link ID and the UE info, which includes the MBSR ID, to the target NG-RAN. The target NG-RAN associates the MBSR with the Link ID, or the new Link ID if received.

5.	In case step 3 is performed, the new AMF-UE responds to the AMF-UE.

6.	In case step 3 is performed, the new AMF-UE updates the subscription towards the AMF-MBSR with the Link ID and the new Link ID (unconditional update).

7.	If the AMF-MBSR is in handover procedure, when the AMF-MBSR does not select a new AMF-MBSR, the AMF-MBSR decreases the count for the Link ID, increases the count for the new Link ID, and associates the MBSR indicated by the Link ID with the new Link ID. When the AMF-MBSR selects a new AMF-MBSR in step 1, the AMF-MBSR forwards the subscription update towards the new AMF-MBSR, and the new AMF-MBSR decreases the count for the Link ID, increases the count for the new Link ID, and associates the MBSR indicated by the Link ID with the new Link ID.

8.	The AMF-MBSR responds to the subscription update.

9.	The handover procedure for the MBSR continues.

10.	If the new AMF-MBSR is selected, the new AMF-MBSR gets the MBSR context during the handover procedure, which includes the Link ID. The new AMF-MBSR notifies the AMF change event with the Link ID. If the new AMF-MBSR receives subscription update in step 7, the new AMF-MBSR notifies the AMF change event with the new Link ID.

11.	The AMF-UE sends Handover Command (UE info) to the source NG-RAN. The UE info includes the associated MBSR info.

12.	The AMF-MBSR sends the Handover Command (MBSR info) to the source NG-RAN.

13.	The source NG-RAN aggregates the Handover Command for the MBSR and the Handover Command for the UEs, and sends Handover Command to the MBSR with MBSR info and UEs info. The MBSR holds all UL packets of the UEs until the MBSR tunes to the target NG-RAN.

If there’re some updates on a UE’s resources, the source NG-RAN also sends Handover Command to the UE via the MBSR.

14.	The MBSR confirms the handover with the target NG-RAN. If there’re some updates on a UE’s resources, the UE confirms the handover, which will be forwarded by the MBSR to the target NG-RAN.

15.	The target NG-RAN sends Bulk Handover Notify to the (new) AMF-UE.

16.	The handover procedures for the UEs continue.

17.	The target NG-RAN sends Handover Notify to the (new) AMF-MBSR.

18.	The handover procedure for the MBSR continues.

6.Y.3.4.2	Xn handover

Figure 6.Y.3.4.2-1: Mobile Base Station Relay Xn handover with UEs

1.	The MBSR performs Xn handover together with the UEs.

2.	The target NG-RAN sends the Path Switch (MBSR info) message to the AMF-MBSR.

3.	The target NG-RAN also sends Path Switch (UE info (MBSR ID), Link ID) per the UEs associated with the MBSR towards the AMF-UE according to the Link ID. The target NG-RAN can combine the Path Switch messages in a bulk way sending to the same destination. The Link ID indicates that the UE is moving together with the MBSR associated with the Link ID.

4.	The AMF-MBSR and the AMF-UE continues the Xn handover procedure for MBSR and UEs respectively.

#### 6.Y.3.5	Mobile termination when UE moves with Mobile Base Station Relay

Figure 6.Y.3.5-1: Mobile termination when UE moves with Mobile Base Station Relay

1.	UE moves with the MBSR, the AMF-UE is triggered to send N1 message or N2 message to UE.

2.	(a) If the UE is in CM-CONNECTED state, the AMF-UE sends the N1/N2 message towards the NG-RAN. If N1 message is received, the NG-RAN sends the N1 message to the UE via the MBSR.

(b) If the UE is in CM-IDLE state and the UE context includes a Link ID, as well as the AMF-UE knows the serving RAN node for the MBSR related to the Link ID (e.g., the MBSR just performed registration with AMF-UE change, c.f. 6.Y.3.3, or the MBSR just performed handover), the AMF-UE may send Paging request (UE ID, Link ID) to the NG-RAN. The NG-RAN maps the Link ID to the MBSR.

(c) The AMF-UE may invoke Page Relay (Link ID, UE ID) towards the AMF-MBSR.

3.	If 2c is performed, the AMF-MBSR maps the Link ID to the MBSR. The AMF-MBSR sends the Paging request (UE ID) to the NG-RAN.

4.	The NG-RAN sends the Paging request (UE ID) to the MBSR, and the MBSR pages the UE.

### 6.Y.4	Impacts on services, entities and interfaces.

AMF (serving UE):

-	Support mapping UE context with Link ID.

-	Support invoking (un)subscribe operation with Link ID and MBSR ID for AMF change event towards the AMF serving MBSR, and receiving notification of AMF change from AMF serving MBSR.

-	Support sending and receiving Link ID along with the UE information to/from NG-RAN.

-	Support sending Registration Area information to UE according to the TAI list configuration of MBSR received from NG-RAN.

-	Support bulk UE context transfer operation according to Link ID.

-	Support paging UE with Link ID and/or redirecting the Paging message to AMF serving MBSR.

AMF (serving MBSR):

-	Support mapping MBSR context with Link ID.

-	Support (un)subscribe operation with Link ID and MBSR ID for AMF change event, and sending notification of AMF change to subscriber.

-	Support redirecting the Paging message from AMF serving UE to the NG-RAN serving the MBSR.

NG-RAN:

-	Support mapping MBSR context with Link ID.

-	Support sending and receiving Link ID along with the UE information to/from NG-RAN.

-	Support aggregating handover procedures for UEs and MBSR.

Editor's note:	Additional impacts are FFS.

# * * * * End of changes * * * *