3GPP TSG-WG SA2 Meeting #150E e-meeting  	S2-2202536

# Elbonia, April 6 – 12, 2022	(revision of S2-2200882)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0090 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of MBS data forwarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Per the incoming LS R3-222867, it is the suitable time for SA2 to coordinate with RAN3 to specify the detail for MBS data forwarding.Therefore, it is proposed to specify the detail of MBS data forwarding in 23.247. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 7.2.3.2 and 7.2.3.3, the detail of how to perform the MBS data forwading, add a reference to clause 7.2.3.5. Also, remove the “Editor’s Note Details on data forwarding, if applicable, needs to wait for RAN WGs” . - Per the RAN3 progres, in clause 7.2.3.5, add some detail descriptions to further clarify how to support MBS data forwarding and how to minimize data loss. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The detail of how to perform data forwading is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.2, 7.2.3.3, 7.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.2.3.2	Xn based handover from MBS supporting NG-RAN node

This clause describes an Xn based handover with MBS traffic delivered to the UE at the source NG-RAN node supporting MBS.

Figure 7.2.3.2-1: Xn based handover with MBS Session

The following additions apply compared to clause 4.9.1.2 of TS 23.502 [6]:

Before Handover:

The source NG RAN has been provided with MBS Session Resource information (including the MBS Session ID and multicast QoS flow information) and the UE Context information contains a mapping information within the PDU Session Resource associated with the MBS Session Resource, e.g. including mapped unicast QoS Flows associated with the multicast QoS flow(s) of the MBS Session Resource.

Handover Preparation Phase:

At Xn handover, the target NG-RAN is provided with MBS session information by the source NG-RAN which causes:

-	an MBS non-supporting target NG-RAN node to prepare the unicast resources according to associated QoS flow(s) information.

‐	an MBS supporting target NG-RAN node to allocate to the UE shared NG-RAN resources according to the MBS session information. If the 5GC Shared MBS traffic delivery for the indicated multicast MBS Session has not been established in target NG-RAN, target NG-RAN triggers setup of the resources for the 5GC Shared MBS traffic delivery, see clause 7.2.1.4 for details.

1.	Target NG-RAN to AMF: the target NG-RAN sends N2 Path Switch Request to AMF.

The target NG-RAN node, if MBS-capable, indicates it supports of MBS to SMF in N2 SM information. Per the received N2 SM information, the SMF knows whether the target NG-RAN node supports MBS and determines the delivery method, i.e., whether the 5GC Shared MBS traffic delivery or 5GC Individual MBS traffic delivery is used for MBS data transferring.

The SMF differentiates two cases:

Case A) The target NG-RAN supports MBS. Step 3 applies and step 4 is skipped.

3.	SMF to UPF: The SMF invokes N4 Session Modification procedure with the UPF (PSA) only for unicast PDU Session.

Case B) The target NG-RAN does not support MBS. Step 3 is skipped, step 4 applies.

4.	This steps is same as described in step 11 of clause 7.2.1.3.

The details of how to perform data forwarding refers to clause 7.2.3.5.

5.	SMF to AMF: The SMF responds to AMF through Nsmf_PDUSession_UpdateSMContext response.

6.	AMF to target NG-RAN: The AMF sends the path switch Ack to target NG-RAN.

#### 7.2.3.3	N2 based handover from MBS supporting NG-RAN node

This clause describes the N2 based handover with MBS traffic delivered to the UE at the source NG-RAN node supporting MBS.

Figure 7.2.3.3-1: N2 based handover with MBS Session

The following additions apply compared to clause 4.9.1.3 of TS 23.502 [6]):

1.	Source NG-RAN to S-AMF: Handover Required (RAN container (MBS Session information, associated PDU session information, associated QoS flow information and corresponding multicast QoS flow information)).

2.	S-AMF to T-AMF: The T-AMF is provided with associated PDU Session information and the MBS session related information.

4.	T-AMF to Target NG-RAN: The Target NG-RAN prepares the radio resource based on the received information:

-	If the Target NG-RAN does not support MBS, the MBS Session related information is not used. The Target NG-RAN uses the associated PDU Session information to allocate resource to deliver MBS data. The MBS data are transmitted via the associated QoS flows within the associated PDU Session.

-	If the Target NG-RAN supports MBS, the Target NG-RAN uses the multicast MBS Session related information to allocate RAN resources to deliver the MBS data. If 5GC Shared MBS traffic delivery for the indicated multicast MBS session has not been established towards the Target NG-RAN, the Target NG-RAN initiates the shared delivery establishment towards the MB-SMF via AMF as described in clause 7.2.1.4.

5.	Target NG-RAN to T-AMF: The target NG-RAN sends Handover Request Ack to T-AMF.

The target NG-RAN node, if MBS-capable, indicates it supports MBS to SMF in N2 SM information. Per the received N2 SM information, the SMF knows whether the target NG-RAN node supports MBS and determines the delivery method, i.e. whether the 5GC Shared MBS traffic delivery or 5GC Individual MBS traffic delivery is used for MBS data transferring.

12.	T-AMF to SMF: The AMF invokes Nsmf_PDUSession_UpdateSMContext request towards SMF, the message includes the received N2 SM information received from the target NG-RAN.

13-14.	Same as described in steps 3-4 of clause 7.2.3.2.

The details of how to perform data forwarding, refers to clause 7.2.3.5.

15.	SMF to T-AMF: The SMF sends the Nsmf_PDUSession_UpdateSMContext Request to the T-AMF.

# * * * * Second change * * * *

7.2.3.5	Minimization of data loss

To minimize the data loss of the UE during the handover procedure the following functions apply:

-	For each MBS QoS flow, the MB-UPF adds a sequence number in each data packet of the MBS session sent by the MB-UPF and forwarded to all related NG-RAN nodes and UPFs via GTP-U tunnel. When the UPF forwards the received MBS data packet, the sequence number shall not be changed. The NG-RAN node uses the received sequence number to determine the related PDCP SN, i.e. the sequence number at PDCP layer.

-	If both the source NG-RAN node and target NG-RAN node support MBS, to minimize data loss the MBS session either or a combination of the following methods may be applied, as specified in TS 38.300 [9]:

-	Data may be forwarded from source NG-RAN node to target NG-RAN node. In handover preparation phase, the MBS progress information (i.e., PDCP SN) is exchanged between source NG-RAN node and target NG-RAN node. The target NG-RAN node uses MBS progress information received from source NG-RAN node to decide whether the data forwarding path needs to be established. The source NG-RAN node uses the MBS progress information received from target NG-RAN node to decide when to stop forwards the MBS data.

-	NG-RAN nodes share a common user plane entity, denoted as shared NG-U termination in TS 38.300 [9], which allows the allocation of identical PDCP sequence numbers to MBS users data when delivered to UEs in cells served by different NG-RAN nodes.

-	If source NG-RAN node supports MBS and target NG-RAN node does not support MBS, the multicast MBS session data is forwarded from source NG-RAN node to target NG-RAN node via the associated QoS flow within the associated PDU session.

To stop the MBS data forwarding, when the individual delivery is enabled, the SMF instructs the UPF to construct the end marker with the associated QoS flow QFI(s) and the MBS data packet sequence number, and to send the end marker packet to the source NG-RAN node via the associated PDU session tunnel. The sequence number of end marker is set based on the sequence number of the first MBS data packet to be sent to the target NG-RAN node.  Based on the associated QFI in the end marker packet and the associated PDU session, the source NG-RAN node determines the corresponding MBS QoS flow for which data forwarding can be stopped.

To minimize data loss, the source NG-RAN node compares the data packet sequence number from the MBS session and the sequence number of the end marker packet. Based on the comparison, the source NG-RAN node determines when to stop sends the MBS data to the target NG-RAN node.

-	If source NG-RAN node does not support MBS and target NG-RAN node supports MBS, for Xn/N2 handover, the multicast MBS session data is forwarded to the target NG-RAN node as the data forwarding mechanism defined in TS 23.502 [6]. Directly after the handover the target NG-RAN node thus still receives MBS session data via individual delivery. The UPF forwards multicast MBS session data within the associated PDU session including the sequence number received from the MB-UPF to the target NG-RAN node. Shared delivery of MBS data towards the target RAN node is being established as described in Clause 7.2.3.4, and the target RAN node receives sequence numbers as part of the MBS data with sequence numbers via shared delivery.

NOTE:	For N2 handover, the detail mechanism of how to transmit the received forwarded multicast MBS session data to UE is defined by RAN WGs.

# * * * * End of changes * * * *