SA WG2 Meeting #150e	S2-2202596r05

# April 06th – 12th, 2022; Elbonia               		  								(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0102 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AMF and AUSF selection for CP authentication and authorisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Interdigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR addresses some issues related with the Layer-3 5G ProSe relaying CP security solution that were brought up during discussion in TSG SA #95. AUSF selection is handled via specifyin an exception to TS 23.501 in TS 23.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remote UE information as AUSF selection criteria is moved from TS 23.304 clause 5.1.4.3.2 to TS 23.501 clause 6.3.4. Title and scope update to avoid conflict with another CR in S2-2202104. Per-user basis authorisation using GPSIs is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Layer-3 CP relay authentication solution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.1.4.3.2	Control Plane based security procedures for 5G ProSe UE-to-Network Relay

Control Plane-based security procedures for 5G ProSe UE-to-Network Relay call flow and procedure is defined in TS 33.503 [29]. It is characterised by the following principles:

-	5G ProSe UE-to-Network Relay's NAS signalling is used for the 5G ProSe Remote UE's authentication and authorisation.

-	The AMF serving the 5G ProSe UE-to-Network Relay selects AUSF as specified in TS 23.501 [4] clause 6.3.4.If the AUSF cannot be selected in roaming case, the procedure fails.

NOTE:	In security procedure failure case, the 5G ProSe Layer-3 Remote UE can select other 5G ProSe Layer-3 UE-to-Network Relay.

##### -	In order to support authorisation of 5G ProSe UE-to-Network relaying on per user basis, the Remote UE's subscriber data includes the GPSIs of the authorised 5G ProSe UE-to-Network Relay UEs as specified in TS 33.503 [29].