SA WG2 Meeting #150e	S2-2202612r01

# April 6th – 12th, 2022; Elbonia               		      (revision of S2-220xxxx)

Source: 	NTT DOCOMO

Title: 	New Solution for KI#2: Enhance group status event reporting

Document for: 	Approval

Agenda Item: 	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract of the contribution: This contribution proposes a new solution for Key Issue #2.

# 1	Discussion

This document proposes to add a new solution for Key Issue #2: Enhance group status event reporting. The Key Issue #2 aims to address the following points:

-	How to enable the group status event reporting via exposure interface, i.e.:

-	What is the subscription information for the event "newly registered or (de)-registered group member";

-	What is the trigger to detect such event;

-	What is the information within the event reporting to reflect the changes of group status.

# 2 Proposal

It is proposed to include the following content under the description of KI#4 in TR 23.700-74.

*** BEGIN CHANGES ***

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 |
| #1 | X | X |  |  |  |
| #2 |  |  | X |  |  |
| #3 |  |  |  | X |  |
| #4 |  |  |  | X |  |
| #5 |  |  |  | X |  |
| #6 |  |  |  |  | X |
| #7 |  |  |  |  | X |
| #X |  | X |  |  |  |

*** NEXT CHANGE (all text is new) ***

## 6.X	Solution #X: Group status event reporting

### 6.X.1	Introduction

This solution aims to address the KI#2, i.e. to enhance group status event reporting in order to allow subscription to group status event reporting for the event "newly registered or (de)-registered group member". Newly registered or de-registered group member should be notified to the AF that has subscribed for changes on the members of the group, i.e. when a new member is added or old member is removed from the group as identified by External Group ID. This may occur e.g. when the corresponding Internal Group ID is removed from the user subscription data by OAM.

### 6.X.2	Functional Description

The solution is based on the following principles:

-	The AF can subscribe for changes in the 5G VN group data. This is done by including a Notification Target Address into the Nnef_ParameterProvision_Create request when the 5G VN group is created. The NEF then notifies the AF by invoking Nnef_ParameterProvision_UpdateNotify service operation, and includes the current content of the 5G VN group data to the notification.

-	If NEF receives 5G VN group creation (Nnef_ParameterProvision_Create) request with a Notification Target Address, the NEF includes its own Notification Target Address to the Nudm_ParameterProvision_Create request when the NEF creates the 5G VN group into UDM. Whenever the UDM determines the 5G VN group data has changed, e.g. because the Internal Group ID is removed from a member's user subscription data by OAM, the UDM invokes Nudm_ParameterProvision_UpdateNotify to the given Notification Target Address, and includes the current 5G VN group data to the notification. The NEF may further notify the AF via Nnef_ParameterProvision_UpdateNotify.

-	If the AF wants to subscribe for any of the monitoring event(s) as described in Table 4.15.3.1-1 in TS 23.502 [3] (e.g. UE reachability or PDU Session status) for the members of the group, the AF can use the existing Nnef_EventExposure service and include the External Group ID as a target to the subscription. In this case the NEF subscribes to the notification for the corresponding monitoring event(s) for each UE group member within the 5GC using existing mechanism as defined in clause 4.15.3 of TS 23.502 [3].

-	The AF may include a Group Reporting Guard Time (as in Table 4.15.1-1 in TS 23.502 [3]) into the Nnef_EventExposure_Subscribe request that is targeted to an External Group ID. In this case the NEF shall accumulate all of the received Monitoring Event reports for the group of UEs until the Group Reporting Guard Time expires or the monitoring duration is reached. No impacts to the existing procedures are foreseen.

Editor's note: Other services other than Parameter Provision service to support subscription/notify for membership changes of a group may also be available and can be evaluated or investigated later.

### 6.X.3	Procedures

### 6.X.4	Impacts on existing entities and interfaces

-	UDM: new service operation Nudm_ParameterProvision_UpdateNotify allows the UDM to notify the NF consumer (NEF) on changes for the 5G VN group data.

-	NEF: new service operation Nnef_ParameterProvision_UpdateNotify allows the NEF to notify the NF consumer (AF) on changes for the 5G VN group data.

*** END OF CHANGES ***