SA WG2 Meeting #S2-150e(e-meeting)	S2-2202622r01

06 - 12 April 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	CATT

Title:	KI#3, New Sol: LMF selection based on GMLC service area

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3

Abstract of the contribution: Introduce a new solution to key issue#3.

# 1. Discussion

This pCR is proposed to add a new solution to key issue#3:

- KI#3: Local Area Restriction for an LMF and GMLC

Based on the current LMF selection mechanism in TS 23.273, the AMF or LMF selects LMF based on the locally available information or by querying NRF. The following factors may be considered during the LMF selection:

-	LCS client type.

-	Requested Quality of Service information, e.g.:

-	LCS accuracy,

-	Response time (latency),

-	Access Type (3GPP /N3GPP).

-	RAT type (e.g. 5G NR, eLTE, or any of the RAT Types specified for NR satellite access) and/or the serving AN node (i.e. gNB or NG-eNB) of the target UE.

-	RAN configuration information.

-	LMF capabilities.

-	LMF load.

-	LMF location.

-	Indication of either a single event report or multiple event reports.

-	Duration of event reporting.

-	Network slicing information, e.g. S-NSSAI and/or NSI ID.

-	LMF Service Area consisting of one or more TA(s).

-	Supported GAD shapes.

In order to support GMLC and LMF which are restricted to supporting local services in a local area, it is proposed that GMLC service area is also considered during the existing LMF selection mechanism.

# 2. Proposal

This paper proposes to add a new solution to key issue#3.

* First change *

## 6.0	Mapping Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  |  |  |  |
| X | KI#3 |  |  |  |

* Second change *

## 6.X	Solution #x: LMF Selection based on GMLC service area

### 6.X.1	Introduction

This solution addresses KI#3: Local Area Restriction for an LMF and GMLC.

### 6.X.2	Functional Description

The existing LMF selection mechanism is re-used with the following enhancements:

-	When AMF selects LMF: a new factor considered for LMF selection is added, i.e. GMLC service area consisting of one or more TA(s) which is locally configured in AMF.

-	When AMF selects target LMF: if the GMLC is restricted to supporting local services in a local area, it selects target LMF able to support location in the same local area.

### 6.X.3	Procedures

The existing LMF Change Procedure in clause 6.4 in TS 23.273 is re-used.

### 6.X.4	Impacts on services, entities, and interfaces

AMF: consider GMLCS service area when selecting LMF.

LMF: if the LMF is restricted to supporting local services in a local area, it selects target LMF in the same local area.

* End of changes *