3GPP TSG-WG SA2 Meeting #150E e-meeting	S2-2202658r01

e-Meeting, 6th-12th Apr, 2022

# Title:	[Draft] Reply LS on Slice list and priority information for cell reselection

# Response to:	S2-2201947(R2-2203807)

# Release:	Rel-17

# Work Item:	NR_Slice_Core

Source:	SA2

To:	RAN2, RAN3, CT1,CT4

Cc:

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu dot Jinguo at ZTE dot com dot cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	23.501 CR3317，23.501 CR3539, 23.502 CR3300

1. Overall Description:

SA2 would like to inform RAN2, RAN3, CT1, CT4 about SA2 progress of supporting Slice Groups and Network Slice priorities required for enabling RAN Slicing as per Work Item NR_Slice-Core.

SA2 confirmed that the mapping of slice to the slice group is per TA, and any slice group priority is sent to the UE over NAS message by the AMF based on the S-NSSAIs priority received in subscription information. The UE shall use the Slice group priority received from the AMF and if no priority is indicated the UE shall assume the Slice groups have equal priority.  SA2 conditionally approved the attached CRs with the condition that the RAN can approve the following dependency:

In order to support per TA granularity, the RAN should be capable to provide the UE with frequency priority per slice group with a slice group associated with a TA

The RAN should report the slice group information per slice per TA basis to the AMF

SA2 also reminds that the support of network sharing is required for all features so an optional PLMN index indication or a similar concept should be considered to be added as part of the slice group format.

We expect that the cell to be able to associate a Slice group with its TAC

For your information in SA2 the slice group is called NSAG (Network Slice AS Group).

SA2 has discussed the storage requirements in the UE and suggest this is UE implementation based whether to store the configuration for more than one PLMN and the UE may store the configuration for equivalent PLMNs. A UE configuration should at least support storing 16 NSAGs and it priorities in a PLMN configuration.

2. Actions:

To: RAN2 and RAN3 CT1 and CT4

ACTION:

SA2 kindly asks to take the information above into consideration for the further development of related specifications.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #151E                               May 16-20, 2022

TSG-SA2 Meeting #152                                 Aug 17-26, 2022