SA WG2 Meeting #150e	S2-2202820r02

# April 6th – 12th, 2022; Elbonia               		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell, Lenovo, Motorola Mobility

Title: 	23.700-41: Solution for KI#3; configuring the UE with per network slice preferred PLMNs

Document for: 	Approval

Agenda Item: 	9.14

Work Item / Release:	FS_eNS_ph3/ Rel-18

Abstract of the contribution:

# 1 Discussion

This pCR proposes a solution to KI#2 in TR 23.700-41, regarding how to configure the VPLMN preferences.

# 2 Proposal

It is proposed to update 23.700-41 as follows

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| Solution X: configuring the UE with network slice aware preferred PLMNs lists |  | x |  |  |  |  |

NEXT CHANGE (2) ALL text is new

## 6.X	Solution #X: configuring the UE with network slice aware preferred PLMNs lists

### 6.X.1	Introduction

This solution aims to address the key issues#2: Support of providing VPLMN network slice information to a roaming UE by enhancing SoR with slice aware preferred PLMN lists.

## 6.X.1	Functional Description

It is normal practice for operators to configure the UE with a (set of) preferred PLMN(s) in a certain country while the UEs are roaming. In the 5GS this set of preferences may e.g. be configured in the UE using the SoR (Steering of roaming) procedure defined in TS 23.502[x] and TS 33.501[y]

TS 23.502 reads:

" During registration the Home Network can provide Steering of Roaming information to the UE via the AMF (i.e. a list of preferred PLMN/access technology combinations or HPLMN indication that 'no change of the "Operator Controlled PLMN Selector with Access Technology" list stored in the UE is needed). The Home Network can include an indication for the UE to send an acknowledgement of the reception of this information. Details regarding the handling of Steering of Roaming information including how this information is managed between the AMF and the UE are defined in TS 23.122 [22]."

It can happen the operator wants the UE to remain with a list of preferred PLMNs  (i.e. PLMNs in the "User Controlled PLMN Selector with Access Technology" and the "Operator Controlled PLMN Selector with Access Technology"), unless the UE requires to use the network slice which these PLMNs do not support. Also, the operator may have specific preferred PLMN list when specific network slices are used. This is not possible today with the current PLMN selection mechanisms as the UE will, in automatic PLMN selection, always remain with the preferred PLMNs (in respective priority order) if they are available.

This solution proposes to configure the UE with such additional network slice aware PLMN selection information, in the form of one or more lists of PLMNs that are preferred when a respective S-NSSAI of the HPLMN is required to be used by the UE. The S-NSSAI values in this information are the HPLMN S-NSSAIs (i.e. the mapped value of the S-NSSAIs in the V-PLMN are not considered, and so there is no impact on this solution if the VPLMNs decide to change the value of S-NSSAI they use to identify the network slice locally). The UE therefore can be configured with the preferred PLMN list as today and additional Network-Slice-Aware PLMN lists per S-NSSAI which override, as an exception, any preferred PLMN list if any is provided to the UE. In other words, the UE may be configured with:

-	nothing (i.e. neither "Operator Controlled PLMN Selector with Access Technology" nor Network Slice Aware PLMN list);

-	just a preferred PLMNs list (e.g. "Operator Controlled PLMN Selector with Access Technology");

-	a mix of preferred PLMN list and at least one Network-Slice-Aware PLMN list; or

-	at least one Network-Slice-Aware PLMN list.

The UE is configured with a Network-Slice-Aware PLMN list of alternate (potentially ordered by a priority value) PLMN IDs associated with the supported S-NSSAIs. Some of the S-NSSAIs may not be supported (or are preferred instead of) by the PLMNs in the "Operator Controlled PLMN Selector with Access Technology" list.

When the Network-Slice-Aware PLMN list is provided for an S-NSSAI and this S-NSSAI is required but not yet allowed or rejected, this is a trigger for evaluation by the UE whether to consider the Network-Slice-Aware PLMN list to perform PLMN selection.

If the Network-Slice-Aware PLMN list is associated with validity parameters (e.g. location in particular country or time period restriction), the UE applies the configuration when the validity restriction is fulfilled.

When attempting to use/register with a S-NSSAI associated with a Network-Slice-Aware PLMN list, the UE prefers and selects one of the PLMNs identified in the Network-Slice-Aware PLMN list if a PLMN ID is present in such list for this S-NSSAI that is of higher priority of the current PLMN, or if the PLMN ID of the current PLMN is not among the PLMN IDs in the Network-Slice-Aware PLMN list for the S-NSSAI.  The UE may return to using the ordinary list of preferred PLMNs when it deregisters from all S-NSSAIs associated to any Network-Slice-Aware PLMN lists it is configured with, so in a sense the UE uses the Network Slice Aware PLMN list when an S-NSSAI is not available in currently registered PLMN.

It is assumed that the UE at any time selects the PLMN ID which maximises the number of S-NSSAIs that the UE can use.  If all S-NSSAIs can be used in a set of PLMNs, the UE selects the common PLMN ID across all the Network-Slice-Aware PLMN list which ranks higher on average. This average may also consider predefined weighted factors assigned to each Network Slice. An example of a UE configuration with set of Network-Slice-Aware PLMN list is in figure 6.x.1-1.  Alternatively, the UE can create internally a list of preferred S-NSSAIs and can select a PLMN ID (from the Network-Slice-Aware PLMN list) which serves the preferred S-NSSAIs.

| Weight | S-NSSAI | PLMN ID list with priority |
| --- | --- | --- |
| 0.5 | S-NSSA1 1 | PLMN1 prio 1 PLMN3 prio 2 PLMN4 prio 3 |
| 0.3 | S-NSSA1 22 | PLMN1 prio 1 PLMN2 prio 2 PLMN3 prio 3 |
| 0.2 | S-NSSA1 378 | PLMN2 prio 1 PLMN3 prio 2 PLMN1 prio 3 |

Figure 6.x.1-1- example of a UE configuration with 3 Network-Slice-Aware PLMN lists

Editor's Note:	It is FFS how the "Weight" factor is derived in the network and how the UE uses it in conjunction with the UE internally determined preferred S-NSSAIs.

### 6.X.2	Procedures

### For sake of argument let's assume a UE is configured with both a preferred PLMN list and one or more Network-Slice-Aware PLMN lists the UE can use.  This is the starting point in step 0 (if this is not yet achieved, then e.g. the normal SoR steps are followed to obtain such configuration)

In step 0, the UE has been already configured with the suitable ordinary preferred PLMN list (e.g. Operator Controlled PLMN Selector with Access Technology) and any Network-Slice-Aware PLMN lists by e.g. using the existing SoR interactions for UEs that indicate support of receiving the Network-Slice-Aware PLMN lists in SoR interactions.

In step 1 the UE detects it is in a country where there is a preferred PLMNs configuration.

In step 2/3 the UE detects that none of the S-NSSAI for which it has Network-Slice-Aware PLMN lists needs to be used, so, it behaves according to the ordinary preferred PLMN list behaviour.

In step 4 the UE detects the need to register with one or more S-NSSAIs that have a Network-Slice-Aware PLMN list associated with them. How this is detected is based on UE implementation (e.g. an application needs a specific slice based on the UE configuration including URSP information).

In step 5 the UE detects that the current PLMN is not preferred based on the S-NSSAI(s) it needs to use so it proceeds to select the suitable PLMN according to the Network-Slice-Aware PLMN list.

In step 6 the UE detects that none of the S-NSSAIs with a Network-Slice-Aware PLMN list is registered, so it returns to operate based on the ordinary preferred PLMN list.

In step 7 the UE selects a PLMN, if that is needed, based on the ordinary preferred PLMN list.

### 6.X.3	Impacts on services, entities and interfaces

The solution has the following impacts:

UE: ability to be configured with Network-Slice-Aware PLMN lists and ability to perform PLMN selection taking this also into account and indication of support of this feature to network. indication of this capability in SoR interactions.

SORAF/UDR: ability to provide Network-Slice-Aware PLMN lists for supporting UEs.

UDM: ability to send Network-Slice-Aware PLMN lists via a SOR transparent container to a supporting UE and to forward indication of support of feature to SORAF/UDR

AMF: noneEND OF CHANGES