SA WG2 Meeting #150E	S2-2202886r01

6 - 12 April 2022, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution proposal: KI#1 How to improve correctness of NWDAF analytics

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes solutions which address aspects of key issue #1.

# 1		Discussion

Among the key issues approved by SA2#149E for FS_eNA_Ph3, key issue #1 relates to enhancements needed in the 5GC to improve the correctness of NWDAF analytics.

One possible way to improve the correctness of NWDAF analytics is to rely on NWDAF analytics and ML models that exhibit a proven record of good performance, according to a given metric. Hence, in general, the ability for a service consumer to use a performance score to select an NWDAF Analytics and/or ML model helps such service consumer to obtain the desired level of correctness.

The performance score is assigned by rating the NWDAF analytics and ML models, but in order to obtain a reliable and robust recommendation system, it is necessary to ensure consistent rating criteria and to collect the rating from trusted sources, to avoid biased scores and unfair ratings.

The solution below proposes a framework to enable the rating of ML models and/or services that produce analytics, and the selection of the best, in terms of performance, ML model and/or service producer available.

Further clarification from the discussion in SA2#150e meeting: The spirit of the proposal is not to prevent a consumer from the same vendor of the producer to cast a rating. It is rather to prevent the same entity to make self-evaluation. For example, an NFc from ACME INC. can cast a rating to the analytics service from ACME Inc., but such Analytics service from ACME Inc. cannot make a self-evaluation and send it to TRLF. This way, it is possible to generate a report with all the ratings received from all vendors and draw conclusions.

# 2		Proposal

It is proposed to include the solution described below in FS_eNA_ph3 TR 23.700-81.

*** Start of Changes ***

*** Change 1 ***

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |
| x | x |  |  |  |

*** Change 2 – all new text ***

## 6.X	Solution #X: Enhancements to NWDAF analytics services

### 6.X.1	Description

In general, the ability for a service consumer to use a performance score to select an NWDAF Analytics and/or ML model helps such service consumer to obtain the desired level of correctness. This solution allows to rate ML models and/or NWDAF analytics provided by different producers (or by the same producer offering multiple ML models for the same NWDAF analytics) in a trusted and fair way.

In particular, an analytics consumer is allowed to rate an analytics service (i.e., Analytics ID) from an NWDAF (see procedure in clause 6.X.2.1), whereas an NWDAF containing AnLF is enabled to rate an ML model provided by an NWDAF containing MTLF (see procedure in clause 6.X.2.2). To avoid unfair and biased rating, in this solution a consumer from the same vendor of the analytics service or model provisioning service may rate the analytics service or the used ML model, while the entity producing/exposing the analytics service or ML model is not allowed to self-rate it. For example, an analytics consumer from the same vendor as the NWDAF containing AnLF producer can rate the analytics service, while self-ratings from the NWDAF containing AnLF are not accepted.

The solution leverages on the introduction of

- A Trusted Rating Logical Function (TRLF) that manages the rating provided by the service consumers. Furthermore, it ensures that only verified consumers (i.e., consumers that really have had access to the ML model and/or Analytics services) can rate the ML model and/or the Analytic service. For example, it prevents rating (of its own) by the producer of the ML model and/or analytics service while an analytics consumer from the same vendor is still able to rate the analytics service. For example, NWDAF(MTLF) that has produced the ML model is forbidden to rate such ML model, while NWDAF(AnLF) from the same vendor could rate the ML model if it has used the ML model.

- A rating format that includes key information regarding the usage of the Analytics service provided or not by using an ML model and that can be exploited by other consumers as well as by other producers/vendors.

### 6.X.2	Procedures

#### 6.X.2.1	Analytics consumer rating an NWDAF analytics provided by an NWDAF containing AnLF

Figure 6.X.2.1-1. Procedure for analytics consumer rating an NWDAF analytics provided by an NWDAF containing AnLF

Pre-condition: NF profile of the NWDAF in NRF contains a metric that should be utilized to rate the analytics service. A metric is associated to each analytics service (i.e. Analytics ID).

1.	An Analytics Consumer sends a discovery request to NRF looking for NWDAFs providing the Analytics ID(s) and other input parameters as specified in TS 23.288 clause 6.1.3.

2.	The NRF returns to the Analytics consumer the list of available NWDAFs matching the filter parameters along with the metrics to rate the analytics.

3.	The Analytics Consumer requests, for the discovered NWDAFs, the ratings of the Analytics ID from the Trusted Rating Logical Function (TRLF) through a Ntrlf_RatingDiscovery service. In the request, the Analytics Consumer specifies the NWDAF(s) and the Analytics ID(s). The Analytics Consumer sets the "rating aggregation level" request parameter according to its preferred value: "Global Average" to receive an aggregated model rating (i.e. a single value), "average per vendor" to receive a single average value for each vendor which issued a rating, or "detailed" in case it is interested to receive the rating from each service consumer which casted a rating.

The aggregated rating is a value, e.g., between 0 (very bad performance) to 5 (very good performance) derived by the TRLF by an average over all ratings. Along with the aggregated rating, the Analytics Consumer receives also the total number of ratings submitted, so that the consumer can derive the accuracy of the rating.

The Analytics consumer may also implement a local cache for such ratings, in order to avoid the need to query TRLF for each NWDAF discovery request. In this case, steps 2 and 3 may be skipped for future requests.

4.	The TRLF returns to the Analytics Consumer the requested ratings according to the specified "rating aggregation level". Based on the "average per vendor" or "detailed" model rating, the analytics consumer can identify potential unfair ratings.

5.	The Analytics consumer selects the NWDAF providing the best performance for the specific use case and scenario. The Analytics consumer requests the analytics service to the selected NWDAF specifying also its Consumer ID comprising the NF (instance or Set) ID and Vendor ID.

6.	The NWDAF generates a token that can be used by the Analytics consumer to rate the analytics service.

NOTE: The normative aspects about token generation, and how it is used for verification and communicated to other NFs should be carried out in coordination with SA3.

7.	The NWDAF sends through the Ntrlf_AnalyticsServiceConsumed service to the TRLF information about the Consumer ID, Analytics ID, information on the ML model used for producing the analytics (if any), its own NWDAF (instance or Set) ID and the token generated for the Analytics consumer. In this way, the TRLF can associate the rating from the Consumer to the analytics service provided by the NWDAF and, implicitly, to the ML model used to generate it in case the analytics service is based on an ML model.

8.	The TRLF sends an acknowledgement to the NWDAF.

9.	The NWDAF sends the analytics response to the Analytics consumer along with the token generated for allowing only verified consumers (i.e., only the ones that really have consumed the service) to evaluate the analytics service.

In case the analytics consumer subscribed to the analytics service, the token is valid for the entire subscription duration and the consumer may update its rating by sending another Ntrlf_AnalyticsRating request. Once the subscription is terminated, the NWDAF shall inform the TRLF about it, such that only a final rating can be provided by the consumer after which the token is revoked.

10.	The analytics consumer evaluates the performance of the analytics service utilizing the metric obtained by NRF during the discovery procedure.

11.	The analytics consumer through the Ntrlf_AnalyticsRating service sends its rating to the TRLF. The request also includes the Consumer ID of the analytics consumer and the received token.

12.	The TRLF, in case the token matches and the analytics consumer is not the model producer, accepts and updates the rating. The TRLF stores the rating per Analytics ID and for each Consumer ID.

13.	The TRLF sends to the analytics consumer a confirmation regarding the update of the rating.

#### 6.X.2.2	NWDAF containing AnLF rating an ML model provided by an NWDAF containing MTLF

This procedure, depicted in Figure 6.X.2.2-1, enables an NWDAF containing AnLF to rate an ML model received by NWDAF containing MTLF.

Figure 6.X.2.2-1. Procedure for NWDAF containing AnLF rating an NWDAF analytics provided by an NWDAF containing MTLF

1.	The NWDAF containing AnLF requests, for the NWDAFs that support the Analytics ID for the desired AOI) the rating(s) of the employed ML model(s) from the Trusted Rating Logical Function (TRLF) through a Ntrlf_RatingDiscovery service. In the request, the NWDAF containing AnLF specifies the NWDAF(s) and the Analytics ID(s). The NWDAF containing AnLF sets the "rating aggregation level" request parameter according to its preferred value: "Global Average" to receive an aggregated model rating (i.e. a single value), "average per vendor" to receive a single average value for each vendor which issued a rating, or "detailed" in case it is interested to receive the rating from each service consumer which casted a rating.

2.	The TRLF returns to the NWDAF containing AnLF the requested ratings per model ID and per analytics ID, according to the specified "rating aggregation level". Based on the "average per vendor" or "detailed" model rating, the analytics consumer can identify potential unfair ratings.

3.	The NWDAF containing AnLF selects the NWDAF containing MTLF providing the required performance of the ML model for the specific use case and scenario. The NWDAF containing AnLF subscribes to ML model provisioning service to the selected NWDAF specifying also its Consumer ID comprising the NF (instance or Set) ID and Vendor ID.

4.	The NWDAF containing MTLF generates a token that can be used by the NWDAF containing AnLF to rate the ML model.

5.	The NWDAF sends through the Ntrlf_AnalyticsServiceConsumed service to the TRLF information about the Consumer ID, Model ID and version used for producing the analytics, its NWDAF ID and version and the token generated for the NWDAF containing AnLF.

6.	The TRLF sends an acknowledgement to the NWDAF containing MTLF.

7.	The NWDAF containing MTLF sends the subscription notification to the NWDAF containing AnLF along with the token generated.

8.	The NWDAF containing AnLF evaluates the performance of the ML model utilizing the metric obtained by NRF during the discovery procedure.

9.	The NWDAF containing AnLF through the Ntrlf_AnalyticsRating service sends its rating to the TRLF. The request also includes the Consumer ID of the NWDAF containing AnLF and the received token.

10.	The TRLF, in case the token matches and the NWDAF containing AnLF is not the model producer, accepts and updates the rating. The TRLF stores the rating per Analytics ID and for each Consumer ID.

11.	The TRLF sends to the NWDAF containing AnLF a confirmation regarding the update of the rating.

This solution addresses aspects of key issue #1 on how to improve correctness of NWDAF analytics.

When an NWDAF acting as service consumer invokes the Nnwdaf_MLModelProvision_Subscribe service of an NWDAF containing an MTLF, the MTLF may perform one of the following:

a)	Select an existing trained ML Model to be provisioned.

b)	Trigger further training of an existing trained ML model before it is provisioned.

c)	Generate a new ML model to be trained and provisioned.

In cases b) and c), the NWDAF containing MTLF may initiate data collection from NFs, AF, DCCF, or OAM, required for the training of the ML model. Therefore, the decision of the MTLF among options a, b, and c can:

i.	lead to very different levels of computational load for the NWDAF containing MTLF.

ii.	increase the networking load on the NWDAF containing MTLF as well as on other Network Functions (NFs), while this increase might highly vary depending on how many data sources are used.

The decision of the MTLF among options a, b, and c is currently (i.e., according to Rel.17 specifications) taken by the MTLF based on local, unspecified logic.

*** End of Changes ***