3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202229

Elbonia, April 6th – 12th, 2022

Source:	Thales

Title:	Solution for predictive Power Saving Mode

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract: A solution for support of UE in discontinuous coverage is presented.

# 1. Introduction/Discussion

In Rel-18 FS_5GSAT_Ph2 study item, it is proposed to study the support of the UE in discontinuous coverage as shown below:

- WT#1: Discontinuous coverage

WT#1.1: Architectural enhancements to support discontinuous coverage for mobility enhancement (e.g. paging enhancement)

WT#1.2: Architectural enhancements considering prediction, awareness & notification of UE wake-up time, power saving optimizations.

The proposed solution is an enhancement of existing PSM procedures, for both EPS and 5GS, to take into account discontinuous coverage, together with satellites and UE motion.

It is also an enhancement compared to R17 solution (S2-2109199), because UE cannot always be considered as fix. For the example of the UE being in coverage for 20 minutes every 10 hours, if we consider the use case of a tracked container, the boat – in the example- can cover a significant distance in 10 hours (e.g: 15nd x 10h = 277.8km) and then the possible connection time with next satellite will change accordingly

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-28.

# * * * * First change (All new text) * * * *

## 6.X	Solution X: predictive Power Saving Mode

### 6.X.1	Description

This solution resolves Key Issue #2 about the power saving enhancements for UE in discontinuous coverage.

In the proposed solution:

- The CN (MME or AMF) is aware of the motion of the satellites of the constellation and is able to extrapolate the position of satellites of the constellation, as well as appearance or disappearance of satellites, in the time window corresponding to power saving mode periods.

- The UE is capable of doing PSM, and as per R17 assumption for NTN access, is equipped with GNSS receiver.

- Subsequently the UE, when requesting periodic update, indicates its current position and its extrapolated position at current time+ requested timer.

- In network response, if PSM is accepted, the network indicates a correction to be applied onto requested timer to fit with the satellite constellation motion. The correction is a delta on the value of GPRS Timer 3, with same coding as defined in TS 24.008 and subsequent. Note that specifying a delta value, with different possible unit values, will enable to be more accurate than providing absolute timer value.

- In addition, the network may indicate in the response the parameters that will shorten the UE Access Stratum reactivation on the next UE wake up.

### 6.X.2	Procedures

In the existing PSM procedure, as stated in ad-hoc specifications (TS 23.682, TS 24.301, TS 24.008):

UE requests the PSM by including a timer with the desired value in the ATTACH, TAU or ROUTING AREA UPDATE. By the way the UE reports how often and for how long it needs to be active in order to transmit and receive data. However, the final values are determined by the network.

In the proposed solution, ATTACH, TAU or ROUTING AREA UPDATE REQUEST are completed with the following IEs, if PSM is requested by the UE:

- UE current position (mandatory in case of satellite access)

- UE position extrapolated at the end of requested sleep time (optional)

ATTACH, TAU or ROUTING AREA UPDATE ACCEPT are completed with the following IEs, if PSM is accepted by the network:

- Delta on GPRS timer 3, with same coding as defined in TS 24.008.

### 6.X.3	Impacts on services, entities and interfaces

UE:

-	Computes its current and extrapolated location depending on its local PSM configuration.

RAN:

-	none

AMF/MME:

-	Configure the power saving parameters based on the coverage information for the UE.

# * * * * End of changes * * * *