3GPP WG SA WG2 Meeting #150E e-meeting	S2-2002641

Elbonia, April 6th – 12th, 2022	(revision of S2-2xxxxxx)

Source:	China Mobile

Title:	New Solution on 5GC information exposure to UE

Document for:	Discussion / Approval

Agenda Item:	9.23

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution proposes a Solution on Key Issue #2: 5GC information exposure to UE in FS_AIMLsys, Rel-18.

- Discussion

This contribution proposes a solution for Key Issue #2: 5GC information exposure to UE in the FS_AIMLsys.

From the UE side, it may benefit from network analytics information which are exposed by 5GS, for example,

-	According to the prediction of network status, the UE can split the AI/ML model into several parts to upload in different network status. In the worst network status, the UE decides to buffer the AI/ML model and not to upload. Also, the UE can choose to upload the model in good air interface environment at mid night at home or, via other RAT for example, the WiFi access.

-	The UE can request the DN performance analytics to decide which application server to connect. Also, the UE can decide the power saving strategy according to the UE behaviour predictions and observed service experience analytics. The NWDAF can also provide the intelligent network selection strategy to assist the UE to select a proper access type, 3GPP or non-3GPP network.

A number of Analytics ID have already been defined in TS 23.288[x]. however, which existing or new analytics ID(s) is benefit to the UE should be identified and how does the UE get the analytics from 5GC needs to be studied.

2.	Proposal

It is propose two solutions on enhancements on 5GC information exposure to UE.

* * * * First Change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  | X |  |  |  |  |  |
| 2 |  | Y |  |  |  |  |  |

* * * * Second Change * * * *

6.X	Solution #X: 5GC Analytics exposure to UE

6.X.1	Description

This solution is proposed to address Key Issue #2: 5GC information exposure to UE.

This solution aims at studying how to enhance the 5GC to expose information to the UE to facilitate its Application AI/ML operation (e.g. Model Training, Splitting and inference feedback etc.):

- How the 5GC exposes data analytics from NWDAF to the UE.

The procedure for 5GC to expose analytics(e.g. UE Mobility) to the UE is illustrated in 6.x.2.

6.X.2	Procedures

Figure 6.X.2-1: Procedure for analytics exposure to UE

- The UE negotiates with the AF though application layer on analytics subscription.

- The AF selects the NWDAF either by local configuration or querying the NRF according to Analytics ID, Area of Interest.

- The AF subscribes to the NWDAF(UE identifier, Analytics ID, Area of Interest) for analytics exposure to the UE.

4a. The NWDAF may return analytics data to the UE through corresponding AMF or UPF.

4b. The NWDAF may return analytics data to the UE via AF.

6.X.3	Impacts on services, entities and interfaces

UE:

-	Supporting negotiation with the AF on analytics exposure.

AMF:

-	Supporting analytics exposure from NWDAF to UE through control plane.

UPF:

-	Supporting analytics exposure from NWDAF to UE through user plane.

NWDAF：

-	Supporting analytics exposure to UE through AMF/UPF.

* * * * Third Change * * * *

6.Y	Solution #Y: 5GC information exposure to UE

6.Y.1	Description

This solution is proposed to address Key Issue #2: 5GC information exposure to UE.

This solution aims at studying how to enhance the 5GC to expose information to the UE to facilitate its Application AI/ML operation (e.g. Model Training, Splitting and inference feedback etc.):

- How the 5GC exposes 5GC information from 5GC NF to the UE.

The procedure for 5GC to expose 5GC information(e.g. delay, bandwidth) to the UE is illustrated in 6.y.2.

6.Y.2	Procedures

Figure 6.Y.2-1: Procedure for 5GC information exposure to UE

- The UE negotiates with the AF though application layer on 5GC information subscription.

- The AF may select the 5GC NF, either by local configuration or querying the NRF according to 5GC information identifier, Area of Interest.

3.	The AF subscribes to the related 5GC NF (UE identifier, Area of Interst, 5GC information identifier) for information exposure to UE.

4a.	The 5GC NF returns 5GC information to the UE through corresponding AMF or UPF.

4b.	The 5GC NF returns 5GC information to the UE through the AF.

6.Y.3	Impacts on services, entities and interfaces

UE:

-	Supporting negotiation with the AF on 5GC information exposure.

AMF:

-	Supporting 5GC information exposure from the 5GC NF to the UE through control plane.

UPF:

-	Supporting 5GC information exposure from the 5GC NF to the UE through user plane.

5GC NF:

-	Supporting of network information exposure to UE through AMF/UPF.

* * * * End of change * * * *