SA WG2 Meeting #150E	S2-2202864

6 - 12 April 2022, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	solution proposal for key issue 1 (inactive reception)

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_MBS2 / Rel-18

Abstract of the contribution: this paper proposes a solution for key issue 1 (inactive reception).

# 1		Discussion

Key issue 1 is defined as follows:

## 5.1	Key Issue #1: Multicast MBS data reception in RRC Inactive state

### 5.1.1	Description

In order to provide MBS service to more UEs in a cell, NG RAN could enable UEs within an MBS multicast session to receive MBS session data while in CM-CONNECTED with RRC Inactive state.

The following aspect will be studied for multicast:

-	Whether, how and what MBS assistance information to provide from 5GC to RAN for an MBS session allowing UEs in CM-CONNECTED with RRC Inactive state to receive MBS content, including the aspect which 5GC NF(s) determine the MBS assistance information and how they do so;

-	Whether and how to enhance the current procedures (including mobility related procedures) for MBS session with member UEs in RRC Inactive state.

NOTE 1:	During the study of this key issue, coordination with RAN WGs is needed before final conclusion.

NOTE 2:	RAN WG will determine how the switching for the UEs belonging to MBS session from CM-CONNECTED state to CM-CONNECTED with RRC Inactive state (and vice versa) is performed by the RAN node.

Solution 1 already provides a baseline for a key issue 1 solution. Although this P-CR is suggesting a new solution, Nokia is also happy to convert this P-CR into an update of solution 1 if this is agreeable to the original source companies.

# 2		Proposal

It is proposed to include the following changes in TR 23.700-47.

*** 1st Change ***

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 MBS session reception in RRC Inactive | 2 MOCN network sharing |  |  |
| 1 | X |  |  |  |
| 2 |  | X |  |  |

*** 2nd Change (all new)***

## 6.X	Solution #X: Procedures for Transmission mode tor inactive data reception

### 6.X.1	Introduction

This solution addresses Key Issue #1.

### 6.X.2	Functional description

It is assumed to reuse the current architecture defined in Rel-17 MBS work (see TS 23.247 [4]). In other words, MB-SMF is used to handle MBS session-level management while SMF performs per-UE MBS session management, e.g. authorization, multicast session information provisioning, managing 5GC Individual MBS traffic delivery.

Whether the transmission mode for inactive reception is applied depends on multiple factors:

-	Backward compatibility with Rel-17 5MBS UEs not capable of receive 5MBS data while in RRC-inactive state. If such UEs that joined an 5MBS session are in a cell, MBS data need to be transmitted using the Rel-17 transmission mode for RRC-connected reception.

-	QoS requirements of service: UEs will likely provide less or no radio feedback when receiving 5MBS data while in RRC-inactive state compared to receiving MBS data in RRC-connected state, and thus they may encounter more packet loss.

-	Scalability: a higher number of UEs within an MBS session in a cell will be enabled if they can receive 5MBS data while in RRC-inactive state

-	UE preferences: UEs could prefer to receive MBS data in RRC inactive state to reduce their battery consumption, or in RRC connected state to increase the service quality.

-	Backward compatibility with Rel-17 RAN nodes: Those nodes will only apply the el-17 transmission mode for RRC-connected reception

-	Cell load: In a highly loaded cell it is preferable to transition UEs to RRC-inactive state.

-	Other activities of UEs: UEs in an MBS multicast session that also exchange data in PDU sessions and thus need to remain in RRC connected state.

Further, it is desirable to transmit 5MBS multicast data only a single time in a cell (rather than one time in transmission mode for RRC-inactive reception and one time in transmission mode for RRC connected reception. However, RRC connected UEs may be able to receive MBS data transmitted using the transmission mode for RRC-inactive reception.  It is thus assumed that the 5MBS transmission mode for inactive reception will not be applied homogenously in all cells for a multicast session and that RAN nodes will select the transmission mode

Editor's note:	Those assumptions need to confirmed by RAN WGs.

An AF creating a multicast session should be able to influence the service quality (more UEs vs. higher reliability) talking into consideration the specific needs of the service it offers. Thus the AF can enable or disable the reception mode for inactive transmission. However, the decision whether to apply that transmission mode if it is enabled remains with the RAN nodes.

UEs need to be informed about and configured for the transmission mode applied in the cell where they are residing by the RAN nodes.

Editor's note:	Those assumptions need to confirmed and related procedures need to be defined by RAN WGs.

UEs that transition to RRC inactive mode can move freely in their RAN notification area without notifying the RAN nodes. Their location is thus not known at cell level. At the moment only the RAN node serving a UE is aware that it is participating in a MBS multicast session. Additional RAN nodes not handling UEs within the multicast session themselves but within RAN notification areas of UEs in the multicast session may need to be made aware of the multicast session and perform procedures for the multicast session such as transmitting data of the multicast session.

Editor's note:	Related procedures need to be defined by RAN WGs. It also neeeds to be studied by RAN WGs whether it is possible to determine the presence of inavtive UEs that desire to receive a multicast session in a cell.

RRC_inactive UEs can move to cells where the transmission mode for RRC connected state is applied and then need to transition to the RRC connected state to receive MBS data.

The current MBS session activation procedures use paging for the MBS session and trigger RRC-inactive UEs to transition to RRC-connected state. For the transmission mode for RRC_inactive reception, RRC-inactive UEs should be made aware that the MBS multicast session is activated but remain in RRC-inactive state.

Editor's note:	Related procedures need to be defined by RAN WGs.

It was discussed in Rel-17 that it would be desirable for location dependent services to enable that the MBS session is active in some areas and inactive in other areas. However, UEs moving in RRC-Inactive state between such areas will not know whether the MBS session is active in their location.

Broadcasting of information about the MBS session in cells comprising information about whether the MBS session is active and about the transmission mode of the MBS session is suggested to solve those issues.

Editor's note:	Related procedures need to be agreed by RAN WGs.

### 6.X.3	Procedures

#### 6.X.3.1	MBS session creation, multicast session join and session establishment procedure

Figure 6.X.3.2-1: Enhancement to current MBS procedures for session creation and join

0.	When UE registers, it indicates its capability to receive MBS multicast using the transmission mode for RRC inactive. It may also indicate a preference. This information is propagated to NG-RAN via AMF.

1.-2. When an AF requests the creation of a multicast MBS session, it indicates whether inactive/idle reception of multicast shall be enabled for that session.

3. The AF may also indicate in the service announcement towards the UE whether inactive/idle reception of multicast is enabled

4.-14 The information whether the inactive transmission mode is enabled for an MBS session is propagated from MB-SMF towards NG-RAN, via PDU session and/or via shared delivery of a multicast session.

15. The NG-RAN decides the transmission mode to apply for the MBS multicast session in a cell.

16. The NR-RAN broadcasts the transmission mode for the MBS session in the cell and also indicates whether the MBS session is active

#### 6.X.3.2	Moving a UE to RRC Inactive state

Figure 6.x.3.2-1: NG-RAN node moves a UE in 5MBS session to RRC Inactive state

- UE joined an MBS session and receives MBS data while in RRC connected state

- IF NG-RAN 1 uses the transmission mode for RRC-inactive reception in a cell where the UE is located, it can move the UE to RRC inactive state. NG-RAN 1 then releases the RRC connection to the UE. The UE can move to other cells in its RAN notification area with notifying NG-RAN 1.

2.	NG-RAN 1 determines that part of the the RAN notification area oft he UE is served by RAN 2. It informs RAN 2 that handling of the MBS session for inactive UEs is required in that area,

Editor's note:	Related procedures need be determined by RAN WGs.

3.	The NG-RAN 2 decides the transmission mode to apply for the MBS multicast session in a cell within the area.

4.	The NR-RAN 2 broadcasts the transmission mode for the MBS session in the cell and also indicates whether the MBS session is active

5-6.	The NG-RAN 2 may establish shared delivery and transmit MBS data. It may obtain information about the MBS session via shared delivery.

Editor's note:	The procedures in this clause need be determined by RAN WGs. However, transfer of assistance information and shared delivery impacts SA2

#### 6.X.3.3	MBS service activation

Figure 6.x.3.2-1: MBS service activation

- RAN 1 receives paging request for MBS session.

- RAN 1 determines inactive UEs it serves and based on that paging area

- RAN 1 decides transmission mode for MBS session in cells

- For cells with transmission mode for inactive reception, RAN uses procedure that keeps UE in RRC-inactive state, for instance broadcast information that the MBS session is active

5-6.	The NG-RAN 2 may establish shared delivery and transmit MBS data. It may obtain information about the MBS session via shared delivery.

- For cells with transmission mode for connected reception, RAN uses paging procedure to make UEs transit to RRC-connected state

8.	NG-RAN 1 determines that part of the RAN notification area is served by RAN 2. It informs RAN 2 that paging of the MBS session for inactive UEs is required in that area,

9 to 13.	Same as 3 to 7, but execute by RAN 2

Editor's note:	The procedures in this clause need be determined by RAN WGs. However, transfer of assistance information and shared delivery impacts SA2

### 6.1.4	Impacts on services, entities and interfaces.

Editor's note:	This clause describes impacts to existing services, entities and interfaces.

Functional entities defined in clause 5.3.2 of TS 23.247 [4] is reused.

Editor's note:	Additional impacts are FFS.