SA WG2 Meeting #S2-150E	S2-2202921

06 - 12 April, 2022, E-Meeting

Source: 	NEC

Title: 	Solution: KI#3 Key Issue #3: Temporary network slices

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNS_ph3 / Rel-18

Abstract of the contribution: This paper proposes a solution to the key issue Key Issue #3: Network Slice Area of Service for services not mapping to existing TAs boundaries, and Temporary network slices.

# Discussion

This contribution addresses key issue#3 Key Issue #3: Temporary network slices

This Key Issue will study how to address the issues described above and whether system level improvements are needed to mitigate e.g. the deployment and control plane issues that arise due to the currently defined system behaviour:

-	The support of network slices which have a limited lifetime (including how to gracefully terminate a network slice which can apply also to network slices which have a longer lifespan in order to avoid abrupt PDU Session release).

# Proposal

Add the following solution to TR 23.700-08.

* * * First Change * * * *

## 6.x	Solution #x associating a validity timer with a temporary slice.

### 6.x.1	Introduction

The solution addresses second part of the key issue #3 " Network Slice Area of Service for services not mapping to existing TAs boundaries, and Temporary network slices " which states that there can be case where a network slice is deployed temporarily for a particular time period

The network (e.g. AMF) associates a validity timer for each S-NSSAI which is deployed temporarily. This validity timer for each temporary S-NSSAI is sent to the UE e.g. in the configured NSSAI list during the registration procedure. When the UE receives the S-NSSAI with a validity timer in the configured NSSAI, the UE runs a validity timer. The UE will register to the S-NSSAI when the validity timer is running. Once the validity timer expires the UE locally removes the S-NSSAI from the allowed NSSAI list and the UE locally release the PDU session associated with the S-NSSAI. The network (e.g. AMF) also runs the validity timer for the S-NSSAI, once the validity timer expires the network removes the S-NSSAI from the allowed NSSAI list and locally release the PDU sessions associated with the S-NSSAI.

### 6.x.2	Procedures

Figure 1: Handling of a temporary network slice.

The detailed procedure are as described below:

1. The UE initiates registration procedure and sends registration request messgae.

2-3) The network may trigger authentication procedure and security mode command procedure.

4-6) The AMF registers to the UDM for the UE. the UDM provides the subscribed NSSAI to the AMF. In the subscribed NSSAI the UDM associate a validity time for each temporary S-NSSAI.

NOTE: the validity timer for a temporary S-NSSAI can also be provided by other NSSF.

7) The AMF sends configured NSSAI = Subscribed NSSAI to the UE in the registration accept message.

8) The UE stores the configured NSSAI.

9) The sends registration complete message to the AMF.

10a) The UE starts t1. The UE initiates registration to the S-NSSAI 1 while the timer t1 is running. Once the t1 expires the UE removes the S-NSSAI 1 from the allowed list locally. The UE removes the S-NSSAI 1 from the configured NSSAI list.

10b) 10b. AMF starts t1 is. The AMF accepts registration to the S-NSSAI 1 while the timer t1 is running. Once the t1 expires the UE removes the S-NSSAI 1 from the allowed list locally.

### 6.X.3	Impacts on services, entities and interfaces

### UE: Handling of validity timer for a S-NSSSAI.

### AMF: Handling of validity timer for a S-NSSSAI.