3GPP SA WG2 Meeting #151e	S2-2204669

# e-meeting, May 16-20, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	KI#9, New Sol: location accuracy analytics

Document for: 	Approval

Agenda Item: 	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution: This paper addresses KI#9, proposing the enhancement of NWDAF with location accuracy analytics

# Discussion

Key Issue #9: Enhancement of NWDAF with finer granularity of location information

In this key issue, the following aspects will be studied:

-	Identify use cases and corresponding existing or new Analytics ID(s) where the analytics require location information with finer granularity than TA/cell level, and how to enhance related existing Analytics ID(s).

-	Identify how an NWDAF determines that location information with finer granularity than cell/TA level is required in output analytics.

-	Identify what input data needs to be collected to deliver analytics with fine granularity location information.

-	Identify how NWDAF acquires the input data to deliver finer granularity location information. Whether and how the functionality and services of NWDAF or other NFs need to be enhanced.

The proposal described below provides a solution to predict the location accuracy for a given UE location estimated by LMF. The LMF uses an NWDAF service to provide its UE location estimate and other related inputs. The NWDAF has a trained ML model that uses these inputs and outputs the predicted UE location accuracy. This accuracy value is used by the LMF to take further action(s) in order to meet the QoS criteria for UE location.

This solution was proposed as a solution to KI#4 in FS_eLCS_Ph3 TR and was agreed in SA2#150 (S2-2203301) with a note to conclude if a new Analytics ID will be required for the service with corresponding eNA impacts. This pCR is the result of that analysis.

# Proposal

It is proposed to update TR 23.700-81 as follows

*** Start of changes ***

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 1 | X |  |  |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |  |  |
| 6 | X |  |  |  |  |  |  |  |  |  |
| 7 | X |  |  |  |  |  |  |  |  |  |
| 8 |  | X |  |  |  |  |  |  |  |  |
| 9 |  | X |  |  |  |  |  |  |  |  |
| 10 |  |  | X |  |  |  |  |  |  |  |
| 11 |  |  | X |  |  |  |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |  |  |
| 13 |  |  |  |  | X |  |  |  |  |  |
| 14 |  |  |  |  | X |  |  |  |  |  |
| 15 |  |  |  |  | X |  |  |  |  |  |
| 16 |  |  |  |  |  | X |  |  |  |  |
| 17 |  |  | X |  |  | X |  |  |  |  |
| 18 |  |  |  |  |  |  | X |  |  |  |
| 19 |  |  |  |  |  |  | X |  |  |  |
| 20 |  |  |  |  |  |  | X |  |  |  |
| 21 |  |  |  |  |  |  |  | X |  |  |
| 22 |  |  |  |  |  |  |  | X |  |  |
| 23 |  |  |  |  |  |  |  | X |  |  |
| 24 |  |  |  |  |  |  |  | X |  |  |
| 25 |  |  |  |  |  |  |  |  | X |  |
| 26 |  |  |  |  |  |  |  |  | X |  |
| 27 |  |  |  |  |  |  |  |  | X |  |
| X |  |  |  |  |  |  |  |  | X |  |

*** Next Change – all new text ***

## 6.X	Solution #X: Enhancement of NWDAF with location accuracy prediction

### 6.X.1	Functional Description

#### 6.X.1.1	General

This solution addresses the KI “Key Issue #9: Enhancement of NWDAF with finer granularity of location information”.

TS 23.273 defines LCS QoS to have the following attributes: (1) Location Accuracy, (2) Response time and (3) QoS Class. Among these, the QoS Class attribute gives the requirement on the other attributes like accuracy or response time. It could take as values Best Effort, Multiple QoS Class or Assured. While the Best Effort class is the least stringent one, the other two require the LMF to determine how accurate its location estimate is.

For example:

-	in case of “Multiple QoS Class”, if the accuracy is less than what is required, the LMF selects an appropriate localization method (e.g. with more TRP measurements) and re-runs the location estimate in order to reach the required accuracy.

-	in case of “Assured” QoS Class, the procedure fails if the accuracy required is not met.

To provide a real-time estimate of location accuracy without need for benchmark/ground truth location, we propose a data analytics-based solution running in the NWDAF with a supervised Machine Learning model (e.g. Neural Network).

A training should be first performed on the selected ML model with labelled data, i.e. input data with corresponding output data:

-	Input data: positioning estimate and optionally: used positioning method, assistance data, UE ID, Area, see Table 6.X.1-1.

-	Output data: corresponding accuracy (calculated with benchmark method such as the output of MDT), see Table 6.x.1-2

Once the model is trained, there is no more need for ground truth (e.g. GPS information). This model is then subsequently used by the LMF to determine if its location estimate meets the accuracy required and take appropriate action. To enable this, the LMF makes use of a Service Based API that is exposed by the NWDAF.

The input data for the Location Accuracy analytics is shown in Table 6.X.1-1

Table 6.x.1-1: Data collection for "Location accuracy" analytics

| Information | Source | Description |
| --- | --- | --- |
| UE location | LMF | UE location as estimated by the LMF with the selected localization method. |
| Location method/Assistance Data | LMF | Selected location method and corresponding parameters. |
| Area | LMF | The region of interest where the UE is located (e.g. could be list of cell IDs). |

The Location Accuracy service output is defined in Table 6.x.1-2.

Table 6.x.1-2: Output analytics for "Location accuracy"

| Information | Description |
| --- | --- |
| Location accuracy | Predicted location accuracy (could be in % or in unit meters) |
| Applicable Area | A list of TAIs or Cell IDs as well as below cell level location information that the analytics applies to. |
| Confidence | Confidence of the prediction. |

### 6.X.2	Procedure

Fig.1 : Location accuracy prediction and usage

Pre-condition: NWDAF has a trained supervised ML model for predicting location accuracy.

- LCS Client requests to LMF the UE location with QoS Class set as either “Multiple QoS Class” or “Assured” alongwith the location accuracy required. The expected behaviour from the LMF is as given in clause 6.X.2.

2.	The LMF initiates the LCS session as given in TS 23.273 and derives the UE location estimate.

3.	The LMF queries the NWDAF for the location accuracy giving as inputs the corresponding Analytics ID . Other inputs could be - location estimate , UE positioning method/assistance data and area (cell ID/TAI).

NOTE. Both Nnwdaf_AnalyticsSubscription and Nnwdaf_AnalyticsInfo services can be used to implement step 3. above. The Nnwdaf_AnalyticsSubscription service can be used by a service consumer to receive notifications about location accuracy, e.g., when a change is detected.

4. The NWDAF uses the corresponding model trained in the precondition and provides the location accuracy as the output to the LMF.

5.	LMF compares the location accuracy received in step 3 with the required location accuracy received in step 1 from the LCS Client.

6.	If the required location accuracy is not met, the LMF takes subsequent actions like re-executing the LCS procedure with more stringent parameters (for example: higher periodicity measurements).

7.	The LMF returns the location estimate with the required accuracy.

NOTE:	The offline training gives the ML model the ability to predict the location accuracy without the need for a reference location estimate. This is made possible by the extensive training done using various accuracy/reference values, which result in the computing of weights in the ML model. Such a trained model can then be used for an inference/prediction of the location accuracy in this step 3.

### 6.X.3	Impacts on services, entities and interfaces

NWDAF

- Supports a new Analytics ID for determining Location Accuracy

- Supports subscription from NFs like LMF to make use of this Analytics

- Supports data collection from LMF to generate the output of this Analytics

# *** End of changes ***