SA WG2 Meeting #S2-151E 	S2-2203803

16 – 20 May, 2022 Electronic meeting

Source:	China Mobile

Title:	KI#1, Solution #10: Update of support 5G VN service area with LADN mechanism

Document for:	Approval

Agenda Item: 	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract of the contribution: This document updates solution 10 to resolve the editor notes.

- Discussion

This document updates solution 10 to resolve the following editor note.

Editor's note:	Whether there are other results if there is no overlapping between LADN service area and 5G VN group service area if the DNN are the same, is FFS.

When there is no overlapping between LADN service area and 5G VN group service area if the DNN are the same, AMF may also trigger the PDU Session release procedure of 5G VN group based on local policy.

- Proposal

It is proposed to update solution 10 in TR 23.700-74.

# * * * * Start of change * * * *

6.10	Solution #10: Support 5G VN service area with LADN mechanism

6.10.1	Key Issue mapping

This solution is for Key Issue #1, which addresses following aspects:

-	How to provision (i.e. set, modify and delete) the service area or QoS applicable to each UE of the group via exposure interface, i.e.:

-	What is the information that constitutes the service area applicable to each UE of the group via the exposure interface, and how to enforce the service area applicable to each UE of the group by reusing the existing mechanisms.

6.10.2	Description

5G LAN type service may only support the UEs, which are belong to 5G VN group, when the they are located in specific service area.

In this solution, it is proposed to use LADN mechanism to support the 5G VN group restriction service area. AF/NEF may provide the External Group ID, 5G VN group membership information and 5G VN group data to the UDM. In the 5G VN group data, the 5G VN group service can be added to show the service area restriction for the 5G LAN type services.

In this solution the LADN DNN is the same as 5G VN group DNN, so the PDU Session for the LADN DNN and of the 5G VN group are the same.

For LADN mechanism, the AMF is configured with the LADN service area by OAM as defined in TS 23.501 [2].

When a UE from 5G VN group registers to the 5G network, the AMF retrieve the subscription information from UDM with the 5G VN group data. After the AMF get the 5G VN group service area from UDM, the AMF should compare the LADN service area and the 5G VN group service area information, to check whether these two areas are the same or not. If there is some overlapping between these two areas, i.e. the same TAs existing in both two areas, the overlapping part area can be decided as the LADN service area and sent to UE. If there is no overlapping between these two areas, the AMF may send the LADN information to UE with indication that the 5G LAN type service cannot be used even the PDU session of the 5G VN group DNN has been establishment or trigger the PDU Session release procedure of 5G VN group based on local policy.

After the UE registration procedure, the UE can get the 5G LADN DNN and related LADN service area.

When a UE request a PDU session establishment for the LADN DNN, the AMF and SMF can treat the PDU session as a general PDU session for the LADN DNN.

6.10.3	Procedures for support of LADN service area

Figure 6.10.3-1: Procedures for support of LADN service area

NOTE:	In the following procedures, 5G VN Group DNN is the same of 5G LADN DNN.

1.	The AF provides 5G VN group data information through NEF to UDM (GPSI, DNN, S-NSSAI and 5G VN Group service area).

2.	AMF is configured with LADN information (LADN service area, LADN DNN).

3.	During the registration procedure, AMF can retrieve the group subscription data from UDM, which contains the 5G VN group data (5G VN group service area, DNN, S-NSSAI). Then AMF should compare the LADN service area and the 5G VN group service area information, to check whether these two areas are the same or not, and generates the overlapped service area. The overlapped service area can be considered as the LADN service area for this UE. The overlapped service area will be sent to UE in registration accept message.

4.	UE has the information of LADN DNN and LADN service area (the one which the AMF send to the UE in step 3). UE can request PDU session establishment for the LADN DNN, and AMF and SMF handle the PDU session establishment request as t PDU session for the LADN DNN.

6.10.4	Impacts on services, entities and interfaces

AMF:

-	Capability enhancement to compare the LADN service area and 5G VN group service area, and then generate the overlapped service area as the new LADN service area.

# * * * *end of change * * * *