3GPP SA WG2 Meeting #151E				     S2-2203925

Elbonia, May 16-20, 2022	(was S2-22xxxx)

Source:	Ericsson

Title:	KI#3: new solution: enabling graceful slice termination with support of UE policies

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This contribution provides a new solution for KI#3.

# 1.	Discussion

Currently architecture and procedures already support the ability to add and remove network slices. What may be missing is to enable a graceful replacement of network slices.

# 2 Proposal

It is proposed to include a conclusion that this solution is not be considered.

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| Solution #1: Additional S-NSSAI associated with the PDU session | X |  |  |  |  |  |
| Solution #2: Slice Re-mapping Capabilities for Network Slice Service Continuity | X |  |  |  |  |  |
| Solution #3: Support of Network Slice Service continuity using SSC mode 3 | X |  |  |  |  |  |
| Solution #4: PDU Session on compatible network slice | X |  |  |  |  |  |
| Solution #5: PDU session handover to a target CN with an alternative S-NSSAI support | X |  |  |  |  |  |
| Solution #6: Extended SoR VPLMN Slice Information transfer to UEs |  | X |  |  |  |  |
| Solution #7: Enabling awareness of Network Slice availability in VPLMNs |  | X |  |  |  |  |
| Solution #8: Gracefully network slice termination |  |  | X |  |  |  |
| Solution #9: Support of a Network Slice with an AoS not matching existing TA boundaries |  |  | X |  |  |  |
| Solution #10: Associating a validity timer with a temporary slice |  |  | X |  |  |  |
| Solution #11: Enabling UEs to Request S-NSSAIs not uniformly available |  |  | X |  | X |  |
| Solution #12: Solution for Centralized Counting for Multiple Service Areas and 5GS-EPS Interworking |  |  |  | X |  |  |
| Solution #13: Hierarchical NSACF Architecture for Maximum UE/PDU Session number control |  |  |  | X |  |  |
| Solution #14: Maximum Number Distribution in multiple NSACFs |  |  |  | X |  |  |
| Solution #x: Enabling graceful slice termination with support of UE policies |  |  | X |  |  |  |

NEXT CHANGE

## 6.x	Solution #x: Enabling graceful slice termination with support of UE policies

### 6.11.1	Introduction

This solution applies to KI#3.

This solution enables a graceful replacement and removal of network slices.

### 6.11.2	Functional Description

The basic principles of this solution are the following:

1.	The "time" or area where a network slice is to be terminated is known in advance

2.	If a network slice is to be replaced by a new network slice then:

-	If same S-NSSAI can be used, then replacement of NFs in the network slice is done by first adding new NFs and then reduce load on those NFs to be removed until there is no or very limited load on the NFs to be removed.

-	If different S-NSSAI can be used then first create a new network slice i.e. NFs and resources etc and then add the S-NSSAI to subscriptions, and then use PCF to update the UE policies (URSP)

-	Update UEs URSP rules such that the UEs use the new S-NSSAI for new access attempts;

-	Update UEs URSP rules Route Selection Validation Criteria with appropriate Time Window and Location Criteria;

-	The UE is assumed to try and use the Time Window and use higher priority rules when the Time Window is no longer valid.

-	Update the UE URSP rules such that PCF can enforce/command the UE to re-apply the URSP rules for also existing usage which, if possible, triggers the UE to move the user data from S-NSSAI to be removed to the new S-NSSAI e.g. using SSC 3 etc.

3.	OAM knows when and which network slice is to be terminated and configures NFs

1.-	UDR/UDM are updated such that Network Slices are added and UDM notifies AMFs

2.	NFs of network slice to be replaced updates NF profile such that load is reduced e.g. no new entrants

3.-	PCF updated as per bullet 2

4.-	UDR/UDM are updated such that Network Slices are added and removed, and UDM notifies AMFs.

4.	AMFs follows current logic i.e.

-	AMF updates NSSAI information towards the UE

-	AMF enforces removal of S-NSSAIs e.g. initiates release of PDU Sessions within the network.

### 6.11.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

The existing procedures are used as is with minimal impacts to the content of the URSP rules.

### 6.11.4	Impacts on services, entities and interfaces

The following impacts have been identified:

-	UE:

-	UE supports the new conditions in the URSP rules such that PCF can enforce/command the UE to re-apply the URSP rules for also existing usage.

NOTE:	The functionality works also without UE support, but there can then be a service interruption when changing network slice.

-	PCF:

-	supports the new conditions in the URSP rules such that PCF can enforce/command the UE to re-apply the URSP rules for also existing usage.

END OF CHANGES