SA WG2 Meeting #151e	S2-2204050

# May 16th – 20th, 2022, Elbonia               		  								     (revision of S2-2203591)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0102 | rev | 2 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve security open items of UE-to-Network Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Nokia], [Nokia Shanghai Bell], [Interdigital Inc.], CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR addresses the SA2 impacts of security open items for UE-to-Network Relay, inlcuding both Control Plane based authentication/authorization procedure for UE-to-Network Relay as specified in clause 6.3.3.3.2 of TS 33.303, and PDU Session secondary authentication of Remote UE via Layer-3 UE-to-Network Relay as specified in clause 6.3.3.3.4.2 of TS 33.303. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The SA2 impacts of CP based security procedure are addressed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete CP based security procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.1.4.3.2	Control Plane based security procedures for 5G ProSe UE-to-Network Relay

Control Plane-based security procedures for 5G ProSe UE-to-Network Relay call flow and procedure is defined in TS 33.503 [29]. It is characterised by the following principles:

-	5G ProSe UE-to-Network Relay's NAS signalling is used for the 5G ProSe Remote UE's authentication and authorisation.

-	The AMF serving the 5G ProSe UE-to-Network Relay selects AUSF as specified in TS 23.501 [4] clause 6.3.4considering also the 5G ProSe Remote UE's SUCI.If the AUSF cannot be selected in roaming case, the procedure fails.

NOTE 1:	It is assumed that 5G ProSe UE-to-Network Relay selected AMF always supports control plane based security procedure as defined in clause 6.3.3.3.2 of TS 33.303.

NOTE 2:	It is assumed that AMF selected AUSF always supports control plane based security procedure as defined in clause 6.3.3.3.2 of TS 33.303.

NOTE 3:	If control plane based security procedure failed because of no roaming agreement between 5G ProSe UE-to-Network Relay's serving PLMN and 5G ProSe Remote UE's HPLMN, the 5G ProSe Remote UE can use trial and error approach to select another 5G ProSe UE-to-Network Relay.

PDU Session secondary authentication of 5G ProSe Layer-3 Remote UE via 5G ProSe Layer-3 UE-to-Network Relay procedure is defined in TS 33.303 [29]. It is characterised by the following principles:

-	This procedure is optional and based on successful completion of control plane based security procedure.

-	5G ProSe Layer-3 UE-to-Network Relay adds indication of 5G ProSe Layer-3 Remote UE Report message in the MM NAS message and based on that AMF adds 5G ProSe Layer-3 Remote UE’s SUPI in the message to SMF.

-	5G ProSe Layer-3 UE-to-Network Relay’s SMF uses 5G ProSe Layer-3 Remote UE’s SUPI to select Remote UE’s UDM and obtains Remote UE’s subscription information.

-	There is no method to guarantee roaming agreements exist between 5G ProSe Layer-3 UE-to-Network Relay’s VPLMN and 5G ProSe Layer-3 Remote UE’s HPLMN, and trial and error approach can be used for selecting another 5G ProSe Layer-3 UE-to-Network Relay in case of PDU session secondary authentication failed.

End of CHANGE