3GPP TSG-SA2 Meeting #152E	S2-2205544

# E-Meeting, 17th – 26th  August 2022

Source: 	Ericsson

Title:	Update on Solution #9: NWDAF-assisted application detection

Document for: 	Approval

Agenda Item: 	9.23

Work Item / Release:	FS_eNA_Ph3/ Rel-18

Abstract of the contribution: This contribution proposes update to solution #9.

# 1 Discussion

# 2 Proposal

It is proposed to update TR 23.700-81 as below.

********** Start Changes*************

## 6.9	Solution #9: NWDAF-assisted application detection

### 6.9.1	Description

This solution is proposed to address Key Issue #2: NWDAF-assisted application detection.

To differentiate an application traffic handling, the first step is to distinguish application traffic from other traffics. In 5G network, an application can be distinguished by a set of packet headers (SDFs) or application IDs. The most common approach to detect traffic is use a set of packet headers which usually contains source and destination address. The application ID can be used for referring to the UPF's specific application detection filter, and AF may provide PFDs according to clause 4.18 of TS 23.502 [3] to update application detection filter information associated with the application ID. A PFD contents could contain flow description (service-side 3-tuple), URL, and domain name/protocol information. When an AF delivers PFD to NEF (PFDF), it will be distributed to SMFs and UPFs to enable flow detection according to clause 5.8.2 of TS 23.501 [2]. The SMF use the procedure described in clause 4.18.3.1 of TS 23.502[3] to provision or remove the PFDs associated to a specific application ID.

However, as indicated by the KI description, the ASP may provide an initial PFD information but does not update it in time or does not update it anymore, then it is unknown how the UPF detect the application traffic. As it may happen when the ASP invokes the Nnef_PFDManagement_Create service to provide PFDs and then invokes the Nnef_PFDManagement_Delete service with a certain delay according to clause 4.18 of TS 23.502 [3].

The objective of this solution is to provide analytics on application traffic. The proposed solution provides analytics including new PFDs for unknown applications and updated PFDs for known applications by extracting network traffic signature and inferring application identity. To realize this solution, one of the main features is 1) collect measurement of an application to extract statistical characteristics, 2) collect payload of packets to extract payload characteristics (such as domain name contained in the payload). The baseline usage of the generated analytics is to store the captured application characteristics as a PFD, and the PFD is used by SMF and UPF to detect an application defined as TS 23.502 [3].

Editor's note:	How new PFDs for unknown applications can be used is FFS.

The NWDAF could collect the current and historical PFD information (including Application ID, historical IP 3-tuple, historical URL, historical Domain name information) from the NRF via NEF (PFDF) and ADRF respectively.

Based on PFD information from ADRF and NRF and traffic information from UPF, new PFD information (including Application ID, new IP 3-tuple, new URL, new Domain name information) for the existing Application ID could be derived by the NWDAF. The new PFD information provided by the NWDAF can be used by the NEF for provisioning SMF/UPF.

The SMF can requests the provision of the PFDs associated to an application ID, then it translates the PCC rules associated with the applications identifier to PDRs to install it on the UPF. So, for a full automatize process, the SMF needs to know the application identifier that contains the PFDs to assigns it to a PCC rule with the desired traffic policies. Also, the consumer of the analytic (e.g. NEF) may be interested only in a subset of the applications found by the NWDAF, so a new Analytics Filter is proposed in this solution to cover both needs:

- Filter the applications that the NWDAF shall provision to the consumer.

- Provide input information to the NWDAF to name the found applications.

The consumer informs the NWDAF about the target applications through the “Applications Characteristics”. The “Application Characteristics” are specific well know KPIs between the traffic characteristics i.e., the relations that will identify each of the application.

The application characteristics are created from the traffic features provided by the UPF as input of the data collection: Data duration, QoS flow bit rate, packet transmission, size of packets, data volume, UL/DL Packet Delay, in combination with an application relation function: min(), application with the minimum value, max(), application with the maximum value.

Several “Application Characteristic” can be used per filter using and/or operators as on Table 5.2.19.2.1-1 of TS 23.502 [3], e.g.:

Table 5.2.19.2.1-1: Example of Event Filters for NWDAF exposure events

| Event ID for NWDAF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Assisted application detection | <Parameter Type = Application Characteristic, Value = max(data duration) and min(qos flow bit rate), Application Name = Application1> <Parameter Type = Application Characteristic, Value = min(packet transmission) and max(size of packets), Application Name = Application2> |

The name given to the “Application Characteristics” is used by the NWDAF to name the applications that match the characteristic.

In case the NWDAF receives “Application Characteristics” filters in the subscription/request of the analytic, the NWDAF only retrieve the applications matching the filters.

In case that there is overlapping PFD information provided by NWDAF and provided by AF, the latter will take precedence over the PFD information provided by NWDAF.

The below is the principles of the solution:

-	A consumer (e.g. NEF) requests to the NWDAF to provide PFDs for newly detected applications.

- [Optionally] Use the filter “Application Characteristics”.

-	The NWDAF requests to provide information on SDF that does not match an installed PDR at the SMF/UPF.

-	The NWDAF analyses the collected data to generate unique traffic patterns, and provides it to the consumer (i.e. NEF).

-	The consumer i.e. NEF stores PFD associated with an application ID to enable the detection of application traffic in the future (out of scope).

### 6.9.2	Input Data

NWDAF collects QoS flow related data from SMF for a specific S-NSSAI, DNN, and UE. The detailed data are described in Table 6.9.2-1.

Table 6.9.2-1: Input data to detect new application from NFs

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF / UPF | UE ID for the UE who uses the application. |
| S-NSSAI | SMF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Start/end time | UPF | Start and end time of traffic detection. |
| Data volume | UPF | Measured data traffic volume (per UL/DL). |
| Data duration | UPF | Measured average data traffic (packets) duration (per UL/DL). |
| QoS flow Bit Rate | UPF | The observed bit rate (per UL/DL). |
| Packet transmission | UPF | The observed average number of packet transmission (per UL/DL). |
| UL/DL Packet Delay | UPF, AF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| IP 3-tuple | UPF | to Identify a service flow of the UE who uses the application. |
| Packet's URL | UPF | URL is extracted from the inspected user plane packets. |
| Packet's Domain Name | SMF/UPF | The domain name is extracted from the inspected user plane packets in a QoS flow. |
| Size of packets | SMF/UPF | Average size of packets. |
| PFD Information | ADRF | Historical PFD Information stored in ADRF |
| > Application ID | ADRF | Identification of the application that refers to one or more application defection filters. |
| > IP 3-tuple | ADRF | Including protocol, server side IP address and port number. |
| > URL | ADRF | the significant parts of the URL to be matched, e.g. host name. |
| > Domain Name | ADRF | a Domain name matching criteria and information about applicable protocol(s). |
| PFD Information | NEF | PFD Information stored in the NRF and retrieved by NEF, as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF | Identification of the application that refers to one or more application defection filters. |
| > IP 3-tuple | NEF | Including protocol, server side IP address and port number. |
| > URL | NEF | the significant parts of the URL to be matched, e.g. host name. |
| > Domain Name | NEF | a Domain name matching criteria and information about applicable protocol(s). |

NOTE:	Extensive reporting of all traffic flows may conflict with requirement to avoid extra UPF load in the key issue The issue regarding the performance of UPF user plane traffic handling will depend on the outcome of FS-UPEAS SID.

### 6.9.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.9.3-1.

The output analysis can be used to provision new PFDs for known applications, or define new PFDs for new applications, not known to NEF yet. Upon receiving the analytics including new PFDs information from the NWDAF, the NEF can assign a PFD ID for the unknown application. And, the assigned PFD ID can be used to identify/audit/report the unknown application.

Table 6.9.3-1: Output for new application detection

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) stored in the UDR. In the case of detection of unknown application, this parameter has a null value. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol). |
| URL | the significant parts of the URL to be matched, e.g. host name defined in. |
| Domain name information | a Domain name matching criteria and information about applicable protocol(s). |

### 6.9.4	Procedures

The procedure depicted in 6.9.4-1 shows new application detection analytics.

Figure 6.9.4-1: A procedure to extract application characteristics

1.	The consumer NF subscribes to the NWDAF to request analytics for application detection. This subscription maybe is triggered by local configuration or OAM. The Analytics Filter Information may optionally include the UE ID, S-NSSAI and/or DNN.

2.	The NWDAF fetches currently stored PFD information from NEF(PFD) and historical data from ADRF (not shown in the figure).

3.	The NWDAF collects session related information from the UPF about URL, Domain name part, and IP 3-tuples of packets from the SDF not matching installed PDRs.

4.	The NWDAF derives PFD analytics.

5.	The NWDAF notifies the analytics consumer NF with outputs.

### 6.9.5	Impacts on existing nodes and functionality

NWDAF:

-	Needs to provide a new analytic for application detection information.

-	Needs to process user plane data for extracting traffic characteristics.

-  	Needs to be able to filter and name the applications detected based on the “Application Characteristics” filter.

UPF:

-	Needs to report URL, Domain name part, and IP 3-tuples of packets from unknown application to NWDAF.

-	Needs to provide requested sessions statistics including data volume, data duration, QoS flow bit rate and packet transmission.

NEF (PFDF):

-	Supports to consume new PFD information from NWDAF.

Editor's note:	ADRF impacts are FFS.

********** End Changes*************