3GPP TSG-SA WG2 Meeting #152E (e-meeting)	S2-2205551

# August 17-26, 2022, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0533 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on granularity of time and location for NWDAF analytics results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NF clients may send a request for analytic to NWDAF for a specific time period (Analytics target period). For both cases (time period in the past or in the future), the value of the analytic might be quite different in different sub-periods within the analytics target period. The NWDAF provides to the consumer the value of the analytic for the requested Analytics target period, but it is not clear whether the value is provided as an average for the whole target period or different instances of the analytic are provided where each instace includes the value of the analytic for each sub-period. A similar thing applies with the spatial locations. So the value of the analytic result might be quite different depending on the area (including cases where the request applies to the whole PLMN, or a big AoI). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that for every analytic request, the NWDAF may decide to provide in the result either a single instance of the analytic with the average value of the analytic for the whole requested analytic target period or multiple instances for different sub-periods within the analytic target period. The decision may be based on the length of the requested analytic target period and the evolution of the predicted value of the analytic during such target period. Similar clarification for spatial locations. So, the NWDAF may decide to provide one or multiple instances depending on the extension of the requested area and the evolution of the predicted value of the analytic within such area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Analytic results from the NWDAF could be not granular enough in terms of time and location, so NF clients might apply wrong actions based on these analytics during some time periods and/or within some location areas. Also it creates CT3 ambiguity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3, 6.3.1, 6.3.3A, 6.4.3, 6.7.5.3, 6.12.3, 6.13.3, 6.14.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

### 6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7 provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information is defined in the analytics related clauses.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Subscription Correlation ID: identifies an existing analytics subscription that is to be modified.

-	Related to analytic consumers that aggregate analytics from multiple NWDAF subscriptions:

-	[OPTIONAL] (Set of) NWDAF identifiers of NWDAF instances used by the NWDAF service consumer when aggregating multiple analytics subscriptions. See clause 6.1A.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1 of TS 23.502 [3].

NOTE 1:	When the Analytics Reporting Parameters indicates a periodic reporting mode and the periodicity of the report is equal to or greater than the Supported Analytics Delay associated with the Analytics ID (if available) defined in clause 6.2.6.2 of TS 23.501 [2], it is expected that the periodic reporting can be provided by the NWDAF as requested.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF.

-	[OPTIONAL] Matching direction: A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

-	[OPTIONAL] Acceptable deviation: An acceptable deviation from the threshold level in the non-critical direction (i.e. in which the QoS is improving) may be set to limit the amount of signalling.

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time, which indicates a subscription for predictions or statistics respectively. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	[OPTIONAL] Data time window: if specified, only events that have been created in the specified time interval are considered for the analytics generation.

-	[OPTIONAL] Preferred level of accuracy of the analytics ("Low", "Medium", "High" or "Highest").

-	[OPTIONAL] Preferred level of accuracy per analytics subset ("Low", "Medium", "High" or "Highest"). When a preferred level of accuracy is expressed for a given analytics subset, it takes precedence for this subset over the above preferred level of accuracy of the analytics. Analytics subsets are defined in the "Output Analytics" clause of applicable analytics.

-	[OPTIONAL] Dataset Statistical Properties: information in order to influence the data selection mechanisms to be used for the generation of an Analytics ID, assuring that the generated Analytics ID reflects the statistical characteristics of the data that are relevant for the NWDAF consumer. The following dataset statistical properties are allowed:

-	Uniformly distributed datasets, which indicates the use of data samples that are uniformly distributed according to the different aspects of the requested analytics (e.g. equivalent data samples for each UE listed as a Target of Analytics Reporting or for S-NSSAIs included in the Analytics Filter Information).

-	Datasets with or without outliers, which indicates that the data samples shall consider or disregard data samples that are at the extreme boundaries of the value range.

-	Time when analytics information is needed (if applicable): indicates to the NWDAF the latest time the analytics consumer expects to receive analytics data provided by the NWDAF. It should not be set to a value less than the Supported Analytics Delay of the selected NWDAF if applicable. If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response or error notification to the consumer. "Time when analytics information is needed" is a relative time interval as the gap with respect to analytics request /subscription (e.g. "in 10 minutes").

NOTE 2:	If the analytics request contains the parameter "Time when analytics information is needed" for Analytics ID(s), this parameter takes precedence over the requested periodicity, if a periodic reporting mode is requested.

NOTE 3:	If the Time when analytics information is needed is provided and it is less than the Supported Analytics Delay per Analytics ID (if available) defined in clause 6.2.6.2 of TS 23.501 [2], it is expected that the NWDAF might not be able to treat the Analytics ID on time.

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Preferred granularity of location information: TA level or cell level.

-	[OPTIONAL] Preferred order of results when a list of analytics is returned, possibly with a criterion for identifying the property of the results to which the preferred ordering is applied.

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shall return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

-	[OPTIONAL] Output strategy: indicates the relevant factors for determining when the analytics reported. The following values are allowed:

-	Binary output strategy: indicates that the analytics shall only be reported when the preferred level of accuracy is reached within a cycle of periodic notification as defined in the Analytics Reporting Parameters.

NOTE 4:	If preferred level of accuracy is more important than providing an output, then the binary strategy is used so that all analytics outputs have equivalent confidence in the prediction.

-	Gradient output strategy: indicates that the analytics shall be reported according with the periodicity defined in the Analytics Reporting Parameters irrespective if the preferred level of accuracy has been reached.

NOTE 5:	If having an analytics output is more important than reaching the preferred level of accuracy, then the gradient output strategy is used so that each NWDAF will timely provide the output indicating the confidence of the prediction at the moment of the output generation.

NOTE 6:	When no output strategy is included in the subscription, the analytics output will be generated based on the gradient strategy and includes the confidence of the prediction for the reporting period.

-	[OPTIONAL] Analytics metadata request: indicates a request from one NWDAF to another NWDAF to provide the "analytics metadata information" related to the produced output analytics. This input parameter indicates which parameters in "analytics metadata information" are required to aggregate the output analytics for the requested Analytics ID(s).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Consumer NF's serving area or NF ID. During a pending analytics subscription transfer, this information can be used by the NWDAF to find out if the analytics consumers may change as described in clause 6.1B.2.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Information of previous analytics subscription. When setting up the analytics generation, this information may be used to retrieve analytics context from the previous NWDAF in order to build upon the context that is already related to this subscription as described in clause 6.1B.2.1.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription_Notify) The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period. The analytics information may include a single instance covering the whole requested Analytics target period or multiple instances where each instance provides the value of the analytics information for a sub-period within the Analytics target period. The same also applies for the Spatial Validity.

-	Timestamp of analytics generation: allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period: defines the time period for which the analytics information is valid.

-	Confidence: probability assertion:, i.e. confidence in the prediction.

-	[OPTIONAL] For each Analytics ID the Termination Request, which notifies the consumer that the subscription is requested to be cancelled as the NWDAF can no longer serve this subscription, e.g. due to user consent revoked, NWDAF overload, UE moved out of NWDAF serving area, etc.

-	[OPTIONAL] Analytics metadata information: additional information required to aggregate the output analytics for the requested Analytics ID(s). This parameter shall be provided if the "Analytics metadata request" parameter was provided in the corresponding Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation.

-	Number of data samples used for the generation of the output analytics;

-	Data time window of the data samples;

-	Dataset Statistical Properties of the analytics output used for the generation of the analytics;

-	[OPTIONAL] Data source(s) of the data used for the generation of the output analytics;

-	[OPTIONAL] Data Formatting and Processing applied on the data;

-	Output strategy used for the reporting of the analytics.

-	(Only for error response or error notification) Revised waiting time: indicates to the consumer a revised waiting value for "Time when analytics information is needed". Each NWDAF may include this as part of error response or error notification to "Time when analytics information is needed" as described in clause 6.2.5. Revised waiting time is the minimum time interval recommended by NWDAF to use as "Time when analytics information is needed" for similar future analytics requests/subscriptions.

-    [OPTIONAL] Spatial Validity which is the area where the analytics information applies.

* * * * Next Change * * * *

### 6.3.1	General

The NWDAF provides slice load level information to a consumer NF on a Network Slice level or a Network Slice instance level or both. The NWDAF is not required to be aware of the current subscribers using the slice. The NWDAF notifies slice specific network status analytics information to the consumer NF that is subscribed to it. A consumer NF may collect directly slice specific network status analytics information from NWDAF. This information is not subscriber specific.

The NWDAF services as defined in the clause 7.2 and clause 7.3 are used to expose slice load level analytics from the NWDAF to the consumer NF (e.g. PCF, NSSF or AMF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "Load level information";

-	Analytics Filter Information:

-	S-NSSAI and NSI ID;

-	optionally, the list of analytics subsets that are requested among those specified in clause 6.3.3A;

NOTE:	The use of NSI ID in the network is optional and depends on the deployment choices of the operator. If used, the NSI ID is associated with S-NSSAI. NSI ID is only applicable when the consumer of analytics is NSSF or AMF.

-	an optional Area of Interest;

-	an optional list of NF types;

-	optionally, Load Level Threshold value; and

-	optionally, "maximum number of objects" indicating the maximum number of Network Slice instances expected in output, when the Analytics Filter Information does not indicate an NSI ID.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

* * * * Next Change * * * *

### 6.3.3A	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the following analytics:

-	Network Slice instance load statistics information as defined in Table 6.3.3A-1.

-	Network Slice load statistics information as defined in Table 6.3.3A-2.

-	Network Slice instance load predictions information as defined in Table 6.3.3A-3.

-	Network Slice load predictions information as defined in Table 6.3.3A-4.

Table 6.3.3A-1: Network Slice instance load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| Network Slice instances (1..max) | List of Network Slice instance(s) within the S-NSSAI. |
| > NSI ID | Identification of the Network Slice instance. |
| > Number of UE Registrations (NOTE 1) | Number of UE registrations of the Network Slice instance (average, variance). |
| > Number of PDU Sessions establishment (NOTE 1) | Number of PDU Session establishments of the Network Slice instance (average, variance). |
| > Resource usage (NOTE 1) | The usage of assigned virtual resources currently in use for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (NOTE 1) | Number of times resource usage threshold is met or exceeded or crossed on the Network Slice instance provided if threshold is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (1..max) (NOTE 1) | Resource usage threshold crossing vector including time elapsed between times each threshold is met or exceeded or crossed on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if Load Level Threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the statistics value of the Load Level if the Load Level Threshold is provided by the consumer as Analytics Filter. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-2: Network Slice load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| > Number of UE Registrations (NOTE 1) | Number of UE registrations at the Network Slice (average, variance). |
| > Number of PDU sessions establishments (NOTE 1) | Number of PDU Session establishments at the Network Slice (average, variance). |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if Load Level Threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the statistics value of the Load Level if the Load Level Threshold is provided by the consumer as Analytics Filter. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-3: Network Slice instance load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| Network Slice instances (1..max) | List of Network Slice instance(s) within the S-NSSAI. |
| > NSI ID | Identification of the Network Slice instance. |
| > Number of UE Registrations (NOTE 1) | Number of predicted UE registrations at the Network Slice instance (average, variance). |
| > Number of PDU Sessions establishment (NOTE 1) | Number of predicted PDU Session establishments of the Network Slice instance (average, variance). |
| > Resource usage (NOTE 1) | The predicted usage of assigned virtual resources for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (NOTE 1) | Number of predicted times resource usage threshold is met or exceeded or crossed at the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (1..max) (NOTE 1) | Predicted Resource usage threshold vector including predicted time elapsed between times each threshold is met or exceeded or crossed on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if Load Level Threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the predicted value of the Load Level if the Load Level Threshold is provided by the consumer as Analytics Filter. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-4: Network Slice load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| > Number of UE Registrations (NOTE 1) | Predicted Number of UE registrations at the Network Slice (average, variance). |
| > Number of PDU sessions establishments (NOTE 1) | Predicted Number of PDU Session establishments at the Network Slice (average, variance). |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if Load Level Threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the predicted value of the Load Level if the Load Level Threshold is provided by the consumer as Analytics Filter. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

NOTE:	If no NSI ID is provided as Analytics Filter, slice load level related output analytics are provided according to Tables 6.3.3A-2 and 6.3.3A-4. Otherwise slice instance load level related output analytics are provided according to Tables 6.3.3A-1 and 6.3.3A-3.

The predictions are provided with a Validity Period and may also include Spatial Validity, as defined in clause 6.1.3.

* * * * Next Change * * * *

### 6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI | Identifies the Network Slice |
| > NSI ID (NOTE 2) | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) for which the slice instance service experience applies. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| Application service experiences (0..max) | List of observed service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location (NOTE 1, NOTE 5) | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered. |
| > UPF Info (NOTE 4) | Indicating UPF serving the UE. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN (NOTE 4) | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) with the same application service experience. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > RAT Type (NOTE 7) | Indicating the list of RAT type(s) for which the application service experience analytics applies. |
| > Frequency (NOTE 7) | Indicating the list of carrier frequency value(s) of UE's serving cell(s) where the application service experience analytics applies. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested. NOTE 2: The NSI ID is an optional parameter. If not provided the Slice instance service experience indicates the service experience for the S-NSSAI. NOTE 3: The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience. NOTE 4: If the consumer NF is an AF, the item "DNN" and "UPF info" shall not be included, and the NEF is responsible for translation of SUPI to GPSI, internal group identifiers to external ones, and UE location to geographical area, by querying UDM, prior to contacting the AF. NOTE 5: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. UE location shall only be included if the Consumer analytics request is for single UE or a list of UEs. Inclusion of UE location requires user consent. NOTE 6: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. NOTE 7: When "any" value has been provided in the request (e.g. "any" RAT type, "any" frequency, or "any" for all the RAT type and frequency indication), the NWDAF provides an instance of the Application service experience per combination of RAT Type(s) and/or Frequency value(s) having the same Service Experience. |  |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI | Identifies the Network Slice |
| > NSI ID (NOTE 2) | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) for which the slice instance service experience applies. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| Application service experiences (0..max) | List of predicted service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location (NOTE 1, NOTE 5) | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered. |
| > UPF Info (NOTE 4) | Indicating UPF serving the UE. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN (NOTE 4) | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) with the same application service experience. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| > RAT Type (NOTE 7) | Indicating the list of RAT type(s) for which the application service experience analytics applies. |
| > Frequency (NOTE 7) | Indicating the list of carrier frequency value(s) of UE's serving cell(s) where the application service experience analytics applies. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 2: The NSI ID is an optional parameter. If not provided the Slice instance service experience indicates the service experience for the S-NSSAI. NOTE 3: The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience. NOTE 4: If the consumer NF is an AF, the item "DNN" and "UPF info" shall not be included, and the NEF is responsible for translation of SUPI to GPSI, internal group identifiers to external ones, and UE location to geographical area, by querying UDM, prior to contacting the AF. NOTE 5: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. UE location shall only be included if the Consumer analytics request is for single UE or a list of UEs. Inclusion of UE location requires user consent. NOTE 7: When "any" value has been provided in the request (e.g. "any" RAT type, "any" frequency, or "any" for all the RAT type and frequency indication), the NWDAF provides an instance of the Application service experience per combination of RAT Type(s) and/or Frequency value(s) having the same Service Experience. |  |

The number of Service Experiences and SUPIs are limited respectively by the maximum number of objects and the Maximum number of SUPIs provided as part of Analytics Reporting Information by the NWDAF Service Consumer.

* * * * Next Change * * * *

#### 6.7.5.3	Output Analytics

Corresponding to the "abnormal behaviour" Analytics ID, the analytics result provided by the NWDAF is defined in Table 6.7.5.3-1 and Table 6.7.5.3-2. When the level of an exception trespasses above or below the threshold, the NWDAF shall notify the consumer with the exception ID associated with the exception if the exception ID is within the list of exception IDs indicated by the consumer or matches the expected analytics type indicated by the consumer. The NWDAF shall provide the Exception Level and determine which of the other information elements to provide, depending on the observed exception.

Abnormal behaviour statistics information is defined in Table 6.7.5.3-1.

Table 6.7.5.3-1: Abnormal behaviour statistics

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of observed exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Scalar value indicating the severity of the abnormal behaviour |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics | Internal Group Identifier, TAC |
| > SUPI list (1..SUPImax) | SUPI(s) of the UE(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Amount | Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE") |
| > Additional measurement | Specific information for each risk (see Table 6.7.5.3-3) |

Abnormal behaviour predictions information is defined in Table 6.7.5.3-2.

Table 6.7.5.3-2: Abnormal behaviour predictions

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of predicted exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Scalar value indicating the severity of the abnormal behaviour |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics | Internal Group Identifier, TAC |
| > SUPI list (1..SUPImax) | SUPI(s) of the UE(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Amount | Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE") |
| > Additional measurement | Specific information for each risk (see Table 6.7.5.3-3) |
| > Confidence | Confidence of this prediction |

The predictions are provided with a Validity Period and may also include Spatial Validity, as defined in clause 6.1.3.

The UE characteristics may provide a set of features common to all UEs affected with the exception.

The number of exceptions and the length of the SUPI list shall respectively be lower than the parameters maximum number of objects and Maximum number of SUPIs provided as part of Analytics Reporting Information.

If PCF subscribes to notifications on "Abnormal behaviour", the NWDAF shall send the PCF notifications about the risk, which may trigger the PCF to update the AM/SM policies.

The NWDAF also sends the notification directly to the AMF or SMF, if the AMF or SMF subscribes to the notification, so that the AMF or SMF may, based on operator local policies defined on a per S-NSSAI basis (for AMF) or on a per S-NSSAI, per DNN, or per (DNN,S-NSSAI) basis (for SMF), take actions for risk solving.

If the AF subscribes to notifications on "Abnormal behaviour", the NWDAF sends the notifications to the AF so that the AF may take actions for risk solving.

The following Table 6.7.5.3-3 gives examples of additional measurement provided by the NWDAF and examples of NF actions for solving each risk.

Table 6.7.5.3-3: Examples of additional measurements and NF actions for risk solving

| Exception ID and description | Additional measurement | Actions of NFs |
| --- | --- | --- |
| Unexpected UE location | Unexpected UE location (TA or cells which the UE stays) | PCF may extend the Service Area Restrictions with current UE location. AMF may extend the mobility restriction with current UE location. |
| Ping-ponging across neighbouring cells | Numbers, frequency, time and location information, assumption about the possible circumstances of the ping-ponging | If the ping-ponging are per UE, then: 1. the AMF may adjust the UE (e.g. a stationary UE) registration area. 2. the AMF and/or the AF may allow the use of Coverage Enhancement for the affected UE. |
| Unexpected long-live/large rate flows | Unexpected flow template (IP address 5 tuple) | SMF updates the QoS rule, e.g. decrease the MBR for the related QoS flow. PCF, if dynamic PCC applies for corresponding DNN, S-NSSAI, updates PCC Rules that triggers SMF updates the QoS rule, e.g. decrease the MBR for the related QoS flow. |
| Unexpected wakeup | Time of unexpected wake-up | AMF applies MM back-off timer to the UE. |
| Suspicion of DDoS attack | Victim's address (target IP address list) | PCF may request SMF to release the PDU session. SMF may release the PDU session and apply SM back-off timer. |
| Wrong destination address | Wrong destination address (target IP address list) | PCF updates the packet filter in the PCC Rules that triggers the SMF to update the related QoS flow and configures the UPF. |
| Too frequent Service Access | Volume, frequency, time, assumptions about the possible circumstances | AF may release the AF session. PCF may request SMF to release the PDU session. SMF may release the PDU session and apply SM back-off timer. |
| Unexpected radio link failures | Numbers, frequency, time and location, assumptions about the possible circumstances | If the unexpected radio link failures are per UE location bases, the AMF may allow the use of CE (Coverage Enhancement) in the affected location. Also, the Operator may improve the coverage conditions in the affected location. If the unexpected radio link failures are per UE bases, then the AMF and/or the AF may allow the use of CE for the affected UE. |

* * * * Next Change * * * *

### 6.12.3	Output Analytics

The NWDAF outputs the SMCCE statistical analytics. The detailed statistical information provided by the NWDAF is defined in Table 6.12.3-1.

Table 6.12.3-1: SMCCE statistics

| Information | Description |
| --- | --- |
| List of SMCCE Analytics (1..max) |  |
| > DNN | DNN that SMCC is applied. |
| > S-NSSAI | S-NSSAI that SMCC is applied. |
| > List of UEs classified based on experience level of SMCC | One, or more than one, of the following lists (SUPI is used to identify a UE). |
| >> List of high-experienced UEs (NOTE 2) | A list of UEs whose experience level of SMCC for specific DNN and/or S-NSSAI is high. |
| >> List of medium-experienced UEs (NOTE 2) | A list of UEs whose experience level of SMCC for specific DNN and/or S-NSSAI is medium. |
| >> List of low-experienced UEs (NOTE 2) | A list of UEs whose experience level of SMCC for specific DNN and/or S-NSSAI is low. |
| NOTE 1: The high/medium/low-experience thresholds values are operator defined. NOTE 2: This information element is an analytics subset that can be used in "list of analytics subsets that are requested". |  |

The predictions are provided with a Validity Period, as defined in clause 6.1.3.

* * * * Next Change * * * *

### 6.13.3	Output Analytics

The NWDAF supporting data analytics on Redundant Transmission Experience shall be able to provide Redundant Transmission Experience analytics as defined in Table 6.13.3-1 and Table 6.13.3-2.

Table 6.13.3-1: Redundant Transmission Experience statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID, any UE | Identifies a UE, any UE, or a group of UEs. |
| DNN | Data Network Name associated for URLLC service. |
| Spatial validity | Area where the Redundant Transmission Experience applies. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot (average and variance). |
| > Redundant Transmission Experience | Redundant Transmission Experience value. |
| > Ratio | Percentage on which UE, any UE, or UE group efficiently use the PDU session with redundant transmission. |

Table 6.13.3-2: Redundant Transmission Experience predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID, any UE | Identifies a UE or, any UE, a group of UEs. |
| DNN | Data Network Name associated for URLLC service. |
| Spatial validity | Area where the estimated Redundant Transmission Experience applies. |
| Time slot entry (1..max) | List of predicted time slots. |
| >Time slot start | Time slot start time within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Redundant Transmission Experience | Predicted Redundant Transmission Experience value during the Analytics target period. |
| > Ratio | Percentage on which the UE, any UE, or UE group may efficiently use the PDU session with redundant transmission. |
| > Confidence | Confidence of this prediction. |

* * * * Next Change * * * *

### 6.14.1	General

This clause specifies how an NWDAF can provide DN Performance Analytics which provides analytics for user plane performance (i.e. average/maximum traffic rate, average/maximum packet delay, average packet loss rate) in the form of statistics or predictions to a service consumer.

The DN Performance Analytics may provide one or a combination of the following information:

-	User plane performance analytics for a specific Edge Computing application for a UE, group of UEs, or any UE over a specific serving anchor UPF.

-	User plane performance analytics for a specific Edge Computing application for a UE, group of UEs, or any UE over a specific DNAI.

-	User plane performance analytics for a specific Edge Computing application for a UE, group of UEs, or any UE over a specific Edge Application Server Instance.

The service consumer may be an NF (e.g. SMF) or an AF.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "DN Performance";

-	Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in table 6.14.1-1; and

-	optionally, a preferred level of accuracy of the analytics;

-	optionally, preferred level of accuracy per analytics subset (see clause 6.14.3);

-	optionally, preferred order of results for the list of Network Performance information:

-	ordering criterion: one of the analytics subset (see clause 6.14.3);

-	order: ascending or descending;

-	optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics subsets (see clause 6.14.3) in order to be notified by the NWDAF;

-	optionally, maximum number of objects and maximum number of SUPIs.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

Table 6.14.1-1: Analytics Filter Information related to DN Performance Analytics

| Information | Description |
| --- | --- |
| Application ID (0..max) | The identification of the application(s) for which the analytics information is subscribed or requested. |
| S-NSSAI | Identifies the Network Slice for which analytics information is subscribed or requested. |
| NSI ID(s) | Identifies the Network Slice instance(s) for which analytics information is subscribed or requested. |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] for which the analytics information is subscribed or requested. |
| UPF anchor identity | Identifies the UPF where a UE has an associated PDU session. |
| DNN | DNN to access the application. |
| DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| Application Server Address(es) | List of IP address(s)/FQDN(s) of the Application Server(s) that a UE, group of UEs, or 'any UE' has a communication session with for which DN Performance Analytic information is requested. |
| List of analytics subsets | List of analytics subsets that are requested among those specified in clause 6.14.3. |
| NOTE: All parameters are optional. |  |

* * * * End of Changes * * * *