3GPP TSG-WG SA2 Meeting #152-e	S2-2205622

E-meeting, 2022‑08‑17 -- 2022‑08‑26	(revision of S2-220xxxx)

Source:	Ericsson

Title:	KI #4, New Sol: PCF controlling common DNAI

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract: This contribution proposes a solution for how the 5GC can select a common DNAI for a set of UEs.

# 1	Discussion

Proposed CRs S2-2205633, S2-2205634 and S2-2205635 outlines how several SMF instances can be used when indication for traffic is provided by the AF.

# 2	Proposal

The following solution is proposed to be include in the TR 23.700-48

**************** Start changes **************

## 6.X	solution X: PCF controlling common DNAI

### 6.X.1	Description

This solution addresses KI#4,

The solution addresses how 5GC can select a common DNAI for a set of UEs,

It is assumed that the AF/EASs solves the issue with a common EAS for the set of UEs,

The set of UEs is identified by the AF by the external group ID. It is assumed that the AF creates the UE set (group) by means of 5G VN group management procedures.

### 6.X.2 procedure

The AF will request a common DNAI from 5GC in AF influence on routing by indicating correlation of traffic for a group of UEs. The group is identified by the External Group ID, which is translated by 5GC (UDM via NEF) to an internal group id.

The request for influencing traffic ends up in PCF, via UDR, see TS 23.502 [nn] clause 4.3.6.2. Once PCF has received the request for the group of UEs, it will need to send PCC rules to the concerned PDU sessions. PCF starts with sending the PCC rule via Npcf_SMPolicyControl_UpdateNotify including indication of traffic correlation and the related Internal Group ID to the SMF having most of the PDU sessions that should be correlated. SMF determines the common DNAI for the Internal Group and responds with the selected DNAI to the PCF. PCF then sends Npcf_SMPolicyControl_UpdateNotify with indication of traffic correlation, the related Internal Group and selected DNAI to the rest of the PDU sessions for the UEs identified by the Internal Group ID.

### 6.X.3	Impacts on Existing Nodes and Functionality

Note that No impact if CR on 23.501, 23.502 and 23.503 in S2-2005633, S2-2005634, S2-2005635 is agreed.

Otherwise impacts on:

SMF:	Selecting common DNAI. Give it to PCF, use selected DNAI when ordered from PCF.

PCF	Receiving and sending selected DNAI.

**************** Next changes **************

## 6.0	Solution-Key issue matrix

The solutions in clause 6 can apply to one or more key issues described in clause 5 of this report. Table 6.0-1 describes the relationship between solutions and key issues.

Editor's note:	The table below will be updated with actual content when generating the TR with approved contributions. Page number is automatically updated to ease reference (ctrl-left click to reach the solution).

Table 6.0-1: Solution-Key issue matrix

| Solution |  | Key issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | (page) | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 | KI#7 |
| 01: EAS discovery in Home Routed roaming scenario | 22 | X |  |  |  |  |  |  |
| 02: Session Breakout in Visited PLMN | 26 | X |  |  |  |  |  |  |
| 03: EAS (re)discovery procedure in roaming scenario | 29 | X |  |  |  |  |  |  |
| 04: Support EAS (re-)discovery in VPLMN via HR PDU Session | 32 | X |  |  |  |  |  |  |
| 05: Accessing V-EHE via HR PDU Session | 38 | X |  |  |  |  |  |  |
| 06: URSP solution to support roamers access to EHE in a VPLMN | 40 | X |  |  |  |  |  |  |
| 07: Using URSP Rules to Establish an LBO PDU Session | 42 | X |  |  |  |  |  |  |
| 08: V-ECS Discovery during Steering of Roaming | 43 | X |  |  |  |  |  |  |
| 09: PDU Session configuration from EASDF | 45 | X |  |  |  |  |  |  |
| 10: LBO PDU Session establishment using PLMN criteria in RSD | 47 | X |  |  |  |  |  |  |
| 11: Exposure of Network Congestion | 49 |  | X |  |  |  |  |  |
| 12: Efficient exposure of RAN information | 52 |  | X |  |  |  |  |  |
| 13: Fast and efficient network exposure improvements | 56 |  | X |  |  |  |  |  |
| 14: Group Management | 58 |  |  |  | X |  |  |  |
| 15: Selection of common DNAI | 62 |  |  |  | X |  |  |  |
| 16: Selecting the same EAS/DNAI for collection of UEs | 70 |  |  |  | X |  |  |  |
| 17: Application layer EAS selection for collections of UEs | 73 |  |  |  | X |  |  |  |
| 18: Discovery of the same EAS for collections of UEs | 75 |  |  |  | X |  |  |  |
| 19: Influencing UPF and EAS (re)location for collections of UEs | 79 |  |  |  | X |  |  |  |
| 20: Global EASDF | 81 |  |  |  |  | X |  |  |
| 21: EAS Deployment information differentiated by PLMN ID | 83 |  |  |  |  | X |  |  |
| 22: EAS discovery for federated OPs | 84 |  |  |  |  | X |  |  |
| 23: Improvements for EHE operated by separate party | 90 |  |  |  |  | X |  |  |
| 24: Reuse Option D after UL-CL insertion | 92 | X |  |  |  |  |  |  |
| 25: EAS discovery in VPLMN via V-EASDF for a HR PDU Session | 94 | X |  |  |  |  |  |  |
| 26: SM Policy for HR Session Breakout in VPLMN | 97 | X |  |  |  |  |  |  |
| 27: EAS discovery with dynamic setup of a LBO PDU Session | 100 | X |  |  |  |  |  |  |
| 28: Support edge computing in Roaming | 102 | X |  |  |  |  |  |  |
| 29: Use of Internal Group ID and constraints in EDI | 107 |  |  | X |  |  |  |  |
| 30: Policies referring to "Allowed services" and/or "Subscriber categories" | 108 |  |  | X |  |  |  |  |
| 31: Providing traffic offload policy for a set of UEs with service information | 111 |  |  | X |  |  |  |  |
| 32: Offload policy for finer granular set of UEs | 114 |  |  | X |  |  |  |  |
| 33: AF requests offload policy for sets of UEs | 115 |  |  | X |  |  |  |  |
| 34: Selecting the same EAS/DNAI for collection of UEs | 116 |  |  |  | X |  |  |  |
| 35: Providing dedicated (re)location information as traffic routing information | 119 |  |  |  | X |  |  |  |
| 36: Providing dedicated (re)location information as EAS Deployment information | 123 |  |  |  | X |  |  |  |
| 37: (Re)location of same EAS and coordination across UEs | 126 |  |  |  | X |  |  |  |
| 38: EAS Discovery for EHE shared with other PLMN | 128 |  |  |  |  | X |  |  |
| 39: Support EAS relocation of inter-PLMN | 129 | X |  |  |  | X |  |  |
| 40: EAS discovery for shared EHE | 133 |  |  |  |  | X |  |  |
| 41: Controlling non-3GPP access of EC traffic via URSP and ATSSS | 136 |  |  |  |  |  | X |  |
| 42: Network-guided EC traffic switching | 139 |  |  |  |  |  | X |  |
| 43: Network-based solution for keeping EC traffic on 3GPP Access | 141 |  |  |  |  |  | X |  |
| 44: EAS traffic switching avoidance | 144 |  |  |  |  |  | X |  |
| 45: Application selected PDU Session | 146 | X |  |  |  |  | X |  |
| 46: Avoid UE switching on-going EC traffic away from 3GPP access | 147 |  |  |  |  |  | X |  |
| 47: Avoiding Switch Away Based on an SMF Indication | 149 |  |  |  |  |  | X |  |
| 48: Avoiding Switch Away Based on an Indication in the URSP | 150 |  |  |  |  |  | X |  |
| 49: URSP based solution to avoid UE to switch away from Edge PDU Session | 151 |  |  |  |  |  | X |  |
| 50: Obtain and maintain mapping table between IP address/IP range with DNAI | 152 |  |  |  |  |  |  | X |
| 51: EDI holding the IP address to DNAI mapping | 154 |  |  |  |  |  |  | X |
| 52: AF obtaining target DNAI provided by NEF | 155 |  |  |  |  |  |  | X |
| Xx: PCF controlling common DNAI | nnn |  |  |  | X |  |  |  |

**************** End changes **************