3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205716

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1, Sol#5 Update the procedure for the RFSP index transfer from AMF to HSS via UDM

Document for:	Approval

Agenda Item:	9.15

Work Item / Release:	FS_AMP / Rel-18

Abstract: This paper proposes to update the Sol#5 to clarify the service operation when providing the RFSP index from AMF to HSS via UDM.

1. Introduction/Discussion

In the current Solution#5, the service operation used for the "Authorized RFSP index" and the Validity period transferring from AMF to the HSS via UDM is unknown.

This paper proposed to use the following service operations:

-	AMF to UDM: Nudm_UECM_Update;

-	UDM to HSS: Nhss_UEContextManagement.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-89.

# * * * * First change * * * *

6.5	Solution #5: Authorized RFSP index provisioning from 5GC to MME without N26 interface

6.5.1	Description

In the current interworking procedures from 5G to 4G, when the UE moves from 5G to 4G, the PCF should provide the "authorized RFSP index" to the MME.

However, after the UE moves from 5G to 4G, all the UE's context in the 5G side should be released (including the AM policy association for the UE between AMF and PCF), so the PCF cannot update the RFSP index for the UE when it finds that the "authorized RFSP index" is not valid, which resulted in that the MME does not know when it should in turn to use the "subscribed RFSP index" instead of the "authorized RFSP index".

As specified in clause 6.1.1.3 of TS 23.503 [4], PCF may use the network analytics on "Service Experience" for an Application Identifier, "any RAT type" and/or "any Frequency value" to determine the "authorized RFSP index" value for the UE when running this application. Furthermore, as specified in clause 6.4.3 of TS 23.288 [10], the "Service Experience" analytics including a Validity period for the Application service experience analytics. So, it is possible for the PCF to determine the "authorized RFSP index" together with a Validity period.

When N26 interface is not supported, the authorized RFSP index and the Validity period can be provided by the PCF to the UDM/HSS via AMF during the without N26-based interworking procedures, then MME obtains the the authorized RFSP index and the Validity period from the HSS.

The MME continues to use the "Authorized RFSP" over the "Subscribed RFSP" during the Validity period. When the Validity period expires, the MME may then send the "Subscribed RFSP" as the RFSP to use to the eNB.

6.5.2	Procedures

Figure 6.5.2-1depicts the procedure of propose solution of providing "authorized RFSP index" provisioning from 5GC to MME without N26 interface.

Figure 6.5.2-1: "authorized RFSP index" provisioning from 5GC to MME in without N26-base architecture

The overall procedure re-uses the 5GS to EPS mobility without N26 interface as defined in TS 23.502 [3]. The change is:

-	Step 1 and Step 2. PCF determines the "authorized RFSP index" together with the Validity period e.g. based on the "Service Experience" analytics from NWDAF and provides these information to AMF.

-	Step 3. If N26 does not apply and the AMF identifies that the "Authorized RFSP index" is different than the "Subscribed RFSP", then the AMF notifies the UDM on the "Authorized RFSP index" and the Validity period received from PCF. If the UDM and HSS are deployed separately then the UDM further notifies the "Authorized RFSP" and the Validity period to the HSS by triggering an Nhss_UEContextManagement service operation as defined in TS 29.563 [xx]. In a combined UDM+HSS deployment this step is not needed or handled in implementation specific way.

The MME continues to use the "Authorized RFSP index" over the "Subscribed RFSP index" for the Validity period. When the Validity period expires, the MME may then send the "Subscribed RFSP" as the RFSP to use to the eNB.

6.5.3	Impacts on services, entities and interfaces

The solution has the following impacts:

PCF:

-	PCF should determine the "authorized RFSP index" together with the Validity period e.g. based on the "Service Experience" analytics from NWDAF.

-	PCF provides the "authorized RFSP index" together with the Validity period to AMF during 5GS to EPS mobility without N26 interface.

AMF:

-	When N26 interface is not supported, AMF provides the "authorized RFSP index" together with the Validity period to UDM during the 5GS to EPS mobility without N26 interface.

UDM:

-	When N26 interface is not supported, UDM receives the "authorized RFSP index" together with the Validity period from AMF during the 5GS to EPS mobility without N26 interface. Optionally, the UDM further provides the "authorized RFSP index" together with the Validity period to HSS when a combined UDM+HSS is not deployed.

MME:

-	When N26 interface is not supported, MME receives the "authorized RFSP index" together with the Validity period from HSS.

-	The MME continues to use the "Authorized RFSP" over the "Subscribed RFSP" for the Validity period. When the Validity period expires, the MME may then send the "Subscribed RFSP" as the RFSP to use to the eNB.

# * * * * Next change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[6]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[7]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[8]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[xx]	3GPP TS 29.563: "Home Subscriber Server (HSS) services for interworking with Unified Data Management (UDM); Stage 3".

# * * * * End of changes * * * *