3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205786

Elbonia, Aug 17th – 26th, 2022	(revision of S2-220xxxx)

Source:	China Mobile, China Southern Power Grid

Title:	Update to solution #19_on the impacts to existing network

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: This contribution updates solution #19 to Key Issue#4 on enhancements of 5G VN group communication.

1. Introduction/Discussion

The solution#19 introduces a new indicator in 5G VN group data which is aiming to inform the SMF whether the procedures used for multiple SMFs is applicable for this 5G VN group. There is one editor’s note as below:

Editor's note:	This clause lists impacts to existing entities and interfaces.

This pCR is intend to address this editor’s note.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.3.0.

# * * * * First change * * * *

## 6.19	Solution #19: Solution of Multiple SMFs for VN group communication

### 6.19.1	Introduction

This solution is to address the Key Issue #4 on enhancements of 5G VN group communication. It introduces a new indicator in 5G VN group data which is aiming to inform the SMF whether the procedures used for multiple SMFs is applicable for this 5G VN group.

For example, if one dedicated SMF is enough to support a certain 5G VN group, then it's not necessary for the SMF to continue the procedures with NRF or GSMF. And also, if the 5G VN group will use N6 based switch, then it's also not necessary for the SMF to continue the procedures with NRF or GSMF. So, this solution can be regard as the trigger point for the KI#4 and can be collaborate with other solutions on the same KI topic.

### 6.19.2	Functional Description

The main idea of this solution is as below:

-	Enhance the parameter provisioning service by introducing the Single-SMF indicator information as a new optional input parameter, and this Single-SMF indicator is stored as part of the group data in group subscription data.

-	SMF can retrieve such indicator from UDM or be notified by UDM via the group subscription data.

-	In case the indicator exist, the SMF can be informed that this 5G VN group does not require multiple SMF to serve and will not continue the other procedures which may lead to additional interaction with other nodes. For example registration or updating its profile to NRF(Solution #4) or Group Session Management Function(Solution #5).

-	In case the indicator doesn't exist, the SMF can continue the procedure which will be addressed by other solutions for KI#4.

Editor's note:	it is FFS whether the existing group data information can serve as the Single-SMF indicator.

### 6.19.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.19.4	Impacts on existing entities and interfaces

NEF/UDM: The parameter provisioning service can support Single-SMF indicator as a new input parameter for management of a 5G VN group.

UDM/UDR: The group subscription data can contain 5G VN group membership information and 5G VN group data including the Single-SMF indicator.

AMF: With the indicator retrieve from UDM or be notified by UDM, the AMF can be notified that to select the same SMF for this 5G VN group.

SMF: With the indicator retrieve from UDM or be notified by UDM, the SMF can be informed that this 5G VN group does not require multiple SMF to serve and will not continue the other procedures which will be defined by KI#4. For example registration or updating its profile to NRF(Solution #4) or Group Session Management Function(Solution #5).

# * * * * End of changes * * * *