3GPP SA WG2 Meeting #152E	S2-2205932

e-meeting, 17-26 August 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Conclusions for KI#3

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper proposes conclusions and way forward for key issue #3 “Support for controlling 5G time synchronization service based on subscription” to the TR 23.700-25.

# Discussion

There are two solutions, Solutions #18 and #19, in the TR that address KI#3. The only aspect of solution #19 that is not covered by solution 18 is the notification to 5GC if the time synchronization status of a UE changes. The notification of time synchronization status of a UE is not in the scope of KI#3 but more relevant to KI#1. Therefore, it is proposed that solution #18 should be used as the basis for normative work.

- In order to provide continuous time synchronization service for targeted UEs, 5GC should be informed if the time synchronization status of those UEs change. The time status changes can be resulted from some unexpected cases, e.g. time sources failure or UE mobility.

Proposal 1: Solution #18 is selected for normative work.

# Proposal

It is proposed to add the following solution to TR 23.700-25.

*** Start of changes ***

## 8.X	Conclusion for KI#3: Support for controlling 5G time synchronization service based on subscription

Controlling 5G time synchronization services based on subscription shall be supported as documented in solution 18.

*** End of changes ***