3GPP TSG-WG SA2 Meeting #152E	S2-2205945

Elbonia, August 17 – 26, 2022

Source:	Orange

Title:	KI #7: Evaluation of solutions

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3/ Rel-18

Abstract: This contribution proposes a categorization of solutions for KI#7, and then focuses on the evaluation of solutions related to Enhancements on QoS Sustainability analytics.

# 1. Discussion

This paper proposes the evaluation and conclusion of the solutions in KI #7

The solutions in KI#7 can be classified into two categories according to the issues that they mainly address for achieving improvements on QoS Sustainability analytics:

- Enhance the QoS Sustainability analytics with a finer granularity area (e.g. below Cell level): Sol #18 (without any specific solution provided), #19, #50

- Enhance the QoS Sustainability analytics with additional input data:  #18, #19, #20, #50

Solutions in group “b” can be then differentiated according to the type of change, either introducing new information elements used for data collection from OAM or adding new sources of information interacting with NWDAF:

- Additional data from OAM: #18, #19, #20

- Additional data from SMF: #19 (with PCF), #20 (optional), #50

- Additional data from UPF: #20 (optional), #50

## 2. Text Proposal

It is proposed the following changes to TR 23.700-81 [x].

# * * * * First change * * * *

# ALL NEW TEXT

## 7.xx	Key Issue #7: Enhancements on QoS Sustainability analytics

The solutions in KI#7 can be classified into two categories according to the issues that they mainly address for achieving improvements on QoS Sustainability analytics:

a)	Enhance the QoS Sustainability analytics with a finer granularity area (e.g. below Cell level): Sol #18 (without any specific solution provided), #19, #50

b)	Enhance the QoS Sustainability analytics with additional input data:  #18, #19, #20, #50

Solutions in group b can be then differentiated according to the type of change, either introducing new information elements used for data collection from OAM or adding new sources of information interacting with NWDAF:

-	Additional data from OAM: #18, #19, #20

-	Additional data from SMF: #19 (with PCF), #20 (optional), #50

-	Additional data from UPF: #20 (optional), #50

The evaluation of the solutions in the category "Enhancements with a finer granularity area" is provided as follows:

-	Sol #18 acknowledges the need for "below cell level location information" and modifies accordingly the output analytics with the "Applicable Area with finer granularity" information but does not provide any details on how to effectively obtain such information.

-	Sol #19 proposes to add interactions with LMF and AMF to get UE positions located in the Cell or TA where requested area or path of Interest is included.

-	Sol #50 is also relying on LMF and AMF as well as GMLC to retrieve the UE location information, e.g. cell ID or finer granularity below cell. This solution describes more in detail the overall procedure involving LCS Service and GLMC.

The evaluation of the solutions in the category "Enhancements with additional input data" is provided as follows:

-	Sol #18 introduces the usage of "Latency and delay of 5G networks" from OAM, described more in details in clause 6.3 of TS 28.554 referring to the Integrity KPI. Among possible specific KPIs, the "Downlink/Uplink latency in gNB-DU", "Integrated Downlink/Uplink delay in RAN" or "E2E delay for network slice" could be obtained for example to enhance the data collection for "QoS Sustainability" analytics.

-	Sol #19 proposes also to add similar information from OAM called "Packet Delay", but referring this time more specifically to TS 28.552, where for instance different performance measurements are described such as the "Average delay UL/DL air-interface", "Round-trip GTP Data Packet Delay and packet delay between NG-RAN and PSA UPF". This solution also introduces new input data from TS 28.552 called "RAN status, load and performance information" referring at least to the "Radio resource utilization" described in chapter 5.1.1.2 and "Packet Loss and/or Drop", certainly referring to the chapter 5.1.3 "Performance measurements valid for split gNB deployment scenario" describing the "Packet Loss Rate" and "Packet Drop Rate" measurements. In addition to OAM source, Sol#19 introduces the usage of "QoS Notification Control of the QoS profiles or Alternative QoS Profiles" and their corresponding "QoS profiles" from SMF and PCF.

-	Sol #20 proposes firstly to add the same "Packet Delay" from OAM already described in TS 28.552 like for Sol#19. In addition, this solution proposes to extend the measurements related to QoS Monitoring defined in TS 23.501 and TS 28.552, to include the IP-layer section capacity and IP-layer available section capacity between UE, NG-RAN and UPF at GTP level. In this way, new data input could be obtained such as "UL/DL capacity GTP between UPF and UE" or "UL/DL available capacity GTP between UPF and NG-RAN" corresponding respectively to the "IP-layer section capacity" and "IP-layer available section capacity" definitions from ITU-T Y.1540. How OAM can perform capacity measurements is to be determined by SA WG5. Optionally, End-to-End measurements from UE to UPF like "UL/DL packet delay GTP" could be also obtained through SMF or UPF.

-	Sol #50 does not rely on OAM but proposes to collect instead QoS information from the UPF directly or through the SMF. Such information may include the "QoS flow Bit Rate" representing the observed bit rate for UL and DL and "QoS flow Packet Delay" representing the observed packet delay for both directions.

# * * * * End of changes * * * *