SA WG2 Meeting #152E (e-meeting)	S2-2205963

August 17 – 26, 2022, Elbonia	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	Conclusion for KI#2

Document for:	Approval

Agenda Item:	9.14 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes conclusion for Key Issue #2.

Discussion

Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change * * * *

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.X	Conclusions for Key Issue #2

It is concluded that the following principles are to be used as basis for normative work:

-	The SoR procedure is enhanced to provide the UE a prioritized list of VPLMN with supported slices for each VPLMN.

-	The UDM can provides Subscribed S-NSSAIs and location of the UE to the SoR AF and the SoR AF take into account Subscribed S-NSSAI and UE location to generate a prioritized list of VPLMN with supported slices.

-	The H-PCF can provide a policy to the UE to control in which condition the UE is allowed to trigger VPLMN selection.

-	How the UE provides its capability will be determined during the normative phase together with CT1.

-	The end-to-end security protection mechanism for URSP rule will be defined by SA3.

* * * * End of Changes * * * *