3GPP SA WG2 Meeting #152E	S2-2206007

Aug 17th – 26th, 2022, Elbonia

Source:	OPPO

Title:	KI#9, New Sol: power mode based QoS adjustment

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: Propose a solution for KI#9.

1. Introduction/Discussion

The Key issue #9 in Rel-18 FS_XRM aims to study how to support trade-off of throughput/latency/reliability and power consumption (e.g. device battery life).

Currently, reporting the device battery life is not supported in RAN, and it may result in privacy data leakage. Instead, this paper proposes to adjust QoS based on the power mode (e.g., power saving mode, ultimate performance mode, etc.). For instance, when a UE is connected to a power supply or has sufficient power, the ultimate performance mode may be applied. When the UE is disconnected from the power supply or the battery is low, the UE can decide whether to switch to power saving mode.

In this solution, the power mode is introduced to PCF for the construction of PCC rule. There are two different approaches to provide power info to PCF. One is sending the power mode info to SMF via NAS signalling, and then SMF can forward such information to PCF. The other is sending the power mode info to the AF over the application layer, and then the AF provides the power mode info to the PCF via NEF.

After UE indicates the power mode to PCF, and the PCF generates the PCC rule with corresponding QoS parameters. For example, if the ultimate performance mode is applied, PCF may generate a PCC rule with high bitrates and low PDB. If the power saving mode is chosen by UE, PCF may generate a PCC rule with low bitrates and high PDB. Besides, UE can decide to switch the power mode to a new one at any time to trigger the QoS update.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change (all new text) * * * *

6.X	Solution #X: power mode based QoS adjustment

6.X.1	Key Issue mapping

This solution addresses Key Issue #9: trade-off of QoE and power saving requirements.

6.X.2	Description

The intent of this solution is to propose a mechanism to assist PCF to adjust the PCC rule based on the power mode currently selected by the user, and thus the trade-off between throughput/latency/reliability and power consumption can be achieved.

It is assumed that the PCF can generate different PCC rules corresponding to distinct power modes. For example, a UE is connected to a power supply or has sufficient power, the ultimate performance mode may be applied. The UE informs PCF of the ultimate performance mode, and then PCF generates corresponding QoS parameters (e.g., high bitrates, low PDB), so that the user experience can be improved. On the contrary, when the UE is disconnected from the power supply or the battery is low, the power saving mode may be selected by the UE, and then the PCF generates corresponding QoS parameters (e.g., low bitrates and high PDB), so that the power consumption will be reduced and the available time of UE is extended.

There are two alternatives to provide the power mode info to PCF:

- UE reuses the existing PDU session establishment/modification procedure to send the power mode info to SMF, and then SMF can forward such information to PCF by initiating the SM establishment/modification procedure.

- UE sends the power mode info to the AF over the application layer, and then the AF provides the power mode info to the PCF via NEF. The power mode info can be carried by the setting up an AF session with required QoS procedure.

Either triggered by AF or by UE, PCF generates a PCC rule based on the received power mode info. Upon receiving the PCC rule, SMF performs the binding of PCC rules to QoS flows, and provides QoS profile to RAN and QoS rule to UE. Furthermore, PCF can also update the PCC rule with new QoS parameters, if the power mode is switched to a new one. Consequently, the throughput/latency/reliability and power consumption can be adjusted correspondingly.

6.X.3	Procedure

Figure 6.X.2-1: Procedure for Power Mode Based QoS adjustment

The procedure for power mode based QoS adjustment is described in Figure 6.X.2-1. According to whether the UE has sufficient power and user preference, UE can decide which power mode (e.g., power saving mode, ultimate performance mode, etc.) should be applied, and then provide the power mode info to the PCF for QoS adjustment.

- UE can directly provide the power mode info to PCF for the QoS adjustment. The power mode info may be carried by PDU session establishment/modification procedure and SM policy association establishment/modification procedure.

- UE can also inform AF of the power mode info over application layer, and then the AF sends the power mode info to PCF via NEF for the QoS adjustment.

- Upon receiving the request message, the PCF makes a policy decision. Considering the received power mode info, PCF can generate a PCC rule with corresponding QoS parameters.

- PCF sends the PCC rule to SMF by reused the existing SM Policy Association Modification Procedure as described in TS 23.502 clause 4.16.5.2.

- SMF determines the QoS profile and QoS rule based on the PCC rule, and then initiates the PDU session modification procedure to deliver the QoS profile and QoS rule to AN and UE respectively.

6.X.4	Impacts on services, entities and interfaces

The solution has impacts in the following entities:

PCF:

-	Needs to generate the PCC rule based on power mode info.

UE:

-	Needs to send the power mode info to PCF.

AF:

-	Needs to send the power mode info to PCF.

# * * * * End of changes * * * *