SA WG2 Meeting #152E (e-meeting)	S2-2206020

August 17 – 26, 2022, Elbonia	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	KI#3, Conclusion

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract of the contribution: This paper provides conclusion for KI#3 "Support direct communication path switching between PC5 and Uu (i.e. non-relay case)".

Discussion

Regarding Key Issue #3: Support direct communication path switching between PC5 and Uu (i.e. non-relay case) in clause 5.3 of TR 23.700-33v0.3.0, there are 7 solutions.

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 16 |  |  | X |  |  | X |  |
| 17 |  |  | X |  |  |  |  |
| 18 |  |  | X |  |  |  |  |
| 19 |  |  | X |  |  |  |  |
| 20 |  |  | X |  |  |  |  |
| 21 |  |  | X |  |  |  |  |
| 22 |  |  | X |  |  |  |  |

This paper provides conclusion for KI#3 based on these solutions.

Proposal

It is proposed to agree the following changes into TR 23.700-33v0.3.0.

* * * * Start of 1st Change * * * *

!!! All New Texts !!!

8.x	Key Issue #3: Support direct communication path switching between PC5 and Uu (i.e. non-relay case)

For Key Issue #3, the following aspects are concluded:

-	Path switching policy is provisioned to the UE as Policy/Parameter for 5G ProSe Direct Communication:

-	The mapping of ProSe services (i.e. ProSe identifiers) to path switching permission (i.e. PC5 permitted, Uu permitted, or both PC5 and Uu permitted)

NOTE:	The path switching policy can be a one mapping for all ProSe services, i.e. same path preference for all ProSe services.

-	Path switching is basically performed between direct PC5 and direct Uu communication path in the following manner.

-	When path switching is decided, the UEs prepare the target path to communicate each other if the target path is not yet ready, e.g., PC5 unicast link establishment if the target path is PC5, PDU Session establishment if the target path is Uu;

-	The UEs start the communication each other over the target path; and

-	The UEs may release the source path if deemed unnecessary, e.g., PC5 unicast link release if the source path is PC5, PDU Session release if the source path is Uu.

-	Switching back to PC5 path from Uu path as proposed in Solution#20 can be optionally supported by UE.

* * * * End of Changes * * * *