SA WG2 Meeting #152e	S2-2206102

# August 17th – 26th, 2022; Elbonia               	  			  		(revision of S2-220)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR |  | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Qos Monitoring also in the Non 3GPP access (N3IWF/TNGF) case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Broadcom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_N3ANQOSM |  |  |  |  |  | Date: |  |  | 2022-07-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Since 3GPP R16, Per QoS Flow per UE QoS Monitoring (TS 23.501 clause 5.33.3.2) is defined only in NG RAN case and not in N3IWF/TNGF case |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding support of Per QoS Flow per UE QoS Monitoring (TS 23.501 clause 5.33.3.2) in N3IWF/TNGF case means - SMF is able to send a QoS Monitoring request to a N3IWF/TNGF via N2 signalling (PDU Session resource create/modify) as in the NG RAN case - The N3IWF/TNGF based on this request exchange IKE signaling with the UE to measure the delay with the UE - The N3IWF/TNGF reports the measured delay using the same mechanisms as defined for NG RAN case in TS 23.501 clause 5.33.3.2 - 5GC reports measurements done over N3GPP access to the AF using the same mechanisms as defined for NG RAN case in TS 23.501 clause 5.33.3.2 - (optionally) The AF may then use Application guidance for URSP determination defined in clause 4.15.6.10 to update Access Type preference for the PDU Session carrying traffic whose monitored delay does not meet applicative requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The following is not possible: - statistics / SLA monitoring / trouble shooting for sensitive applications / users over Untrusted/Trusted non 3GPP access to 5GC - adapting the application behaviour based on QoS monitoring results for applications over Untrusted/Trusted non 3GPP access to 5GC - AF guiding the UE (via URSP) to use another access when the current access does not meet QoS requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.12.X(new); 4.12a.X (new); 4.15.6.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS 23.501 CR FFS |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (all text is new)

### 4.12.X	QoS monitoring

This procedure describes how QoS monitoring as defined in TS 23.501 [2] clause 5.33 is supported for Untrusted non-3GPP access.

The flow assumes the UE has indicated support of QoS monitoring over Untrusted non-3GPP access via NAS SM signaling at PDU Session establishment.

Figure 4.12.X-1: Support of QoS monitoring for Untrusted non-3GPP access

- QoS monitoring is requested as defined for 3GPP access. The SMF configures the UPF as for 3GPP access

- The SMF issues a QoS Monitoring request via N2 signaling (PDU Session Resource Setup or Modify Request). This is as defined in TS 23.501 [2] clause 5.33.3.2.

- Based on the SMF request, the N3IWF starts exchanging IKE signalling with the UE to measure the delay over Nwu

- The N3IWF report the delay measured over Nwu via GTP-u as defined in TS 23.501 [2] clause 5.33.3.2

- The UPF reports the QoS monitoring results as defined for 3GPP access

As part of this reporting, 5GC i.e. the UPF (via Nupf_EventExposure) when direct reporting applies or the PCF otherwise (via Npcf_PolicyAuthorization_Notify) reports QoS monitoring measurements done over N3GPP access to the AF .

The AF may then use Application guidance for URSP determination defined in clause 4.15.6.10 to update Access Type preference for the PDU Session carrying traffic whose monitored delay does not meet applicative requirements

Steps 3 to 5 may be repeated depending on the QoS monitoring parameters provided by the SMF

NEXT CHANGE all text is new

### 4.12a.X	QoS monitoring

This procedure describes how QoS monitoring as defined in TS 23.501 [2] clause 5.33 is supported for Trusted non-3GPP access.

The flow is identical to the flow defined in clause 4.12.X with following differences:

- The N3IWF is replaced by the TNGF

NEXT CHANGE

#### 4.15.6.10	Application guidance for URSP determination

This clause describes the procedures to allow an AF to provide guidance for URSP determination to 5G system via NEF. The AF may belong to the operator or to an external party. The PCF considered in this clause is in the Home PLMN as it is the PCF that determines the URSP for the UE.

NOTE 1:	The operator can negotiate with external party (typically a Corporate represented by an AF) dedicated DNN(s) and/or S-NSSAI(s) for the traffic of UE(s) of this external party. UE(s) of the external party can be identified by a group identifier.

The guidance for URSP determination may be used to provide 5GC with guidance for the URSPs depending on the UE location. This is further described in TS 23.548 [74].

For providing guidance for URSP determination, the procedure defined in clause 4.15.6.7 is performed with the following considerations:

1)	Service Description indicates an AF Identifier.

2)	Service Parameters.

Information on the AF guidance for URSP determination which consists of a list of rules that associate an application traffic descriptor with requested features for the candidate PDU sessions the application traffic may use:

-	An application traffic descriptor, whose definition corresponds to that of the URSP Traffic Descriptors (as defined for the URSP rule in TS 23.503 [4] Table 6.6.2.1-2).

-	one or more sets of Route selection parameters, each parameter may correspond to:

-	(DNN, S-NSSAI). This may be provided by the AF or determined by the NEF based on the AF Identifier when it is not provided by the AF and the AF provides only one instance of AF guidance for URSP determination

Editor's note:	It is FFS whether the AF can provide SSC mode.

-	a default Route selection precedence value to be used for the application traffic when Route selection precedence with a corresponding spatial validity condition is not provided.

-	Route selection precedence with a corresponding spatial validity condition that indicates where the Route selection parameters apply. This may correspond to a geographical area (e.g. a civic address or shapes).

NOTE 2:	The different sets of Route selection parameters indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic descriptor. Each set is meant to apply for a specific (set of) spatial validity condition. Each set is associated with a Route selection precedence to cope with the case where multiple spatial validity conditions overlap.

-	Access Type preference

NOTE 2.5:	the application is responsible to avoid ‘ping-pong” of a PDU Session between 3GPP and Non-3GPP access that would be due the AF keeping on changing the Access Type preference within URSP rules.

If the AF provides a geographical area as spatial validity condition, it is up to the NEF to transform this information into 3GPP identifiers (e.g. TAI(s)).

NEF may, based on local configuration, complement missing service parameters. Additionally, based on operator's local policy, NEF may request for service specific authorization for the service parameters for an individual UE (e.g. to authorize the Corporate or MTC provider represented by the AF and the requested DNN, S-NSSAI for the related UE) before storing the service parameters into the UDR via Nnef_ServiceParameter_Create operation. If the request is targeting a group of UEs, NEF may also request for service specific authorization for the group related data (see table 4.15.6.3b-1), i.e. the DNN, S-NSSAI associated to the group. If the request is targeting any UE (all UEs), NEF authorizes the request based on local policy (e.g. based on AF Id) without requesting for any service specific authorization from UDM.

If a group of UEs or any UE is requested, each individual UE authorization is performed at a later stage by PCF.

NOTE 3:	The operator needs to ensure the consistency between the group related data and the UE group members subscription data, i.e. if a group is authorized for a given DNN/S-NSSAI as defined in the group related data, it needs to be ensured that all UE members of the group are provisioned with such DNN/S-NSSAI, since no individual UE check is required to be done by NEF against UDM.

NOTE 4:	AF guidance for application traffic is not related with 5G VN group.

3)	a specific UE, or a group of UE(s) or any UE that the AF request may be associated with.

4)	Subscription to events.

The AF may subscribe to notifications about the outcome of the UE Policies delivery due to application guidance for URSP determination.

The usage of the AF guidance for application traffic is described in clause 6.2.4 in TS 23.548 [74].

Figure 4.15.6.10-1 shows the enhanced procedure as defined in clause 4.15.6.7.

Figure 4.15.6.10-1: Service Specific Authorization for an individual UE or group of UEs

1.	The AF initiates the procedure as specified in clause 4.15.6.7.

2.	The NEF sends Nudm_ServiceSpecificAuthorisation_Create Request including S-NSSAI/DNN and service type received from AF.

3.	If the request is for an individual UE, the UDM checks the list of subscribed/allowed S-NSSAI/DNNs for the UE and other service info (e.g. MTC provider is authorized for the UE).

If the request is for a group of UEs, the UDM checks whether the group related data (e.g. DNN/S-NSSAI group related data, see table 4.15.6.3b-1) and other service info, e.g. MTC provider is authorized for the group.

4.	The UDM responds to the NEF with the service authorization result. If authorization fails (e.g. DNN is not subscribed for the UE or it is different from the group related data, UE subscription or group related data does not allow to modify URSP rules dynamically by an AF or by such specific AF or MTC provider), UDM returns a negative response with an appropriate error code and the NEF rejects the request with the proper error code to inform the AF about the request not authorized.

NOTE 5:	The MTC Provider Information can be used by any type of Service Providers (MTC or non-MTC) or Corporate or External Parties for, e.g. to distinguish their different customers.

5.	The procedure continues as specified in clause 4.15.6.7.

Figure 4.15.6.10-2 illustrates the procedure for updating or revoking an existing Service Specific Authorization.

Figure 4.15.6.10-2: Service Specific Authorization Update procedure

1.	The UDM may send a Service Specific Authorization Update information using Nudm_ServiceSpecificAuthorisation_UpdateNotify Request (GPSI, External Group Id, S-NSSAI, DNN, Result) message to the NEF to update a UE's or group of UEs' authorization.

2.	The NEF sends Nudm_ServiceSpecificAuthorisation_UpdateNotify Response (cause) message to the UDM to acknowledge the authorization update.

3.	If the authorization is revoked, the NEF removes the service specific parameters from the UDR.

4.	The NEF informs the AF that the service parameters authorisation status has changed by sending Nnef_ServiceParameter_Notify Request (GPSI, External Group Id, TLTRI, Result) message to the AF.

5.	The AF responds to the NEF with Nnef_ServiceParameter_Notify Response message.

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES