3GPP TSG-SA WG2 Meeting #152-e	S2-2206147

# E-Meeting, 17th – 26th August 2022

# Title:	Reply LS on NSSRG restriction on pending NSSAI

# Release:	Rel-17

# Work Item:	eNS_Ph2

Source:	SA2

To:	CT1

Cc:	-

Contact Person:

Name:	Alessio Casati

E-mail Address:	alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-xxxxxxx

1. Overall Description:

SA2 has been discussing applicability of NSSRG restriction on pending NSSAI case identified in the incoming LS from CT1 in C1-224073. SA2 has agreed that the answer to following question:

Question: Should the UE take the S-NSSAIs in the Pending NSSAI into account when applying NSSRG restrictions in the Requested NSSAI?

is:

the UE as per current NSSRG handling only checks compatibility with the S-NSSAIs in the Allowed NSSAI. if NSSAA is ongoing for a S-NSSAI that is incompatible with the S-NSSAIs in the Allowed NSSAI, and the NSSAA completes successfully, the AMF updates the Pending NSSAI in the UE by removing this S-NSSAI from the Pending NSSAI in the UE configuration by a UCU procedure.

the Attached CRs implement this agreement.

2. Actions:

To SA2

ACTION:	SA2 kindly request CT1 to take this decision into account.

3. Date of Next SA2 Meetings:

FUTURE SA2 MEETINGS CALENDAR