3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206162

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI #3, Update solution #9 for PIN management and PINE management

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: This pCR proposed to address the issue to manage PIN and PINEs

1. Introduction/Discussion

This pCR would like to update solution #9 with comparing PEGC and PEMC’s PLMN ID to determine whether PEGC can join the PIN network in PINE and PIN management procedure in order to avoid roaming situation.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

Solution #9: PIN management and PINE management

6.9.1	Description

This solution is to address the KI#3 “Management of PIN and PIN Elements”, how an authorized UE to create a PIN. And how to add a device (PINE) into a PIN, when a PIN is created and PEMC is available. In this solution, it assumed that UDM is used to create/update/remove PIN profile that includes PIN information and PINEs information, and allocate/manage the PIN ID. And may allocate/manage PINE ID for the PIN.

In this solution both PINE Info and PIN Info are defined as following:

PINE Info:

-	PINE ID, unique ID within a PIN that could be allocated by PEMC locally, or network

-	PINE type, indicating whether it is PEMC, or PEGC, normal PINE

-	PINE name, human readable information, e.g., printer in bedroom, or lamp in living room.

-	Service, what the PINE can provide, e.g., printer, or lamp, or camera, etc.

-	Valid time, how long for the PINE to be as member in the PIN

-	Capability, indicating the communication capability, e.g., WiFi, BT, etc.

PIN Info:

-	PIN ID, unique ID within a PLMN and allocated by network,

-	PIN name, is readable information for user

-	PIN Services, the list of the services that the PINE(s) can provide, e.g., printer, lamp, camera, etc.

-	PIN valid time, to define how long the PIN can work, e.g., 30 minutes

-	List of PINE Info, including all the information of PINEs in this PIN

Editor's note: it is FFS how to authorize the UE to create PIN.

6.9.2	Procedures

6.9.2.1	An authorized UE to create a PIN

Figure 6.9.2-1: authorized UE creates a PIN

Here the authorized UE is allowed by the PIN owner to initiate to create a PIN. When an authorized UE triggers a request to network for creating a PIN, it may bring the candidates PIN Info, e.g., the PIN name, etc., optionally candidates PINE Info, which some of the PIN/PINE Info may be designed by user before creating the PIN, e.g., PINE name, capability, etc.

1.	UE sends the request to AMF via gNB for creating a PIN, which including candidate PIN name, candidate PIN Service(s), candidate PIN valid time, optional if available, a list of candidates PINE information containing (candidates PINE ID, candidate PINE name, candidate PINE type, candidate PINE service, candidate PINE capability, candidate PINE valid time).

When PINE ID is allocated by network, the candidates PINE ID is not included.

NOTE:	Create PIN request message can be carried by Registration Request.

Editor’s note: Whether Service Request or others possible is FFS.

2.	AMF requests to UDM for creating the PIN, with information carried in step 1. AMF may check with UDM whether the UE is authorized to create PIN. If no, the AMF rejects the UE with proper cause.

3.	AMF sends the Create PIN response to UE/PEMC if the request is rejected by AMF..

4.	UDM allocates the PIN ID based on UE request, and may allocates PINE ID for each candidate PINE if PINE info included in the request, and create PIN profile in the UE’s subscription data, including e.g.,

PIN Info:

-	PIN ID

-	PIN name, e.g., Jian’s smart home

-	PIN Services, e.g., printer service, lamp service, camera service.

-	PIN valid time, e.g., 1 year

-	List of PINE Info, including all the information of PINEs in this PIN

PINE1 Info:

-	PINE ID, e.g., J001

-	PINE type, e.g., PINE

-	PINE name, e.g., printer in bedroom.

-	Service, e.g., printer service.

-	Valid time, e.g., 10:00-18:00 each day

-	Capability, e.g., WiFi, BT.

PINE 2 Info……

PINE 3 Info……

5, UDM response to AMF with result of creating PIN, including the results, full PIN Info{PIN ID, PIN name, PIN Service, PIN valid time, optional list of PINE Info}

6, AMF sends the service accept to UE via gNB, including results, and full PIN Info.

6.9.2.2	A device joins a PIN

Figure 6.9.2-2: a device to join a PIN

In this solution, it assumed that PEMC broadcasts PIN network information, e.g., PIN ID, PIN name, etc. so that the device discovers and selects the target PEMC, and connection between device and PEMC is established via non-3GPP RATs, e.g., WiFi, BT, etc.

1.	A device sends the Join PIN Request to PEMC via the established connection between the device and PEMC, including PIN ID, Device ID, PINE type (optional), capability, Service, name, etc. If PINE type is PEGC, the Join PIN Request should also contain PEGC’s PLMN ID.

2.	PEMC determines to accept the request. How to determine to accept the request is up to implementation. PEMC allocates the PINE ID for the device. And PEMC determines the PINE type, e.g., act as PEMC, or PEGC, or PINE. If the PINE type is PEGC, the PEMC shall determine whether to accept the request based on the PLMN ID of both PEGC and PEMC. If the PLMN ID of PEGC is the same as the one of PEMC, the request is accepted. Otherwise, the request should be denied.

3.	PEMC sends the Join PIN response to the device, including result (accept, or reject), PIN Info and PINE Info as defined in 6.9.1.

PIN info: PIN ID, PIN name, PIN Service(s), PIN valid time, list of PINE info;

PINE info: PINE ID, PINE name, PINE type, capability, Service(s) and valid time.

4.	PEMC sends the PIN update request to AMF via gNB, including UE ID, PIN Info container [PIN ID, new PINE info {PINE ID, PINE type, PINE name, capability, service, valid time}]

NOTE 1:	The PIN update request can be carried by Registration Request.

Editor’s note; Whether Service Request or others possible is FFS.

5.	AMF sends the Nudm_UECM_Registration Request to UDM for updating the UE’s PIN Info, including message received in step 4.

Before updating the PIN Info, UDM may determine whether to accept the updating, e.g., accept new PINE.

6.	UDM updates the received PIN info to the UE’s subscription data.

When a PIN is created, the PIN related information is established as well in the requested UE’s subscription data.

7.	UDM sends the response to AMF, including the results, and updated PIN Info.

8.	AMF sends the PIN update response to PEMC with result (accept, or reject), and updated PIN Info.

9.	For another option instead of step 3, PEMC sends the join PIN Response to device with result, and PIN/PINE info same as in step 3 after PEMC updating the PIN info to UDM.

NOTE 2:	In step 1, 3 and 9, both application layer and new adapted layer are possible options for delivering the messages.

6.9.3	Impacts on Existing Nodes and Functionality

Network impacts:

-	UDM allocates PIN ID for each create PIN request

-	UDM allocates PINE ID for each requested PINE

-	UDM manages the PIN info and PINE Info

-	AMF requests management of PIN/PINE to UDM based on the PIN related request

-	UE(PEMC) determines whether PEGC can join PIN based on PLMN ID

# * * * * End of changes * * * *