# 3GPP TSG SA WG2 Meeting #152E	S2-2206255

# Elbonia, Aug 17-26, 2022                                                                   (revision of S2-220xxxx)

Source:	Nokia, Nokia Shanghai Bell, Verizon UK Ltd

Title:	New WID: TEI18_CHFSel; CHF Selection information in Subscription Data

Document for:	Approval

Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	CHF Selection information in Subscription Data

## Acronym: TEI18_CHFSel

## Unique identifier:

Potential target Release:	Rel-18

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | x |  |
| No | x | x | x |  | x |
| Don’t know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A | N/A |  |

Dependency on non-3GPP (draft) specification:

## 3	Justification

The CHF discovery and selection procedure used by NF consumers like the SMF, the AMF, the SMSF and the PCF is described in clause 6.3.11 of TS 23.501. For example: the PCF may discover the address(es) of the CHF (Primary and Secondary) based on either PDU Session policy control subscription information in UDR or locally configured data or by discovering using NRF. Similarly an SMF may discover a CHF based on either PCF provided information or charging charateristics recieved from the SM subscription data in UDM or using NRF discovery or based on local configuration.

Currently SUPI is used for NRF based discovery of CHF. The CHF discovery mechanism does not support discovery of a CHF by the NF consumers like the SMF, the AMF, the SMSF and the PCF based on information about network segments that the CHF belongs to, in network deployments with geographical (regional) or vertical (e.g. consumer vs enterprise) segmentation. For example:

-	In a network deployment it may be possible that group(s) of CHF(s) is deployed for supporting charging of enterprise customers and another group is deployed for non-enterprise consumers. In this case SUPI based lookup at NRF will add to the latency as the NF consumer has to perform NRF discovery query for every subscriber.

-	It may be also possible to deploy group(s) of CHF(s) to support charging for customers belonging to specific regions (for example users subscribed to some regional or provincial branch of the operator).

The SUPI based CHF discovery through NRF is not efficient in such network deployments and a Group ID based CHF discovery is needed. Although in 3GPP Rel-17 the CHF can register with the NRF using CHF-Group-ID (along other parameters such as CHF-Hostname, IP Address etc.) and an NF consumers can use CHF-Group-ID during CHF discovery process through the NRF, the current specification is not sufficient to support the above use cases because:

-	The NF consumers like the SMF, the AMF, the SMSF and the PCF does not have CHF-Group-ID information associated with UE.

-	Group ID Mapping stored in UDR is not accessible to all NF consumers (e.g. AMF, SMF etc.). This also requires the UDR to store/maintain the mapping of all the subscribers in the network to the CHF-Group-ID

The AM and SM subscription data in the UDM includes a "Charging Characteristics" information that may also contain CHF address(es) selection information; however, the encoding is not standardized and is freely defined by Operators.

Additional CHF selection data providing information on CHF segmentation or grouping in the deployment will benefit the NF consumers by allowing them to perform group-based CHF discovery instead of CHF discovery per SUPI. One enhancement that is required is to extend the subscription data (e.g. policy control subscription information in UDR or UE's subscription data in the UDM) to include the CHF-Group-ID information in order to enable the NF consumers like the SMF, the AMF, the SMSF and the PCF to perform CHF discovery and selection using this CHF-Group-ID. The subscription data extension in UDM/UDR should support to further extend the CHF selection information with additional data that may be required in future.

## 4	Objective

The objective of this work item is to enhance UE's subscription data in UDR and UDM to include a CHF discovery and selection information section (in this release containing only the CHF-Group-ID).

This work item will require 0.5 TU(s) to discuss and agree on the corresponding CRs.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Updates to 6.3.11 for CHF discovery and selection | TSG#99 (Mar 2023) |  |
| TS 23.502 | Update clause 5.2.3.3.1 and Table 5.2.3.3.1-1 to include CHF discovery and selection information containing CHF-Group-ID. | TSG#99 (Mar 2023) |  |
| TS 23.503 | Updates to specify how the PCF gets CHF discovery and selection information. | TSG#99 (Mar 2023) |  |

## 6	Work item Rapporteur(s)

Pallab Gupta, Nokia, pallab.gupta@nokia.com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

SA5 may have to update:

-	Clause 5.1.8 of TS 32.255 for CHF selection by the SMF

-	Clause 5.1.3 of TS 32.256 for CHF selection by the AMF

-	Clause 5.4 of TS 32.274 for CHF selection by the SMSF

## 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

Verizon UK Ltd

Oracle