SA WG2 Meeting #SA2-152E 	S2-2206292

17 – 26 August, 2022 Electronic meeting

Source:	China Mobile

Title:	Conclusion for KI#1 HR PDU session supporting access to V-EHE

Document for:	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_EDGE_Ph2

Abstract of the contribution: This document proposes to conclude KI#1

- Discussion

- Proposal

It is proposed to capture the following aspects in TR 23.700-48.

# * * * * Start of change * * * *

8	Conclusions

8.1	Conclusion for KI#1

The conclusion for UE accessing V-EHE via a Home Routed (HR) PDU Session (i.e. with PSA in HPLMN):

1. H-SMF decides whether the HR PDU session can support the local breakout in VPLMN.

2. If the H-AF has the knowledge of V-AF(e.g. V-DNS server address)based on Operator’s agreement,

-	H-PCF can create PCC rules for V-SMF to support the local breakout, and

-	H-UDM configures the ECS address to VPLMN.

3. If H-AF has no knowledge of V-AF,

-	V-SMF requests the authorization from H-SMF to support the local breakout

-	V-SMF requests the PCC rules for local breakout traffic from V-PCF.

-	V-SMF selects the V-EASDF.

-	V-SMF sends the V-EASDF/Local DNS address to H-SMF to create the PCO information.

# * * * *end of change * * * *