3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206318

Elbonia, August 17th – 26th, 2022	(revision of S2-2206318)

Source:	Huawei, HiSilicon, China Mobile

Title:	Service Operations used in IMS Data Channel Architecture

Document for:	Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract: This paper gives service operations used in IMS data channel architecture.

1. Introduction/Discussion

Based the IMS data channel architecture described in pCR S2-2205772, this paper lists service operations used in IMS data channel architecture.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-87.

# * * * * First change * * * *

6.X	Solution #Y: Service operations used in Data Channel architecture

6.X.1	Description

6.X.1.1	General

All the services described below will be registered to the NRF and discovered and selected via the NRF.

6.X.1.2	DCSF Services

6.X.1.2.1	General

Figure 6.X.1.2.1-1 and Figure 6.X.1.2.1-2 show the reference architecture for the DCSF services in SBI representation style and reference point representation style respectively.

Figure 6.3.1.3.1-1: Reference Architecture for the DCSF Services; SBI representation

Figure 6.3.1.3.1-2: Reference Architecture for the DCSF Services; reference point representation

NOTE:	The DC3 interface is out of the scope of 3GPP Release 18 and will be discussed in the later release.

The following table illustrates the DCSF Services and Service Operations.

Table 6.3.1.3.3-1: NF services provided by the DCSF

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_MediaEventReport (MER) | Create | Request/Response | IMS AS |
|  | Update | Request/Response | IMS AS |
|  | Delete | Request/Response | IMS AS |

6.X.1.2.2	Ndcsf_MediaEventReport (MER) service

6.X.1.2.2.1	General

Service description: This service enables the IMS AS to report IMS Data Channel related media events with session status in a SIP session to the DCSF.

6.X.1.2.2.2	Ndcsf_MER_Create service operation

Service operation name: Ndcsf_MER_Create

Description: The IMS AS requests the DCSF to create a session context and reports the initial media event and session status related to this session context.

Inputs, Required: Call-ID of the SIP session, GPSI, media event type, session status.

Inputs, Conditional: If the media event type is Data Channel media update, Data Channel media information including operation type (Add/Update/Remove a data channel stream), dcmap and originator (local or remote) are required.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.1.2.2.3	Ndcsf_MER_Update service operation

Service operation name: Ndcsf_MER_Update

Description: The IMS AS reports to the DCSF the latest media event and session status related to a session context.

Inputs, Required: Call-ID of the SIP session, GPSI, media event type.

Inputs, Conditional: If the media event type is Data Channel media update, Data Channel media information including operation type (Add/Update/Remove a data channel stream), dcmap and originator (local or remote) are required. If the session status changes, the latest session status are required. If the media event type is allocated Data Channel resource, Data Channel resource URL allocated by the DCMF and IP address and port of the DCMF are required.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.1.2.2.4	Ndcsf_MER_Delete service operation

Service operation name: Ndcsf_MER_Delete

Description: The IMS AS reports to the DCSF the last media event and session status related to a session context, and requests the DCSF to delete the session context.

Inputs, Required: Call-ID of the SIP session, GPSI, media event type.

Inputs, Conditional: If the media event type is Data Channel media update, Data Channel media information including operation type (Add/Update/Remove a data channel stream), dcmap and originator (local or remote) are required. If the session status changes, the latest session status is required.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.1.3	DCMF Services

6.X.1.3.1	General

Figure 6.X.1.3.1-1 and Figure 6.X.1.3.1-2 show the reference architecture for the DCMF services in SBI representation style and reference point representation style respectively.

Figure 6.3.1.3.2-1: Reference Architecture for the DCMF Services; SBI representation

Figure 6.3.1.3.2-2: Reference Architecture for the DCMF Services; reference point representation

The following table illustrates the DCMF Services and Service Operations.

Table 6.3.1.3.2-1: NF services provided by the DCMF

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| NDCMF_MediaResourceManagement (MRM) | Create | Request/Response | IMS AS |
|  | Update | Request/Response | IMS AS |
|  | Delete | Request/Response | IMS AS |

6.X.1.3.2	Ndcmf_MediaResourceManagement (MRM) service

6.X.1.3.2.1	General

Service description: This service enables the IMS AS to create, update and delete IMS Data Channel related resources required by some specified data channel.

6.X.1.3.2.2	Ndcmf_MRM_Create service operation

Service operation name: Ndcmf_MRM_Create

Description: The IMS AS requests the DCMF to allocate IMS Data Channel related resource required by some specified data channel.

Inputs, Required: Data Channel media description (stream ID, direction, max_retry, max_time, priority), App resource URL allocated by the DCSF, IP address and port of the DCSF.

Inputs, Conditional: If it’s the data channel between the local UE and the DCMF, the remote end point address (IP address and port) is required.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication, Allocated Data Channel resource URL, local end point address (IP address and port of the DCMF).

Outputs, Optional: None.

6.X.1.3.2.3	Ndcmf_MRM_Update service operation

Service operation name: Ndcmf_MRM_Update

Description: The IMS AS requests the DCMF to update IMS data channel related resource.

Inputs, Required: Allocated Data Channel resource URL, local end point address (IP address and port of the DCMF), Data channel media information (stream ID, direction, max_retry, max_time, and priority), the remote end point address.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.1.3.2.3	Ndcmf_MRM_Delete service operation

Service operation name: Ndcmf_MRM_Delete

Description: The IMS AS requests the DCMF to delete IMS data channel related resource.

Inputs, Required: Allocated Data Channel resource URL, local end point address (IP address and port of the DCMF).

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.1.4	IMS AS Services

6.X.1.4.1	General

Figure 6.X.1.4.1-1 and Figure 6.X.1.4.1-2 show the reference architecture for the IMS AS services in SBI representation style and reference point representation style respectively.

Figure 6.3.1.3.3-1: Reference Architecture for the IMS AS Services; SBI representation

Figure 6.3.1.3.3-2: Reference Architecture for the IMS AS Services; reference point representation

The following table illustrates the IMS AS Services and Service Operations.

Table 6.3.1.3.3-1: NF services provided by the IMS AS

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_DataChannelControl (DCC) | Open | Request/Response | DCSF |
|  | Close | Request/Response | DCSF |
|  | Refuse | Request/Response | DCSF |
|  | Continue | Request/Response | DCSF |
|  | Anchor | Request/Response | DCSF |

6.X.1.4.2	Nimsas_DataChannelControl (DCC) service

6.X.1.4.2.1	General

Service description: This service enables the DCSF to indicate the IMS AS to execute operations on data channel streams.

6.X.1.4.2.2	Nimsas_DCC_Open service operation

Service operation name: Nimsas_DCC_Open

Description: The DCSF requests the IMS AS to open some specified data channel(s) between the local/remote UE and the local network in a SIP session.

Inputs, Required: Call-ID of the SIP session, control list of Data Channel streams in which each item includes Data Channel stream ID and data channel type (application data channel or bootstrap data channel), App resource URL allocated by the DCSF, IP address and port of the DCSF, and role (the calling party or the called party).

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.1.4.2.3	Nimsas_DCC_Close service operation

Service operation name: Nimsas_DCC_Close

Description: The DSCF requests the IMS AS to close some specified data channel(s) in a SIP session.

Inputs, Required: Call-ID of the SIP session, control list of Data Channel streams in which each item includes Data Channel stream ID and data channel type (application data channel or bootstrap data channel).

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.1.4.2.4	Nimsas_DCC_Refuse service operation

Service operation name: Nimsas_DCC_Refuse

Description: The DCSF requests the IMS AS to refuse the establishment of some specified data channel(s) in a SIP session.

Inputs, Required: Call-ID of the SIP session, control list of Data Channel streams in which each item includes Data Channel stream ID and data channel type (application data channel or bootstrap data channel).

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.1.4.2.5	Nimsas_DCC_Continue service operation

Service operation name: Nimsas_DCC_Continue

Description: The DCSF requests the IMS AS to forward some specified data channel(s) between UEs or between the local UE and the remote network in a SIP session.

Inputs, Required: Call-ID of the SIP session, control list of Data Channel streams in which each item includes Data Channel stream ID and data channel type (application data channel or bootstrap data channel).

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

6.X.1.4.2.6	Nimsas_DCC_Anchor service operation

Service operation name: Nimsas_DCC_Anchor

Description: The DCSF requests the IMS AS to anchor some specified data channel(s) between UEs or between the local UE and the remote network in a SIP session.

Inputs, Required: Call-ID of the SIP session, control list of Data Channel streams in which each item includes Data Channel stream ID and data channel type (application data channel or bootstrap data channel).

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *