3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206376

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation for KI#4

Document for:	Approval

Agenda Item:	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: This paper proposes the evaluation for KI#4.

1. Introduction/Discussion

This KI#4 focuses on the evaluation of Operation Control for Ranging/Sidelink positioning.

The following table summarises the overall of solutions on KI#4 in this TR.

| Solution | Highlights |
| --- | --- |
| #3 | Ranging control signaling via PC5-S, and PC5 connection is needed. Interaction between Ranging layer and AS layer is described. |
| #4 | It proposes RSPP like LPP via PC5-S transmission. |
| #5 | High-level procedure, and leave the detailed Sidelink Positioning procedure to RAN. |
| #14 | A general reference architecture |
| #19 | Ranging control signaling via PC5-U, and transmitted over LTE PC5 and NR PC5 |
| #26 | Location Server functionality is discussed, and Location Server UE is newly involved |
| #27 | LMF selection enhancement |
| #28 | On ranging result calculation modes |

The solutions can be summarized and evaluated in detail as the following,

-	Sol#3 proposes that the coordination and configuration information can be exchanged and the ranging results can be shared between the UEs via the PC5-S. The ranging request or response can be a new PC5-S signalling carried by the PC5 connection. The coordination and configuration information can be the Ranging role (Reference UE or Target UE), one time or period ranging, ranging for distance or direction measurement or both. The interaction between upper layer (e.g., ranging layer) and lower layer (e.g., AS layer) within one UE is also described. The ranging procedure in AS layer according to the ranging configuration is left to RAN WG.

-	Sol#4 proposes to use RSPP (Ranging/Sidelink Positioning Protocol) like LPP on top of existing PC5-S protocol. RSPP message is used to exchange the coordination & configuration information (e.g., Ranging capability, Ranging assistant data, Ranging measurement results), and a new kind of PC5-S message is used to carry the RSPP message. Target UE and the Reference UE pass the interacted coordination & configuration information down to the AS layer, and then the Ranging/Sidelink Positioning measurement and result calculation can be performed. It also proposed that RSPP will be determined by RAN WG.

-	Sol#5 presents a high-level procedure, and leave the detailed Sidelink Positioning procedure to RAN WG. During the Sidelink Positioning procedure, two UEs can perform the capability exchange, reference UE negotiation, Ranging/Sidelink positioning measurement and calculation, Ranging/Sidelink positioning result transmission. Ranging/Sidelink positioning request and response are performed before the Sidelink Positioning procedure, and ranging layer gets the ranging parameters (e.g., required location QoS) from application layer. UE-1 based sidelink positioning and UE-1 assisted sidelink positioning are proposed, and it is related to who triggers the sidelink positioning procedure and calculates the result.

-	Sol#14 proposes a general reference architecture for Sidelink positioning and ranging-based services, and this solution is applied to all KIs. The corresponding architecture is documented into the clause 4.3.

-	Sol#19 proposes that ranging control signaling is transmitted via PC5-U, and transmitted over either LTE PC5 or NR PC5. Since only NR PC5 based ranging is studied in R18 RAN scope, that means that the Ranging UE needs to support NR PC5 at least, so it does not need to use LTE PC5 to transmit the Ranging control signalling. And LTE PC5 only support broadcast, so there is no security protection for Ranging control signalling.

-	Sol#26 proposes that the functionality offered towards the UE by a Location Server can reside in a UE. Location Server UE can be an another UE separated from Reference UE and Target UE, and the interaction between Location Server UE and Reference UE/Target UE is needed. The interaction is more complex with the new UE involved, and Reference UE and Target UE can have the Location Server functionality, thus the Location Server UE is not needed.

-	Sol#27 focuses on the LMF selection by considering the Ranging and SL positioning requirements. This solution addresses KI#2, KI#3, KI#4, KI#5, KI#6, and KI#7.

-	Sol#28 proposes the LMF involved Ranging/Sidelink positioning. Three modes on ranging result calculation are introduced including UE based, LMF based and Hybrid. This solution addresses KI#2, KI#3, KI#4, KI#5, KI#6, and KI#7.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change * * * *

## 7.X	Key Issue #4: Control of Operations for Ranging/Sidelink positioning

For Key Issue #4: "Control of Operations for Ranging/Sidelink positioning ", based on Table 6.0-1, the solutions can be summarized and evaluated as the following,

-	Sol#3 proposes that the coordination and configuration information can be exchanged and the ranging results can be shared between the UEs via the PC5-S. The ranging request or response can be a new PC5-S signalling carried by the PC5 connection. The coordination and configuration information can be the Ranging role (Reference UE or Target UE), one time or period ranging, ranging for distance or direction measurement or both. The interaction between upper layer (e.g., ranging layer) and lower layer (e.g., AS layer) within one UE is also described. The ranging procedure in AS layer according to the ranging configuration is left to RAN WG.

-	Sol#4 proposes to use RSPP (Ranging/Sidelink Positioning Protocol) like LPP on top of existing PC5-S protocol. RSPP message is used to exchange the coordination & configuration information (e.g., Ranging capability, Ranging assistant data, Ranging measurement results), and a new kind of PC5-S message is used to carry the RSPP message. Target UE and the Reference UE pass the interacted coordination & configuration information down to the AS layer, and then the Ranging/Sidelink Positioning measurement and result calculation can be performed. It also proposed that RSPP will be determined by RAN WG.

-	Sol#5 presents a high-level procedure, and leave the detailed Sidelink Positioning procedure to RAN WG. During the Sidelink Positioning procedure, two UEs can perform the capability exchange, reference UE negotiation, Ranging/Sidelink positioning measurement and calculation, Ranging/Sidelink positioning result transmission. Ranging/Sidelink positioning request and response are performed before the Sidelink Positioning procedure, and ranging layer gets the ranging parameters (e.g., required location QoS) from application layer. UE-1 based sidelink positioning and UE-1 assisted sidelink positioning are proposed, and it is related to who triggers the sidelink positioning procedure and calculates the result.

-	Sol#14 proposes a general reference architecture for Sidelink positioning and ranging-based services, and this solution is applied to all KIs. The corresponding architecture is documented into the clause 4.3.

-	Sol#19 proposes that ranging control signaling is transmitted via PC5-U, and transmitted over either LTE PC5 or NR PC5. Since only NR PC5 based ranging is studied in R18 RAN scope, that means that the Ranging UE needs to support NR PC5 at least, so it does not need to use LTE PC5 to transmit the Ranging control signalling. And LTE PC5 only support broadcast, so there is no security protection for Ranging control signalling.

-	Sol#26 proposes that the functionality offered towards the UE by a Location Server can reside in a UE. Location Server UE can be an another UE separated from Reference UE and Target UE, and the interaction between Location Server UE and Reference UE/Target UE is needed. The interaction is more complex with the new UE involved, and Reference UE and Target UE can have the Location Server functionality, thus the Location Server UE is not needed.

-	Sol#27 focuses on the LMF selection by considering the Ranging and SL positioning requirements. This solution addresses KI#2, KI#3, KI#4, KI#5, KI#6, and KI#7.

-	Sol#28 proposes the LMF involved Ranging/Sidelink positioning. Three modes on ranging result calculation are introduced including UE based, LMF based and Hybrid. This solution addresses KI#2, KI#3, KI#4, KI#5, KI#6, and KI#7.

# * * * * End of changes * * * *