SA WG2 Meeting #152e	S2-2206400

# August 17th – August 26th, Elbonia

Source: 	China Mobile

Title: 	KI #2, New Sol: Support to subscribe UPF change notification from the NRF

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS / Rel-18

Abstract of the contribution: Solution proposal for supporting to subscribe UPF change notification from the NRF in case of UPF change.

1 Discussion

The solution proposal presented here addresses the Key Issue #2, which reads:

-	How and what specific information the UPF can expose to NEF/Local NEF/trusted AF, e.g. the information which can be exposed in Rel-18 SA WG2 studies such as FS_EDGE_Ph2 and FS_XRM, or information which has been justified for exposure.

This paper proposes to support the updating of target UPF for UPF event exposure service subscription in the perspective of consumer NF in case of UPF change for the scenario that a consumer NF has requested a target UPF to expose information targeting a single UE. The consumer NF subscribes UPF change notification from the NRF by updating UE ID and PDU Session ID list served by the UPF in the UPF profile in NRF.

2 Proposal

It is proposed to update TR 23.700-62 v0.3.0 as follows.

****************************First change****************************

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1: UPF event exposure service framework enhancements to support registration, deregistration and discovery via NRF | X |  |
| #2: UPF event exposure service for TSC management |  | X |
| #3: using the proper subscription mechanism depending on the event targeted by the UPF event consumer |  | X |
| #4: upgrading N4 to pass necessary event filtering information to the UPF |  | X |
| #5: registering UPF(s) serving a PDU session at UDM |  | X |
| #6: Determining the UPF(s) that serve a UE address |  | X |
| #7: Support to existing (Rel-16-Rel-17) data analytics with PDU Session Data Usage Events |  | X |
| #8: Support to existing (Rel-16-Rel-17) data analytics with QoS Flow level measurements |  | X |
| #9: NWDAF collects information from UPF by event exposure |  | X |
| #10: UPF event exposure service to NWDAF |  | X |
| #11: UPF event exposure service to NWDAF subscribed directly from UPF |  | X |
| #12: UPF registration and NWDAF collecting data from UPF | X | X |
| #13: Subscription to UPF Event Exposure Services in the event of UP Path change |  | X |
| #14: Reduce the UPF performance impacts due to data reporting to NF consumer |  | X |
| #15: Subscription of UPF Event Exposure Service |  | X |
| #16: Direct/indirect subscription of the UPF event exposure service |  | X |
| #17: Update/Release subscription of the UPF event exposure service |  | X |
| #18: QoS parameters exposure by UPF |  | X |
| #19: QoS Monitoring results exposure by UPF |  | X |
| #20: Support to subscribe UPF change notification from the NRF |  | X |

****************************Second change (all text is new) ****************************

6.X	Solution #X: Support to subscribe UPF change notification from the NRF

6.X.1	Key Issue mapping

This is a solution for KI#2.

6.X.2	Description

In the scenario that a consumer NF requests a single UE related information from the target UPF(s) via UPF event exposure service, a subscription update may be needed when the target PDU Session related UPF changes.

This solution aims to support the updating of target UPF for UPF event exposure service subscription in the perspective of consumer NF in case of UPF change during the life time of the PDU Session. A mechanism is proposed to add UE ID (SUPI, GPSI) and PDU Session ID list served by the UPF in the UPF profile in NRF. Then the consumer NF can subscribe the UPF information from the NRF directly.

6.X.3	Procedures

Figure 6.X.3-1: Update UE ID and PDU Session ID list in the UPF profile in NRF

1.	The UPF requests SMF to subscribe the list of UE ID and PDU Session matching with the UPF via the N4 procedure or over the Nsmf_EventExposure_Subscribe service operation providing UPF ID. The SMF responds the list of UE ID and PDU Session ID to the UPF, and notifies the update.

2.	The UPF sends Nnrf_NFManagement_NFUpdate Request message (the list of UE ID and PDU Session ID) to NRF to inform the NRF of its updated UPF profile.

3.	The NRF updates the NF profile of UPF instance.

4.	The NRF acknowledge UPF Update is accepted via Nnrf_NFManagement_NFUpdate response.

Figure 6.X.3-2: consumer NF subscribes UPF information notification from the NRF

0.	The UPF sends Nnrf_NFManagement_NFUpdate Request message (the list of UE ID and PDU Session) to NRF to inform the NRF of its updated UPF profile.

1.	The consumer NF invokes Nnrf_NFManagement_NFStatusSubscribe Request to obtain the UPF ID providing the list of UE ID and PDU Session.

2.	The NRF responds the UPF ID list to consumer NF via the Nnrf_NFManagement_NFStatusNotify service operation, and notifies the update.

3.	The consumer NF sends Nupf_EventExposure_Subscribe to relevant UPF(s) received in step 2 for event exposure service.

4.	The UPF notifies regarding the subscribed events.

6.X.4	Impacts on services, entities and interfaces

NRF:

-	Discovery of several UPFs that accords with the parameters that related to the UEs (list of UE ID and PDU Session ID).

**End of change**