3GPP SA WG2 Meeting #152E	S2-2206427

E-meeting, 17-26 Aug 2022

Source:	CATT

Title:	KI #1: Evaluation and conclusion for key issue #1 edge relocation in HR roaming

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract of the contribution: Evaluation and conclusion for KI #1: edge relocation in HR roaming in TR 23.700-48.

1. Discussion

The solution #3, #4, #5 and #39 address edge relocation in HR roaming. The evaluation is performed for EAS rediscovery or EAS relocation based on the scenarios UE mobility and AF triggered EAS relocation.

2. Proposal

It is proposed to agree the following texts in TR 23.700-48.

>>>> Start of 1st Change <<<<

7	Evaluation

Editor's note:	This clause will capture the evaluations related to the solutions per KI.

7.1	Evaluation of Solutions for Key Issue #1

7.1.x	Evaluation for Key Issue #1: Edge relocation in HR roaming

The solution #3 and #4 proposes EAS rediscovery for the scenario UE mobility to enable UE to discover new EAS information. The solution #5 (S2-2206428) proposes EAS rediscovery for the scenarios UE mobility and AF triggered EAS relocation. The solution #39 is to address the scenario EAS relocation triggered by AF using EAS IP replacement.

The solution #3 proposes to perform EAS rediscovery based on Option D, and V-SMF stores the rediscovery indication and sends it to UE.

The solution #4 proposes to perform EAS rediscovery based on Option A/B, and it covers the multiple SMF case. The SM context including authorization policy and impact field corresponding to old target DNAI from source is transferred between V-SMF and H-SMF.

The solution #5 (S2-2206428) proposes to perform EAS rediscovery based on Option A/B, and it covers the multiple SMF case. The SM context including roaming offload policy and the information for EAS relocation is transferred between V-SMF and H-SMF.

The solution #39 proposes the solution as described in clause 6.39.2.2. It supports the EAS relocation between different PLMNs using EAS IP replacement mechanism. The AF sends PLMN ID corresponding to target EAS to the network. The H-SMF determines target DNAI and provides to H-AF for V-SMF selection. Also the H-SMF sends an inter-PLMN relocation indicator to V-SMF to indicate the EAS relocation is from HPLMN to VPLMN.

>>>> End of 1st Change <<<<

>>>> Start of 2nd Change <<<<

8	Conclusions

Editor's note:	This clause will capture the conclusions of the study.

8.1	Conclusions for Key Issue #1

8.1.x	Conclusions for Key Issue #1: EAS discovery in HR roaming

Regarding the issues for edge relocation in HR roaming, for the UE mobility scenario, the solution #3, #4, #5 perform EAS rediscovery based on Option D and Option A/B respectively are recommended as baselines for the normative phase, and for the scenario EAS rediscovery for AF triggered relocation, the solution #5 is recommended as baselines for the normative phase, and for the scenario EAS relocation triggered by AF using EAS IP replacement, the solution #39 is recommended as baselines for the normative phase.

>>>> End of Changes <<<<