SA WG2 Meeting #S2-152E	S2-2206439

17 - 26 August, 2022, E-Meeting

Source: 	KDDI

Title: 	KI #4: Evaluation and Conclusion of Key Issue #4

Document for: 	Agreement

Agenda Item: 	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract of the contribution: This paper proposes an evaluation and a conclusion of Key Issue #4.

Discussion

There are four solutions addressing KI#4 from the perspective on how to represent standardized and operator-specific traffic category:

-	Solution #21: DNN, and “OS ID +OS App ID (”new OS Id value: Traffic Category”)

-	Solution #22: Traffic Category (New component type with new ATC rule)

-  Solution #23: Traffic Category (New component type)

-	Solution #24: Traffic Category (New component type with new mapping rule)

-  Solution #25: application descriptor with special value

-  Solution #26: Traffic Category based on 5G QoS characteristics

-  Solution #35: Connection Capability

Proposal

Add the following evaluation and conclusion for Key Issue #4 to TR 23.700-85.

*** BEGIN 1st CHANGES ***

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.x	Key Issue #4: Support standardized and operator-specific traffic categories in URSP

The key issue #4 is addressed from the perspective on how to represent standardized and operator-specific traffic category as follows:

-	Solution #21: DNN, and “OS ID +OS App ID (”new OS Id value: Traffic Category”)

-	Solution #22: Traffic Category (New component type with new ATC rule)

-  Solution #23: Traffic Category (New component type)

-	Solution #24: Traffic Category (New component type with new mapping rule)

-  Solution #25: application descriptor with special value

-  Solution #26: Traffic Category based on 5G QoS characteristics

-  Solution #35: Connection Capability

The evaluation on each solution is summarised as the following table:

Table 7.x-1: Evaluation of KI#4 related principles

| Type | New/Reuse of component type |  | Encoding |  | Number of values |  | Advantages |  | Disadvantages |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DNN | Reuse |  | DNN length (1 octet) + DNN |  | Variable |  | -Many values to specify -Reuse |  | -DNN has a very specific meaning from network perspective |  | Solution #21 |  |
| Application descriptor | Reuse |  | OS ID (16 octet) + OS APP ID length (1 octet) + OS APP ID |  | Variable |  | -Many values to specify -Reuse |  | -For the OSID, it is hard to be refarmed. |  | Solution #21, #25 |  |
| Traffic Category with ATC rule | New | Traffic Category (no assumption, not yet specified) | No Assumption | -No concern on backward-compatibility. | -Need to define new component type -Need to distribute new rule (ATC rule) to UE | Solution #22 |
| Traffic Category | New |  | Traffic Category (1 octet is assumed, not yet specified) |  | Max. 256 (Assumption) |  | -Not many values to be assumed, however, no concern on backward-compatibility if encoding changes |  | -Need to define new component type |  | Solution #23 |  |
| Traffic Category with new mapping rule | FFS (a part of TD or a new layer) | Traffic Category (no assumption, not yet specified) | No Assumption | -No concern on backward-compatibility . | -Need to define including Traffic Category is whether a part of TD or a new layer. | Solution #24 |
| Traffic Category based on 5G QoS characteristics | New | Traffic Category (no assumption, not yet specified) | No Assumption | -No concern on backward-compatibility. | -Need to define new component type | Solution #26 |
| Connection Capability | Reuse |  | Connection Capability (1 octet) |  | Max. 256 |  | -Reuse |  | -Not many values to specify, because operators may see possibility to specify more values than the maximum number of values, assuming operators assign the several values for per vertical services and/or MVNOs. -Concern on backward-compatibility if encoding changes |  | Solution #35 |  |

*** END 1st CHANGES ***

*** BEGIN 2nd CHANGES ***

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.x	Key Issue #4: Support standardized and operator-specific traffic categories in URSP

The following principles is proposed to be conclusion for normative work:

-  If encoding of component type to represent traffic categories is fixed length, the number of traffic category values shall secure at least 2 octets.

-	Solution #35 is recommended for normative work to support standardized and operator-specific traffic categories in URSP. In this case, an enhancement of encoding shall be done in stage 3 in order to represent more than 256 traffic category values in this component type. In addition, if the enhancement is done, backward compatibility issue shall be taking care of.

-	If it is difficult to realise the previous principles, Solution #23 is also recommended for normative work to support standardized and operator-specific traffic categories in URSP. In this case, the example of the Traffic category encoding mentioned in 6.23.1 shall be reconsidered (this is a stage 3 decision).

*** END 2nd CHANGES ***