3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206514

Elbonia, 17th – 26th Aug, 2022	(revision of S2-220xxxx)

Source:	Xiaomi

Title:	FS_XRM K#4#5 Sol#52 Update with Charging Offset

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: This paper proposes to update solution 52.

1. Introduction/Discussion

The packets or PDU set dropping because of PDU Set importance or PDU Set dependency is described in many solutions for KI#4 (PDU Set integrated handling) and KI#5 (Differentiated PDU Set handling) of the TR 23.700-60, e.g. solution 7, 11, 14, 19, 22, 24.

The UPF supports functionality to collect and report usage data to SMF. The SMF supports the interactions towards the CHF. For the downlink traffics, the consequence of using PDU set dropping procedure is that SMF charging data does not correspond to the volume that sent to the UE accurately, because that it is not count the downlink packets dropped by the RAN.

With these clarifications, it is proposed to add one note to describe the charging offset issue.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60 V0.3.0.

# * * * * First change * * * *

6.52.2	Description

This solution provides a framework for the PDU Set based QoS handling. The following principles apply:

PDU Set QoS policies/rules/profiles and PDU Set QoS Flow establishment

-	Dynamic PCC rules and non-dynamic PCC rules are supported.

-	PDU Set QoS requirements provisioning by AF supported (for dynamic PCC).

-	PDU Set QoS policies are provided to SMF by PCF for dynamic PCC rules.

-	RAN receives PDU Set QoS profiles from SMF.

PDU Set identification and marking over the user plane

-	PDU Sets are detected in the UPF.

-	Between AS and UPF, multiple PDU Set marking techniques can be supported (see bullet 2 below).

-	PDU Set identification between AS and UPF can be negotiated over Control Plane (via NEF for non-trusted AF), or based on pre-configuration.

Editor’s Note: PDU Set identification for peer-to-peer scenarios (i.e. between two UEs routed via the 5G CN) is FFS.

-	UPF marks PDU Sets in GTP-U layer via GTP-U header extension. GTP-U marking is independent from and common to different PDU Set markings between AS and UPF.

The following aspects are included:

In the following list, a set of baseline parameters are identified as part of PDU Set QoS framework. Additionally, a set of potential parameters for further study (as proposed by other solutions) is described.

NOTE 1: The baseline set of parameters does not exclude additional parameters to be agreed in the future.

1.	UPF identifies the PDUs belong to a PDU Set and the following information for each PDU Set:

Info for intra-PDU Set handling (i.e. KI#4, PDU Set integrated handling)

Baseline parameters:

- 	PDU Set Sequence number (SN) (solution 7, 8, 9, 11, 12, 14, 19, 20, 21, 22)

-	Start/End PDU of the PDU Set (solution 11, 12, 15, 18, 21, 22)

- 	PDU SN within a PDU Set (solution 11, 20, 22)

- 	Number of PDUs within a PDU Set (solution 9, 20)

Editor’s Note: Whether PDU Set Sequence number can also convey Start/End PDU of a PDU Set instance is FFS.

The QoS Flow is identified using QoS Flow ID and each PDU Set within the QoS Flow is identified using PDU Set SN. Each QoS Flow can be used to deliver one or more PDU Set.

Info for inter-PDU Set handling (i.e. KI#5, PDU Set differentiated handling)

Baseline parameters:

-	PDU Set importance (solution 7, 11, 14, 15, 18, 19, 22, 24)

Parameters for further study:

-	PDU Set dependency (solution 11, 14, 19, 22, 24)

2.	UPF identifies the above information (listed in bullet #1) based on the following mechanism(s):

Parameters for further study:

Option 1: by matching RTP/SRTP header and payload (RFC 3550/6184/draft-ietf-avtext-framemarking)

(solution 7, 12, 14, 15, 16, 17, 18, 23, 24)

Option 2: new RTP extension header (solution 8)

Option 3: by information provided by the AS in N6 encapsulation header e.g. GTP-U (solution 9, 22)

Option 4: by detection based on traffic characteristics (solution 12, 17, 18, 24)

Option 5: by non-standardized mechanisms UPF implementation (solution 20, 25)

Editor’s Note: What information can be achieved by each option need further discussion.

3.	UPF provides the above PDU Set related information (listed in bullet #1) to the RAN.

For PDU Set importance:

Options for further study:

Option 1: UPF classifies the DL traffics into different QoS Flows based on PDU Set importance (solution 10, 14, 24, 26)

Option 2: UPF classifies the DL traffics into different sub-QoS Flows based on PDU Set importance (solution 17, 18).

Option 3: UPF adds PDU Set importance into GTP-U header (solution 7, 11, 14, 15,17, 18, 19, 22)

For other PDU Set related info (listed in bullet #1):

UPF adds them into GTP-U Header (solution 7, 11, 14, 15, 19, 22)

4.	New QoS parameters for PDU Set based QoS handling in 5GS:

Baseline parameters:

-	PDU Set Delay Budget (PSDB) (solution 8, 9, 12, 14, 25, 26)

-	PDU Set Error Rate (PSER) (solution 8, 12, 25, 26)

Editor’s Note: The definition of the PDU Set Error Rate is FFS.

-	Whether all PDUs are needed for the usage of PDU Set by application layer (solution 22, 23, 25)

Parameters for further study:

-	Whether to drop a PDU Set in case PSDB is exceeded (solution 8, 22, 24, 25)

- 	PDU Set Priority

Editor’s Note: Whether the “PDU Set Priority” will be the same for all PDU Sets (i.e. same as existing QoS Flow Priority) or it will be different for each PDU Set (i.e. same as “PDU Set importance”) is FFS.

5. Information from AF

Baseline QoS parameters for each PDU Set within the QoS Flow:

-	PDU Set handling indication (solution 8, 15, 25) (indicating if PDU Set based handling should be activated to a flow)

NOTE 2: This indication may be implicitly indicated via other PDU Set related information provided by the AF.

-	Whether all PDUs are needed for the usage of PDU Set by application layer (solution 22, 23, 25)

Editor’s Note: Whether more detailed indicators are needed for different PDU Set QoS handling features (e.g. PDU Set integrated handling, QoS handling based on PDU Set importance, PDU Set dropping due to delivery failure of dependent PDU Sets) is FFS.

-	PDU Set Delay Budget (solution 12, 25, 26)

-	PDU Set Error Rate (solution 12, 25, 26)

Baseline parameters on frame identification:

-	Burst periodicity (solution 8, 12, 24, 26)

NOTE 3: The details of RAN behaviour is up to RAN WG.

Editor’s Note: The PDU Set QoS handling for Uplink media services is FFS.

NOTE 4:	In this release, it does not address the charging offset issue of DL PDU set eligible dropping by the NG-RAN.

# * * * * End of changes * * * *