3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206526

Elbonia, 17 - 26 August, 2022	(revision of S2-22xxxxx)

Source:	Huawei, HiSilicon

Title: 	KI#1: Update of solution#8 figure

Document for: 	Approval

Agenda Item: 	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution: This paper provides update the figure in solution 8.

1 Discussion

As described in solution 8, the AF-influenced traffic steering enforcement control and N6-LAN traffic steering enforcement control can be applied to the uplink data simultaneously. But the Figure 6.8.1-1 doesn’t reflect it. It is proposed to update the Figure 6.8.1-1 to reflect that when both work together how the traffic flow is going.

2 Proposal

It is proposed to update 23.700-18 on the figure 6.8.1-1.

FIRST CHANGE

6.8	Solution #8: AF request predefined SFC for traffic flow(s) related with target UE(s)

6.8.1	Description

The AF-influenced traffic steering enforcement control information is used to influence SMF routeing decisions for an identified traffic of PDU Session. It steers the user traffic to a local access to a Data Network (identified by a DNAI) per AF request. The AF-influenced traffic steering enforcement control information can be derived by the PCF based on the N6 Traffic Routing requirements provided by the AF. Until Rel-17 this is supported in non-roaming scenarios and to LBO deployments only.

The N6-LAN traffic steering enforcement control steer the identified traffic to appropriate operator 3rd party service functions in the N6-LAN. When the PCF generate the PCC rule for the SMF, the N6-LAN traffic steering enforcement control information is generated based on the operator's policy. This is supported in non-roaming and home-routed scenarios only. However that information is not generated or derived based on AF request.

The AF-influenced traffic steering enforcement control is only applied to the uplink traffic in the existing specification. The N6-LAN traffic steering enforcement control can be applied to the downlink traffic, uplink traffic or both. The N6-LAN Traffic steering policy identifiers can be different for uplink and downlink direction.

Table 6.3.1 of TS 23.503 [4] indicates that either N6-LAN Traffic Steering Enforcement Control or AF influenced Traffic Steering Enforcement Control can be included within a PCC rule. It means that traffic steering and SFC can't be applied to the same application traffic simultaneously.

From above analysis, it can be concluded that the N6-LAN traffic steering control (i.e. SFC policy) is different from the AF-influenced traffic steering enforcement control (i.e. traffic steering policy).

It is proposed that SFC policy and traffic steering policy is separated. An Application Function may send a request to influence SFC enforcement for the application traffic. In the request, the AF shall include the N6-LAN traffic steering control information, which includes target UE Identifier(s), DNN, S-NSSAI, SFC identifier (uplink and/or downlink), to the 5GC based on the SLA between the operator and the 3rd party. The PCF authorizes the N6-LAN traffic steering control information received from AF based on the SFC identifier (uplink and/or downlink) and operator's policy when generating the PCC rule. And then the N6-LAN traffic steering enforcement control information provisioning and enforcement defined in clauses 6.1.3.14 and 6.2.2.6 of TS 23.503 [4] can be applied, i.e. installing the related rule at UPF.

Figure 6.8.1-1: Architecture and traffic steering control flow

NOTE 1:	based on the local configuration, the UPF can differentiate the packet coming from N6-LAN or from N6 interface as described in clause 5.4.8 of TS 29.244 [9]

It is possible that N6-LAN traffic steering control and AF-influenced traffic steering control are both applied to the same application traffic simultaneously.

-	If the N6-LAN traffic steering enforcement control in downlink is applied when the AF-influenced traffic steering enforcement control is applied, the PCF include N6-LAN traffic steering policy identifier in downlink, and AF-influenced traffic steering policy identifier or N6 traffic routing information per DNAI within a PCC rule.

-	If the N6-LAN traffic steering enforcement control in uplink is applied simultaneous as the AF-influenced traffic steering enforcement control is applied, the PCF shall include N6-LAN traffic steering policy identifier in uplink and AF-influenced traffic steering policy identifier or N6 traffic routing information per DNAI within a PCC rule.

Similar with the AF influenced traffic steering control defined in clause 4.3.6.1 of TS 23.503 [4], the following cases can be distinguished:

-	AF requests targeting an individual UE by a UE address; these requests are routed (by the AF or by the NEF) to an individual PCF using the BSF. This is described in clause 6.8.2.1.

-	AF requests targeting a group of UE(s), or any UE accessing a combination of DNN and S-NSSAI, or targeting individual UE by a GPSI. This is defined in clause 6.8.2.2.

Same general handing as described in clause 4.3.6.1 of TS 23.502 [3] are executed, e.g. how the NEF map the AF request to the DNN, S-NSSAI.

NOTE 2:	The solution assumes that the 5GC and the AF have agreed via SLA on the mapping between a SFC identifier and for each DNAI/ N6 interface: the corresponding SFP, the metadata that the UPF should send to the SFP once it has done its SFC related classification.

END OF CHANGES