3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206533

Elbonia, 17 - 26 August, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1, Sol#18: update of solution 18

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2/ Rel-18

Abstract: This document tries to update Solution #18 to add Editor’s Note to seek more clarification.

1. Introduction/Discussion

As described in TR 23.700-47, step 7 and step 9 of solution#18:

7. The AMF determines that there are UEs in CM-IDLE state among the UEs provided by the SMF in step 6, and triggers group paging as specified in TS 23.247 [4].

…….

9. For MBS session activation, steps 6-15 of clause 7.2.5.2 of TS 23.247 [4] applies.

Step 7 is related to the CN-initiated group paging during the multicast session activation procedure. It is possible that the CM-IDLE UE shares the same PO as the RRC-inactive UE. Therefore the RRC-inactive UE may receive the CN-initiated group paging for the indicated MBS session ID.

Step 9 includes the NG-RAN initiated paging during the multicast session procedure, i.e., step 12 of clause 7.2.5.2 of TS 23.247. For the NG-RAN initiated paging, the RRC-inactive group UE could also receive the NG-RAN initiated group paging for the indicated MBS session ID.

As per TS 38.300 and TS 38.331, after the RRC-inactive UE receive the group paging MBS session ID, they need trigger the RRC resumption procedure. And for RRC-IDLE UE, they need initiate the service request procedure to move to the RRC connected state.

In light above, it is unclear whether solution#18 always require the UE go to the RRC connected state same as Rel-17 UE behaviour? If yes, the MBS session data receiving in RRC inactive state is not supported. If not, i.e. it is permitted to receive the MBS session data in RRC Inactive state for RRC Inactive state UE, the UE behaviour is not same as Rel-17. Then how UE determine it, i.e. based on what information?

This is the essential part of solution 18. It is proposed to add the related EN that the UE behaviour after receive the group paging ID during multicast MBS session activation need be clarified, e.g. how the UE is aware it can receive the MBS data in RRC inactive state.

Proposal 1: It is proposed to add one EN that the UE behaviour after receive the group paging ID during multicast MBS session activation need be clarified.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

6.18	Solution #18: MBS session management for RRC Inactive MBS data receiving UE

6.18.1	Introduction

This solution addresses Key Issue #1, especially on the enhancement of MBS session management for RRC Inactive MBS data receiving UE.

6.18.2	Functional description

This solution builds on top of solution 1. The multicast session management include following procedures:

-	MBS session activation, for UEs in RRC-IDLE state, group paging may be performed to bring the UE to RRC_CONNECTED as specified in TS 23.247 [4] for Rel-17.

Editor's note:	For UEs in RRC-INACTIVE state, the handling of MBS Session Activation in NG-RAN is to be determined by RAN WGs.

-	Multicast session deactivation/multicast session update, the existing procedure as defined in TS 23.247 [4] apply.

-	Multicast session release, the handling for UEs in CM-IDLE or CM-CONNECTED with RRC-CONNECTED state follows clause 7.2.2.3 of TS 23.247 [4]. For UEs without activated UP, the following SMF behaviour in step 2 is assumed to avoid paging the IDLE UEs:

Alternatively, for UEs without activated UP, the SMF does not trigger message to the AMF, instead the SMF marks that the UE is to be informed of the MBS Session release. In this case, the SMF initiates PDU Session Modification to inform the UE of the MBS Session release at next UP activation of the associated PDU Session, if needed.

Editor's note:	How MBS Session Release is handled for RRC_INACTIVE UEs is to be coordinated with NG-RAN.

Editor's note:	Paging procedures are under remit of the RAN groups and any related enhancements need to be confirmed by RAN groups.

6.18.3	Procedures

Figure 6.18.3.1-1: Multicast Session Activation/Release Procedure.

1.	UE joins the multicast MBS session via the procedure as defined in clause 7.2.1.3 of TS 23.247 [4].

2.	In some cases, e.g. due to radio resource shortage, NG-RAN could move one or multiple multicast group member UEs to RRC-INACTIVE state and those UEs are still able to received multicast MBS data.

3.	The multicast MBS session becomes inactive via the procedure as defined in clause 7.2.5.3 of TS 23.247 [4]. The group member UE can be moved to any state (i.e. CM-IDLE or CM-Connected with RRC Connected/Inactive state).

4.	After some time, MB-SMF triggers the multicast session activation or multicast session release.

5.	MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify to SMF(s), same as step 2 of clause 7.2.5.2 or step 1a of clause 7.2.2.3 of TS 23.247 [4], which also includes the MBS session status, i.e. activation or release.

NOTE:	Steps 1, 3-5 are same as the one defined in TS 23.247 [4].

6.	Based on the event information, the SMF determines whether the event is for MBS session activation or MBS session release. The SMF invokes Namf_MT_EnableGroupReachability Request to AMF. as specified in TS 23.247 [4].

Editor's note:	Whether SMF needs to know that the function that RRC-INACTIVE UE can receive the MBS data is supported or not is FFS.

7.	The AMF determines that there are UEs in CM-IDLE state among the UEs provided by the SMF in step 6, and triggers group paging as specified in TS 23.247 [4].

8.	The NG-RAN performs the group paging by sending the MBS session ID.

Editor's note:	How to handle RRC-inactive UE(s) that joined the MBS session and is able to receive multicast service in RRC-INACTIVE state is to be determined by RAN WGs.

Editor's note:	It needs to be confirmed that RRC-INACTIVE UEs also listen to the paging for RRC IDLE UEs, and not only to RAN paging.

Editor's note:	It is FFS whether and how UEs, NG-RAN nodes, or AMF also need to consider the transmission mode used in the cell where the UEs are comping to decide whether they can remain in RRC-INACTIVE state.

For the UE(s) joined the MBS session and need receive multicast service in RRC-Connected state, the handling specified in TS 23.247 [4] will apply.

Editor's note:  It needs clarify the behaviour of RRC-inactive UE after it receives the group paging of the related MBS session ID, e.g. how the UE is aware it can receive the MBS data in RRC inactive state.

9.   For MBS session activation, steps 6-15 of clause 7.2.5.2 of TS 23.247 [4] applies.

Editor's note: After receive step 12 in clause 7.2.5.2 of TS 23.247 [4], it is FFS whether the NG-RAN need page the UE if the UE is in RRC Inactive state.

Editor's note:  After receive step 12 in clause 7.2.5.2 of TS 23.247 [4], it needs clarify the behaviour of RRC-inactive UE after it receives the group paging of the related MBS session ID, e.g. how the UE is aware it can receive the MBS data in RRC inactive state.

For MBS session release, steps 3-9 of clause 7.2.2.3 of TS 23.247 [4] is not executed as for UEs without activated UP, the SMF does not trigger message to the AMF, instead the SMF marks that the UE is to be informed of the MBS Session release. In this case, the SMF initiates PDU Session Modification to inform the UE of the MBS Session release at next UP activation of the associated PDU Session, if needed.

6.18.4	Impacts on services, entities, and interfaces

UE:

-	For RRC_INACTIVE UEs, multicast session activation is to be determined by RAN WGs.

Editor's note:	Detailed impact will be determined by (and/or in collaboration with) RAN WGs.

# * * * * End of changes * * * *