3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206564

Elbonia, Aug 17 – 26, 2022

Source:	Huawei, HiSilicon

Title:	KI#6, New Sol, Add new 5QI and QoS Characteristics for AIML

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract: Add new 5QI and QoS Characteristics to support the AI/ML traffics.

1. Introduction/Discussion

According to TS 22.261 AI/ML operation has different performance characteristics with other types of application. Some new policy enforcement requirements, such as group MBR and transmission time window, may be needed for AI/ML operation. Separate QoS flows should be applied for AI/ML operation. The following KPI tables are described in the TS 22.261.

Table 1 KPI Table of split AI/ML inference between UE and Network Server/Application function

| # Uplink KPI |  |  |  |  | # Downlink KPI |  |  |  | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Max allowed UL end-to-end latency | # Experienced data rate | # Payload size | # Communication service availability | # Reliability | # Max allowed DL end-to-end latency | # Experienced data rate | # Payload size | # Reliability |  |
| 2 ms | 1.08 Gbit/s | # 0.27 MByte | # 99.999 % | # 99.9 % |  |  |  | # 99.999 % | # Split AI/ML image recognition |
| 100 ms | 1.5 Mbit/s |  |  |  | # 100 ms | # 150 Mbit/s | 1.5 MByte/‌frame |  | # Enhanced media recognition |
|  | # 4.7 Mbit/s |  |  |  | 12 ms | 320 Mbit/s | 40 kByte |  | Split control for robotics |
| NOTE 1: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets can take place in order to satisfy the reliability requirement. |  |  |  |  |  |  |  |  |  |

Table 2 KPI Table of AI/ML model downloading

| # Max allowed DL end-to-end latency | # Experienced data rate # (DL) | # Model size | # Communication service availability | # Reliability | # User density | # # of downloaded AI/ML models | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1s | 1.1Gbit/s | 138MByte | 99.999 % | 99.9% for data transmission of model weight factors; 99.999% for data transmission of model topology |  |  | AI/ML model distribution for image recognition |
| 1s | 640Mbit/s | 80MByte | 99.999 % |  |  |  | AI/ML model distribution for speech recognition |
| 1s | 512Mbit/s(see note 1) | 64MByte |  |  |  | Parallel download of up to 50 AI/ML models | Real time media editing with on-board AI inference |
| 1s |  | 536MByte |  |  | up to 5000~ 10000/km2 in an urban area |  | AI model management as a Service |
| 1s | 22Mbit/s | 2.4MByte | 99.999 % |  |  |  | AI/ML based Automotive Networked Systems |
| 1s |  | 500MByte |  |  |  |  | Shared AI/ML model monitoring |
| 3s | 450Mbit/s | 170MByte |  |  |  |  | Media quality enhancement |
| NOTE 1: 512Mbit/s concerns AI/ML models having a payload size below 64 MB. TBD for larger payload sizes. NOTE 2: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets can take place in order to satisfy the reliability requirement. |  |  |  |  |  |  |  |

Table 3: KPI Table of Federated Learning between UE and Network Server/Application function

| # Max allowed DL or UL end-to-end latency | # DL experienced data rate | # UL experienced data rate | # DL packet size | # UL packet size | # Communication service availability | # Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1s | 1.0Gbit/s | 1.0Gbit/s | 132MByte | 132Mbyte |  | Uncompressed Federated Learning for image recognition |
| 1s | 80.88Mbit/s | 80.88Mbit/s | 10Mbyte | 10Mbyte | TBD | Compressed Federated Learning for image/video processing |
| 1s | TBD | TBD | 10MByte | 10Mbyte |  | Data Transfer Disturbance in Multi-agent multi-device ML Operations |

It can be seen that there are no appropriate standard 5QI as defined in the TS 23.501 for the AI/ML operations, and new 5QI definition and corresponding QoS characteristics mapping will facilitate the specific QoS and Policy control. Therefore, it is proposed to add new Standardized 5QIs and characteristics mapping for AIML operations as the following table.

Table 4: AIML Operation and proposed QoS characteristics

| AIML operations | Resource Type | QoS characteristics | Comments |
| --- | --- | --- | --- |
| Split control for robotics | Delay-critical GBR | Default Priority Level: 22 Packet Delay Budget: 12 ms Packet Error Rate: 10-4 Default MDBV: 400000 bytes Default Averaging Window: 2000 ms | - Split control for robotics may be used for factory automation, so Delay-critical GBR resource type is needed. - Default Priority Level is same as Advanced Driving. - PDB, PER and MDBV are based on KPIs in TS 22.261. - Default averaging Window is used for implementation. |
| Split AI/ML image recognition | GBR | Default Priority Level: 15 Packet Delay Budget: 2 ms Packet Error Rate: 10-3 Default Averaging Window: 2000 ms | - Split AI/ML image recognition may be used for factory automation. Resource Type and Default Priority Level are same as Mission Critical Video user plane. - PDB is based on KPIs in TS 22.261. 1 ms air interface latency is assumed for factory automation, with the assumption of 1 ms CN delay in 2 ms end-to-end latency. - PER is based on KPIs in TS 22.261. - Default averaging Window is used for implementation. |
| Enhanced media recognition | GBR | Default Priority Level: 20 Packet Delay Budget: 100 ms Packet Error Rate: 10-4 Default Averaging Window: 2000 ms | - Resource Type and Default Priority Level are same as Conversational Video. - PDB is based on KPIs in TS 22.261. - PER is same as Video (Live Streaming). - Default averaging Window is used for implementation. |
| AIML model/data distribution and sharing. Federated Learning between UE and Network Server/Application function. | Non-GBR | Default Priority Level: 90 Packet Delay Budget: 1000 ms Packet Error Rate: 10-5 | - Resource Type and Default Priority Level are same as TCP-based file sharing. - PDB and PER are based on KPIs in TS 22.261. |

2. Text Proposal

It is proposed the following changes vs. TR 23.700-80.

# * * * * First change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | X |  |  |  |  | X |  |
| 2 |  | X |  |  |  |  |  |
| 3 |  | X |  |  |  |  |  |
| 4 |  | X |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |
| 6 |  |  | X |  |  |  | X |
| 7 |  |  | X |  |  | X |  |
| 8 |  | X | X |  |  |  |  |
| 9 |  |  |  | X |  |  |  |
| 10 |  |  |  |  | X |  |  |
| 11 | X |  |  |  | X |  |  |
| 12 |  |  |  |  | X |  |  |
| 13 |  |  |  |  | X |  |  |
| 14 |  |  |  |  |  | X | X |
| 15 |  |  |  | X |  | X |  |
| 16 | X |  |  |  | X | X | X |
| 17 |  |  |  |  |  |  | X |
| 18 |  |  |  |  |  |  | X |
| 19 |  |  |  |  |  |  | X |
| 20 |  |  |  |  |  |  | X |
| 21 |  |  |  |  |  |  | X |
| 22 |  |  |  |  |  |  | X |
| 23 |  |  |  |  |  |  | X |
| 24 |  |  |  |  |  |  | X |
| 25 |  |  |  |  |  |  | X |
| 26 |  |  |  |  |  | X | X |
| 27 |  |  |  |  |  |  | X |
| 28 | X | X | X | X | X | X | X |
| 29 |  | X |  | X |  |  |  |
| 30 |  | X |  |  |  |  |  |
| 31 |  | X |  |  |  |  |  |
| 32 |  |  | X | X |  |  |  |
| 33 |  |  | X |  |  |  |  |
| 34 | X |  | X |  |  |  |  |
| 35 |  |  |  | X |  |  |  |
| 36 |  |  |  | X |  |  |  |
| 37 | X |  |  |  | X | X | X |
| 38 |  |  |  |  |  | X | X |
| 39 |  |  |  |  |  |  | X |
| 40 |  |  |  |  |  | X | X |
| 41 |  |  |  |  |  |  | X |
| 42 |  |  |  |  |  | X | X |
| 43 |  |  |  |  |  |  | X |
| 44 |  |  |  |  |  |  | X |
| 45 |  |  |  |  |  |  | X |
| 46 |  |  |  |  |  |  | X |
| 47 |  |  |  |  |  |  | X |
| X |  |  |  |  |  | X |  |

# * * * * Second change (All texts new) * * * *

6.X	Solution #X: Add new 5QI and QoS Characteristics for AIML

6.X.1	Description

This solution addresses KI#6: QoS and Policy enhancements.

The 5GS should bind the AI/ML traffic to separate QoS flow(s). The 63, 68, 81 and 91 standard 5QIs, and corresponding QoS characteristics mapping are newly added into Table 5.7.4-1 of TS 23.501[3], which is to be used for the QoS flow(s) of the AI/ML operation.

Table 6.X.1-1: Standardized 5QI to QoS characteristics mapping

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 63 | Non-GBR | 90 | 1000 ms | 10-5 | N/A | N/A | AIML model/data distribution and sharing (see TS 22.261 [2]). Federated Learning between UE and Network Server/Application function (see TS 22.261 [2]). |
| 68 | GBR | 20 | 100ms | 10-3 | N/A | 2000 ms | Enhanced media recognition (see TS 22.261 [2]). |
| 81 | GBR | 15 | 2 ms | 10-3 | N/A | 2000 ms | Split AI/ML image recognition (see TS 22.261 [2]) |
| 91 | Delay Critical GBR | 22 | 12 ms | 10-4 | 400000 bytes | 2000 ms | Split control for robotics (see TS 22.261 [2]) |

6.X.2	Procedures

None.

6.X.3	Impacts on services, entities and interfaces

None.

# * * * * End of changes * * * *