SA WG2 Meeting #152E (e-meeting)	S2-2206579

17 - 26 Aug, 2022, Electronic meeting

Source:	Samsung

Title:	New WID: AF managing user priorities for particular slice

Document for:	Agreement

Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Support for AF to manage user priorities within a slice

Acronym: TEI18_AFUPS

Unique identifier:

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

SA6 is currently studying the Network Slice Capability Exposure for Application Layer Enablement as part of a Release 18 Study Item (FS_NSCALE). As a part of the study, a key issue (refer to KI#10 in 3GPP TR 23.700-99) is agreed which focuses on required application level support for managing trusted third-party owned application(s). The key issues considers a case (clause 5.7.1 of 3GPP TR 22.835) where the third-party offers its consumers different contract qualities level (e.g. gold, silver and bronze), and needs to provide high priority to the higher level of contract qualities. A solution was also discussed in clause 6.9 of TR 23.700-99.

At present SA2 has not defined any mechanism for AF to provision different priorities for the users within a slice and then taking some suitable action for NSACF to allow higher priority UEs during the case of quota been exceeded for either UE count or PDU count. .

# 4	Objective

The objective is to address the support for AF to manage user priorities within a slice by:

-	providing service operation by NSACF using which trusteed AF can configure the policy which will include the user priorities within a slice.

-	NEF will provide service operation using which untrusted AF will configure the policy.

-    During NSAC if the quota has been exceeded but NSACF found that the request is for a higher priorities UE then instead of rejecting the request it will accept it and release one of the lower priority UEs

Note: This work will require 0.5 TU.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.502 | Add one service operation in NSACF using which AF will configure the policy which will include the user priorities for a particular slice. Then NSACF use this policy to find out the request is from higher priority UEs during NSAC and accept the request even if quota has been exceeded by releasing one of the lower priority UEs. | SA#99 (March 2023) |  |

# 6	Work item Rapporteur(s)

Ashok Kumar Nayak, Samsung (ashok.nayak@samsung.com)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Samsung